Source:	Xiaomi Technology

Title:	Update draft DaCAS test script

## Document for:	discussion

## Agenda Item:	7.6

# Introduction

This contribution updates the DaCAS‑2 test script based on prior discussions and the initial draft in [1]. This update adds frequency response and sending loudness analysis to the script.

# Content

The revised script now includes calculation for all the characterise listed in [2]: loudness, frequency response (single source), and directional information (single source).

Editor’s note:

-	Support for delay measurement depends on the progress of DaCAS-2

The script uses SciPy's power spectral densities estimator (scipy.signal.welch) to compute power spectral densities, forming the basis for frequency response and loudness.

## Frequency response for single source

Computes the 1/12‑octave bandwidth spectrum using ISO‑3 R40 preferred numbers for frequencies from 100 Hz to 12 kHz:

100, 106, 112, 118, 125, 132, 140, 150, 160, 170, 180, 190, 200, 212, 224, 236, 250, 265, 280, 300, 315, 335, 355, 375, 400, 425, 450, 475, 500, 530, 560, 600, 630, 670, 710, 750, 800, 850, 900, 950, 1000, 1060, 1120, 1180, 1250, 1320, 1400, 1500, 1600, 1700, 1800, 1900, 2000, 2120, 2240, 2360, 2500, 2650, 2800, 3000, 3150, 3350, 3550, 3750, 4000, 4250, 4500, 4750, 5000, 5300, 5600, 6000, 6300, 6700, 7100, 7500, 8000, 8500, 9000, 9500, 10000, 10600, 11200, 11800.

[According to TS 26.261 [3], Clause 5.4.1.3, the reported results use a reduced set of frequencies: 100, 200, 300, 6000, 8000, and 12,000 Hz (represented as 11,800 Hz in the code, because 12,000 is not an ISO-3 preferred number)).]

Editor’s note:

-	Support for different format will be added in future

## Send loudness

The function p79_slr computes Send Loudness Rating according to TS26.260[4]

Editor’s note:

-	The IVAS reference renderer will be added in the future, and the decision on whether use ITU-R BS.1770/P.700 via potential Binaural rendering need be made by the group.

## Next plan

The script is proposed for the group to check the script’s calculations for each characteristic.

licensing information, requirements and environment will be added in future revisions.

The missing components—database reading, loudness test rendering, generate final report, using the right reference signal, and support for different formats—will be added after the details in DaCAS‑2 are added, including the setup of database and the setup of the reference signal (in dBFS).

The further updated version will propose to upload to repo under the Audio subgroup.

# Conclusion

This contribution presents an updated version of the DaCAS-2 test script, incorporating draft frequency response and loudness analysis. It is submitted for group review and proposed as a foundation for further refinement based on feedback and progress in DaCAS-2.

## References

- S4aA250122: Draft script for DaCAS-2

- S4aA250105: On single source capture evaluation procedure

- 3GPP TS 26.261: Terminal audio quality performance requirements for immersive audio services

- 3GPP TS 26.260: Objective test methodologies for the evaluation of immersive audio systems