3GPP TSG-SA WG4 Meeting #134	S4-251856

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | 0004 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AvCall-MED] Support for node-level interactivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Avatar-based AR call, the user’s avatar may be part of a scene and represented as a node within the scene graph of that scene. TS 26.119 defines a number of media capabilities for different types of AR devices, including scene description capabilities. Among these scene description capabilities, is the capability of supporting interactivity as defined by the SD-Rendering-glTF-Interactive capability. However, this capability does not include node-level interactivity which is needed for avatars. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution proposes to include MPEG_node_interactivity extension as part of the SD-Rendering-glTF-Interactive scene processing capability defined in clause 9.2 to support node-level interactivity for avatars. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interactions for Avatar in the scene is limited. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 9.2	glTF-based Scene Description capabilities

This clause defines client cababilities for rendering and presenting Scene Description based on glTF2.0 [10]. Additional extended capabilites are defined, primarily to support real-time media based on the extensions defined in the MPEG-I Scene Description in ISO/IEC 23090-14 [11-13].

The following scene processing capabilities are defined.

-	SD-Rendering-glTF-Core: The capability to process glTF2.0 [10] scene description files and to render the described scenes with the following restrictions:

-	The glTF 2.0 scene description can either be a standalone JSON file or an encapsulated GLB file.

-	The glTF 2.0 scene description can have an aggregate geometry complexity, including all mesh primitives of the scene, of at least 100k faces and/or points. The requirement on the number of faces in mesh nodes allows for moderate complexity scenes.

-	The glTF 2.0 scene description can have at least 20 materials using static image textures. The requirement on the number of materials with image texture maps allows for moderate complexity scenes.

-	The glTF 2.0 scene description can use glTF2.0 animations and skinning.

-	The glTF 2.0 scene description can use the KHR_lights_punctual and KHR_materials_specular extensions.

-	SD-Rendering-glTF-Ext1: On top of the SD-Rendering-glTF-Core capability, the capability to process MPEG-I Scene Description documents as specified in [11-13] and to render the described scenes with the following restrictions:

-	The MPEG-I Scene Description document can use the MPEG_media extension.

-	The MPEG-I Scene Description document can use the MPEG_accessor_timed and the MPEG_buffer_circular extensions.

-	The MPEG-I Scene Description document can use the MPEG_texture_video extension.

- 	The MPEG-I Scene Description document can use the MPEG_audio_spatial extension.

-	The MPEG-I Scene Description document can use the scene description update mechanism as defined in clause 5.2.4 of [9].

-	Processing of scene description updates as defined in ISO/IEC 23090-14 [11-13] to allow the device to contribute to a scene description of a dynamic shared scene.

-	The MPEG-I Scene Description document can use at least 4 materials using video textures

NOTE: This functionality implies that at least 4 concurrent video decoder instances are available

-	The MPEG-I Scene Description document may use at least 4 object-based mono audio sources, which allows for relatively simple audio support.

NOTE: This functionality implies that at least 4 concurrent mono sources are available as the output of the audio decoding engine instances are available, potentially requiring concurrent audio decoder instances

Editor’s Note: the audio-specific minimal support, such as the type of audio sources and the number of audio sources to support as part of this capability, are to be agreed with the Audio SWG.

-	SD-Rendering-glTF-Ext2: On top of the SD-Rendering-glTF-Core capability, the capability to process MPEG-I Scene Description documents as specified in [11-13] with the following restrictions:

-	The MPEG-I Scene Description document can use the MPEG_anchor extension.

-	The MPEG-I Scene Description document can use the EXT_lights_image_based and the MPEG_lights_texture_based extensions.

-	SD-Rendering-glTF-Interactive: On top of the SD-Rendering-glTF-Core capability, the capability to process the following glTF 2.0 extensions defined in [11-13] to enable user input and local interaction processing.

-	The MPEG-I Scene Description document can use the MPEG_scene_interactivity extension with the following triggers: TRIGGER_USER_INPUT, TRIGGER_PROXIMITY, TRIGGER_COLLISION, TRIGGER_VISIBILITY.

-	The MPEG-I Scene Description document can use the MPEG_scene_interactivity extension with actions: ACTION_ACTIVATE, ACTION_TRANSFORM, ACTION_BLOCK, ACTION_ANIMATION, ACTION_MEDIA, ACTION_MANIPULATE, ACTION_SET_MATERIAL.

-	The MPEG-I Scene Description document can use the MPEG_node_interactivity extension with the following triggers: TRIGGER_USER_INPUT, TRIGGER_PROXIMITY, TRIGGER_COLLISION, TRIGGER_VISIBILITY.

* * * End of Changes * * *