3GPP TSG-SA WG4 Meeting SA4 #134 	S4-241889

17th- 23rd November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0033 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### [FS_AMD_Ph2] Baseline procedure for establishment of a multi-access uplink media streaming session using ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This present contribution relates to 5G Media Streaming (5GMS) systems defined in 3GPP TS 26.501 and related specifications (for example TR 26.804, TR 26.802). In particular, this contribution addresses “Network Assistance procedures” specified for media delivery sessions. This contribution applies to 3GPP SA4 work on “Study on Advanced Media Delivery Phase 2- FS_AMD_MED_Ph2”, the 3GPP Rel 20 study item that focusses on advanced media delivery architectures. The phase 2 of this study has just been approved in SA#109 and the work has begun in SA4. Several new Work Taks (WTs) have been recently added to the study. Among others, one key issue is extracted below: - WT#8: Multi-access media delivery phase 2: Clause 5.18.7 of TR 26.804 lists topics for further study on multi-access media delivery, including the impact on splitting M4 media flows when transported as GBR QoS flows; implications of the ATSSS architecture for UE multipath management, dynamic policy, network assistance, and network slicing procedures; and alignment with the study on media delivery from multiple service locations/endpoints. Additionally, based on discussion with 5G-MAG, the impact of multi-access media delivery for uplink streaming is to be studied. It is proposed to add the proposed content to the latest draft of TR 26.804 v19.1.0 under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.804 on “Clause 5.18 Mulit-access Media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change (All new text)

#### 5.18.4.3	Multi-Access media delivery for uplink direction using ATSSS

This section describes Multi-Access media delivery for uplink direction using Access Traffic Steering, Switching, and Splitting (ATSSS). From clause 5.32 of TS 23.501 [23] and clause 4.22.3 of TS 23.502 [24], the ATSSS support is enabled by dedicated user-plane functions distributed in UE and UPF, and new policy rules at the control-plane required to enforce traffic management on the two links by enabling a Multi-Access (MA) PDU layer.

The Policy and Charging Configuration (PCC) rules generated by the ATSSS-enabled Policy Control Function (PCF) for each Service Data Flow (SDF) type are translated by the ATSSS-enabled Session Management Function (SMF) into N4 rules for the User Plane Function (UPF). These rules include routing and Quality of Service (QoS) parameters, such as Forwarding Action Rules (FARs) and Packet Detection Rules (PDRs), as well as dedicated Multi-Access Rules (MARs) and ATSSS rules. The SMF also delivers these ATSSS rules to the User Equipment (UE) via the Access and Mobility Management Function (AMF), along with the UE Route Selection Policy (URSP) rules, which are transmitted over the NAS (N1) interface.

The ATSSS functions in the UE are in charge of managing the uplink traffic (from UE to DN), while the ATSSS functions in the UPF are in charge of managing the downlink traffic, by enabling a combination of MPTCP (higher layer) and/or lower layer multi-link technology, called in general ATSSS-LL.

A “Multi Access (MA) PDU Request” in the UL NAS Transport message indicates to the CN that it is required to establish a new MA PDU Session and to apply the ATSSS-LL functionality, or the MPTCP functionality (or both) to implementing a multi-access PDU session. The multi-access session will be governed by a specific set of possible ATSSS rules, which are:

- Selection on whether use Multipath-TCP (MP-TCP) or ATSSS-LL for each matching SDF

- The indication of a steering mode

A Multi-access PDU Session may be set up in one of three different ways for uplink media steaming:

1.	The UE may set up a Single Access PDU Session over one access network, and then register over another access network and request a Multi-access PDU Session to be set up using both the access networks.

2.	The UE may indicate its capability for ATSSS, and request the setting up of a Multi-access PDU Session to begin with.

3.	The UE may request to set up a Single-Access PDU Session, but the network may transparently set up a Multi-access PDU Session instead.

For simplicity, for the high-level call flows for 5G Uplink Media Streaming with multi-access media delivery, the first option above is used. Figure 5.18.4.2-1 shows a high-level call flow for a 5G uplink Media Streaming session over a Multi-access PDU Session that uses two different access networks: a 3GPP access and a non-3GPP access.

Assumptions:

-	The 5GMS Client is unaware of the UE ATSSS steering functionality.

-	5G Uplink Media Streaming session is set up over 3GPP access first before the UE switches to a Multi-Access PDU Session to use both the access networks.

Figure 5.18.4.3-1: Basedline procedure for 5G Uplink Media Streaming session with multi-access media delivery

### The steps are as follows:

### Step 1 to 7: Single Access (initial state): UE app requests to start uplink streaming. MSH coordinates with 5G core network (AF → PCF → SMF) to establish a PDU session and apply any ATSSS/PCF policies. Media server (MAS) and MAF complete the media-level session setup and streaming begins on a single access. Baseline single-access operation including resource provisioning, QoS flows set-up, and media paths are provisioned and established before multi-access is considered.

### •	Step 1: Start uplink streaming using single access

### •	Step 2: PDU session establishment request for UL transport session using single access

### •	Step 3: SMF retrieves ATSSS policy from PCF

### •	Step 4: PDU session establishment response for UL transport session using single access

### •	Step 5: PDU session establishment request for UL media streaming session using single access

### •	Step 6: PDU session establishment for UL media streaming using single access

### •	Step 7: Uplink (UL) media streaming using single access

### Steps 8 to 9: Access availability detection: The UE must detect candidate extra access before activating MA to improve throughput/resilience. The Modem / OS continuously monitors access technologies (e.g. Wi-Fi, 5G NR). When a new access becomes available (e.g. Wi-Fi association completed, IP address assigned), the OS notifies upper layers via events (netlink, Android ConnectivityManager, or similar). The UE Modem/OS provides an Access Availability Report to the MAF (Media Access Function).

### •	Step 8: Event: new access discovered (Wi-Fi association complete)

### •	Step 9: Notify: AccessAvailable (accessId=Wi-Fi, IP ready)

### •	Step 10: Evaluate: check eligibility for media streaming

### Steps 11: Local decision and reporting to MSH: MAF sends detailed per-access metrics (capabilities) to MSH so media control can decide. The MAF evaluates whether the new access is eligible for media traffic (e.g., same PDU session or via ATSSS policy). The MAF sends a MultiAccessDetected notification to the Media Session Handler (MSH), including: Access ID and type (e.g., Wi-Fi / 5G), QoS capability or path metrics (latency, throughput, loss rate), Possible IP endpoints. MSH needs per-access info (e.g., throughput, latency) to choose whether to activate MA and how to split/steer streams for non-GBR flows or GBR flows.

### •	Step 11: Notify: MultiAccessDetected(accessId=Wi-Fi, QoS, latency, loss, throughput)

### Steps 12 to 13: MSH triggers multi-access activation: MSH informs the application and simultaneously contacts the Media AF (M5 API) to request authorization/assistance for MA activation. The MSH analyzes the event and decides whether to activate MA operation. It notifies the Application (App)  about multi-access availability. Parallelly, it sends a “Activate MA” request to the Media AF to align control-plane and policy configuration. Control-plane coordination is needed: AF can consult PCF and decide on ATSSS rules, delivery boosts, or QoS changes.

### •	Step 12: Inform: additional access detected, preparing MA activation

### •	Step 13: POST /events MultiAccessDetected (M5 API)

### Steps 14 to 18: Media AF & network provisioning: Media AF asks PCF to permit MA; PCF/SMF allocate per-access QoS flows, steering rules, and any PCC templates; AF returns activation parameters to MSH. The Media AF contacts the PCF/SMF to:

### •	Authorize dual access usage (ATSSS rules or PCC policy),

### •	Allocate new QoS flows for the added access path,

### •	Return updated transport / steering instructions to the MSH.

### •	The PCF/SMF sends a confirmation upon acceptable conditions and informs this to the AF

### The 5G core network must authorize resource usage per operator policy; QoS flows must be created/mapped per access for correct scheduling.

### •	Step 14: Request: authorize dual access (update ATSSS / PCC policy)

### •	Step 15: Allocate QoS Flows for new access (e.g. QFI for Wi-Fi)

### •	Setp 16: Confirm: QoS rules provisioned

### •	Step 17: Ack: multi-access allowed / parameters

### •	Step 18: Response: activation authorized, steering rules

### Steps 19 to 22. Establish new subflow: MSH orders MAF to open a transport subflow on the new access (MP-TCP add_addr or MP-QUIC create PATH). Modem/OS returns socket ready, MAF confirms to MSH. The MAF establishes a new subflow (e.g. MP-TCP, MP-QUIC) for the newly detected access based on the confirmation received from the MSH. While at the same, the MAF also requests to the modem to open a socket to the newly detected access. As the modem confirms this, and once the new subflow is ready, the MAF notifies the MSH: “Multi-access active”. Only after the transport subflow is up can traffic be sent/received on that access.

### •	Step 19: Command: establish subflow for accessId=Wi-Fi (MP-TCP add_addr / MP-QUIC path)

### •	Step 20: Request: open socket/interface binding for Wi-Fi

### •	Step 21: Response: socket ready, interface active

### •	Step 22: Notify: new path established (Wi-Fi)

### Steps 23 to 24. Update transport policy: MSH decides split ratios/steering policy and updates MAF/transport. App is informed that MA is active. The MSH updates scheduling / path selection rules (e.g. round-robin, split, weighted load). It informs the App: “Multi-access activated — streaming now uses both accesses.” App can now adjust encoder (e.g., send enhancement layers over Wi-Fi) or UI can reflect improved connectivity.

### •	Step 23: Update scheduling policy (split / aggregation)

### •	      Step 24: Notify: Multi-access active (5G + Wi-Fi)

## End of change