3GPP SA4 WG4 meeting #134	S4-251822r03

# Dallas, Texas, 17-21 November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0014 | rev | 0 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI5 Client-based Media Application Server selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.13 adding a solution to Key Issue #5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#5 will not be solved. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |
| #11(a) |  |  |  |  | X |  |
| #11 (b) |  |  |  |  | X |  |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

2nd change
(All new text)

## 7.13	Solution #11 (b): Client-based Media Application Server selection based on asynchronous notification to the Media Client

### 7.13.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5.

### 7.13.2	Functional description

#### 7.13.2.1	Introduction

This Candidate Solution proposes a method that allows re-selection of Media AS service locations during a media delivery session when content is delivered between the UE Media Client and the Media AS via reference point M4, based on the UE application QoE requirements (e.g., resolution, bit rate, etc.) while also taking into account the various  characteristics (e.g. Media AS load, Media AS energy consumption, current Media AS QoE metrics, etc.) of each individual service locations of the Media AS which are being used to support media delivery. As a concrete example, "F-droid" is an Android app which allows the user to stream real-time sensor data from a UE to WebSocket clients, etc.

This solution leverages interfaces existing reference points in the Media Delivery System that support communication between a Media Application Provider and the 5G System, particularly for exchanging Quality of Experience (QoE) data. The interfaces at existing reference points are enhanced by the solution with network energy-related characteristics, allowing the Media Application Provider to use this information for service location reselection. In particular the solution leverages extensions for edge processing defined for the 5GMS System in clause 4.5 of TS 26.501 [26501] and for the RTC System in clause 6 of TS 26.506 [26506] in which instances of the Media AS are deployed as Edge Application Server (EAS) instances and they offer different service locations to the Media Client.

Figure 7.13.2-1: Illustration of application servers reselection based on energy characteristics

This collaboration scenario shown in figure 7.13.2-1 represents the case of accessing two different Data Networks using the same network slice. A CDN server (e.g., 5GMS AS service location) is either deployed in each of the trusted Data Networks, or presents a multi-homed interface at reference point M4 through each of the trusted DNs.

The Media Application Provider negotiates with the MNO to provision a network slice as described in clause 4.3 of TR 26.941 [26941]. The operator provisions the network slice, and provides connectivity services to both the DNs through the slice.

The Media Application Provider may deliver Service Access Information through reference point M8. The Service Access Information delivered to the Media-aware Application may have information about different service locations accessible through each of the DNs. For example, an enterprise may utilise an enterprise-specific CDN (e.g., hosting enterprise-related video tutorials). Media delivery sessions for enterprise-related video tutorials use the PDU Session terminating in the enterprise-specific service location, while some other video tutorial requests are sent through PDU Sessions terminating at a different service location. The Energy Information AF instantiated in the Media AF of figure 7.13.2-1 is in relaton with the architecture of clause 7.6.

In the above architecture, how to relocate the Edge AS based on the QoE requirements of the UE in combination with energy usage/characteristics of the Media AS service locations? Imagine a case when the media application is accessing media content from AS#1. Can the AF relocate the UE now to AS#2 based on: (a) the energy consumption of AS#1? (b) the QoE requirements of the application?

The Application Function (AF) may need to relocate the Edge AS considering different scenarios e.g. AS load, AS energy consumption, AS current QoE metrics, etc. In some deployments this would require collective relocation.

### 7.13.3	Procedures

Figure 7.12.3-2 below details the different steps for application server re-selection process.

Figure 7.13.3-1: Procedures for application server re-selection based on energy characteristics

The call flows and procedures are described below:

Assumption: The Application Provider may deliver Service Access Information through reference point M8. The Service Access Information delivered to the media aware Application may have information about different Service Operation Points accessible through each of the DNs. For example, an enterprise may utilise an enterprise-specific service location of the Media AS (e.g., hosting enterprise-related video tutorials). 5G Media Streaming sessions for enterprise-related video tutorials use the PDU Session terminating in the enterprise-specific service location of the Media AS, while some other video tutorial requests are sent through PDU Sessions terminating in a different service location of the Media AS.

In the below call flow, steps 1 to 8 are retained from the baseline procedure for downlink media streaming found in clause X of TS 26.501 [26501], which is common steps followed to establish any media downlink session establishment.

These steps follow the usual setup while establishing a downlink media delivery session between the UE and the 5GMS AS. However, they are adapted for the generalised Media Delivery architecture The Media-aware Application requests a media delivery session, following which a media delivery session is established, and the media is transferred between the Media Client and the Media Application Provider via the Media AS. The media delivery session starts at step 8. Based on step 1 to 8, an on-going media delivery session is established.

9.	The Energy Information AF subscribes to EIF to be informed and to receive energy-related information from the different NFs.

Steps 10-15: Reporting UE-related and apps-related QoE metrics.

10.	The OAM periodically requests the UE to report its current QoE. It first reaches the 5G Core entities, such as PCF and SMF.

11.	The 5G Core forwards this request to the Media AF.

12.	The Media AF in turn forwards this to Media Session Handler.

13.	The Media Session Handler (after querying the current QoE information from Media Access Function), reports this information to the Media AF.

14.	The Media AF now forwards this information to the 5G core, which in turn forwards this to the OAM. This way, the UE periodically reports the QoE metrics upon request from the OAM. This could be for instance, based on the QMC framework, as described in clause TS 36.300 Clause 23.16, where UE provides application relevant metrics, limited to a certain duration of X ms, in order to protect the user privacy.

Steps 16-20: Reporting Application Server related metrics.

16.	The EIAF requests the EIF to report the energy-related characteristics of Media AS from individual service locations. This could be for instance, the total KW/hr of energy consumed by each Media AS service location for a particular time period. This time period is left to the Media Application Service Provider or the owner of Media AS of the service location to define.

17.	EIF collects (or retrieves) the energy-related information of each Media AS service locations.

18.	EIF reports this energy-related information to the EIAF.. Following this, the EIAF forwards this information to the AF.

19.	Based on this information, the Media AF now internally processes the received energy-related information about the Media AS service locations.

20.	The Media AF maps the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the service location of AS energy-related information.

In steps 21 to 28 the Media AS service location is reselected based on the reported energy-related characteristics.

21.	The AF sends the new configuration information to the UE client (Media session Handler), with the list of new available application servers (ASs) with their service locations, as per clause 5.19.3.3. of the present document. This could be sent as a meta-data to the UE client, via M5 interface via Service Access Information which may include Media Player Entry URLs and BaseURL of the AS endpoint.

22.	The UE client is allowed to select another new service location of Media AS most suitable for its current session without modifying the on-going session.

23.	This is done based on the information provided in the Media Player Entry by the Media Session Handler, the Media Access Function request a new energy-efficient Media AS service location and as a result a new service location of Media AS is selected.

24.	The UE client sends a new request to modify the existing session to the Media AF.

25.	 This is forwarded to the respective service location of the Media AS from the Media AF as described per clause 5.1 of TS 26.501.

26.	The existing media delivery session now successfully modified with the new service location of the Media AS.

27.	Media transfer from the most energy efficient server is sent to the Media Player.

28.	This is informed to the Media Session Handler.

### 7.13.4	Summary

This candidate solution introduces a mechanism for enabling Media Application Providers to dynamically select the most energy-efficient Media AS service location. The proposed methods are applicable to downlink media sessions, but also for uplink media sessions and RTC communication sessions. This is based on the following factors:

1.	UE application and characterisation data: Information pertaining to the User Equipment (UE),  factors such as, current QoE, and application-specific needs (e.g., resolution, bit rate).

2.	Energy efficiency metrics from Network Operator: Energy-related characteristics of the application servers, provided by the mobile network operator. These metrics, including server power consumption profiles and energy performance indicators, are transmitted through the Energy Information Function (EIF) to the Energy and Infrastructure Application Function (EIAF) within the Media Application Function (Media AF).

This approach aims to optimise the energy consumption of multimedia content delivery, particularly video streaming services, while maintaining service quality within defined thresholds by enabling client-based Media Application Server selection based on asynchronous notification to the Media Client as specified in TS 26.512.

The candidate solution operates entirely within the network layer, which involves the UE, QoE characteristics as the steering and optimisation mechanisms are fully supported by standard adaptive streaming frameworks. This ensures compatibility with existing adaptive streaming ecosystems, preserving end-user experience while reducing the environmental footprint of content delivery.

End of changes