3GPP TSG-SA WG4 Meeting SA4 #134 	S4-241890

17th- 23rd November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0034 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### [FS_AMD_Ph2] Network assistance and Delivery boost for Uplink media streaming session using multi-access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This present contribution relates to 5G Media Streaming (5GMS) systems defined in 3GPP TS 26.501 and related specifications (for example TR 26.804, TR 26.802). In particular, this contribution addresses “Network Assistance procedures” specified for media delivery sessions. This contribution applies to 3GPP SA4 work on “Study on Advanced Media Delivery Phase 2- FS_AMD_MED_Ph2”, the 3GPP Rel 20 study item that focusses on advanced media delivery architectures. The phase 2 of this study has just been approved in SA#109 and the work has begun in SA4. Several new Work Taks (WTs) have been recently added to the study. Among others, one key issue is extracted below, this is the target of this contribution. - WT#8: Multi-access media delivery phase 2: Clause 5.18.7 of TR 26.804 lists topics for further study on multi-access media delivery, including the impact on splitting M4 media flows when transported as GBR QoS flows; implications of the ATSSS architecture for UE multipath management, dynamic policy, network assistance, and network slicing procedures; and alignment with the study on media delivery from multiple service locations/endpoints. Additionally, based on discussion with 5G-MAG, the impact of multi-access media delivery for uplink streaming is to be studied. The proposed solution allows the UE application (App) or UE client to connect to multiple access paths to positively improve Quality of Experience (QoE) while leveraging the “network assistance” feature. This takes into account dynamic session conditions and is especially tailored for media streaming or content delivery between an Application Server (AS) and the UE, in the uplink direction. It is proposed to add the proposed content to the latest draft of TR 26.804 v19.1.0 under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.804 on “Clause 5.18 Mulit-access Media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change (All new text)

#### 5.18.4.4	Network Assistance and Delivery Boost for Multi-Access uplink media streaming using ATSSS

##### 5.18.4.4.0	Introduction

This clause provides details on how to enable a User Equipment (UE) device to determine the use of multi-access PDU Sessions (e.g., Wi-Fi and cellular) under Access Traffic Steering, Switching, and Splitting (ATSSS) frameworks during uplink media streaming sessions, through the use of the AF-based Network Assistance feature defined in clause 5.5.1.5.2 of the present document. It elaborates on how a 5GMSu-Aware Application or 5GMSu Client is required connect to multiple access paths to positively improve network Quality of Service in the uplink direction with the goal of improving service Quality of Experience (QoE) while leveraging the Network Assistance feature.

According to clause 4.0.5 of TS 26.501 [15] and clause 5.4.4 of TS 26.510 [108], a 5GMSu Client can request bit rate recommendations or delivery boosts from the 5GMSu AF. The 5GMSu AF interacts with PCF/SMF to adjust the network QoS or to provide throughput guidance in the form of bit rate recommendations. This takes into account dynamic session conditions and is especially tailored for content delivery between a UE and an Application Server (AS) in the uplink direction.

##### 5.18.4.4.1	Gaps in multi-access delivery path Network Assistance and Delivery Boost for multi-Access media delivery for uplink direction

##### In the context of multi-access PDU sessions, where traffic is distributed across both 3GPP and non-3GPP accesses using ATSSS, the specifications do not clarify:

-	Whether a Client can request or receive Network Assistance specific to a given access network (e.g., 3GPP NR vs. Wi-Fi).

-	How bit rate recommendations or boosts are applied per access when the session spans multiple access links.

-	How Network Assistance interacts with ATSSS steering/switching rules managed by PCF.

As a result:

-	The 5GMS Client receives only a session-level (aggregate) bitrate recommendation, not per-access insight.

-	Delivery boosts are applied at the PDU session level, not selectively to one access leg.

-	Uplink media delivery (e.g., live streaming, user-generated content) cannot dynamically exploit multiple paths.

-	There is no standardized signaling linkage between Network Assistance (M5 interface) and ATSSS policy control (Npcf, Nsmf).

In this clause these gaps are addressed, which calls for clarification of Network Assistance behaviour in multi-access scenarios in uplink direction. As stated above, in the current 5GMS architecture:

-	AF-based Network Assistance (per clause 4.0.5 of TS 26.501 [15] and clause 5.4.4 of TS 26.510 [108]) allows a 5GMS Client to:

-	Request bit rate recommendations, and

-	Request delivery boosts, both via the 5GMS Application Function (AF) using the M5 API.

These functionalities assume that the media streaming session runs over a single access (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current Network Assistance procedures lack clarity on a few key points. The definition of Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

-	AF-based Network Assistance is only available when the system is configured for it; it may not be universally enabled.

##### 5.18.4.4.2	Access-specific Network Assistance and Delivery Boost extensions to support multi-access

In this section we study access-specific Network Assistance extensions to the 5GMS control plane. These extensions enable a 5GMS Client and AF to perform per-access QoS interrogation and manipulation during ongoing multi-access uplink streaming sessions. This allows to optimize per-access bitrate and path selection to maximize end-user media delivery quality subject to dynamic network assistance and ATSSS constraints.

The following call flow illustrates the steps in this Candidate Solution for extending the AF-based Network Assistance feature for uplink media streamingto include multi-access (MA) functionality via the ATSSS process.

Figure 5.18.4.4.2-1: Procedures for access-specific Network Assistance and Delivery Boost extensions to multi-access

- PHASE 1: Initial Setup and Single Access Streaming

1. Provisioning

- The UE is provisioned with configuration data for the media service, including media access information, network assistance endpoints, etc.

2. Acquire Service Access Information

- The UE client retrieves access info for media services (e.g., via MbU or MsU interface).

3. Start Uplink Streaming Using Single Access

- The UE initiates streaming using a single network interface (e.g., Wi-Fi or cellular).

4–7. Establish Uplink Transport and Media Session

- Transport and streaming sessions are set up using a single access path.

- Uplink media (UL) data is transmitted using single access paths.

8. QoE Metrics Collection

- The application layer collects QoE metrics and reports them. Quality of Experience (QoE) metrics are monitored between the application and application server.

- PHASE 2: Multi-Access Activation

9. Detect Multiple Access Networks

- The UE detects the presence of multiple access networks (e.g., 4G and Wi-Fi available).

10–11. Request to Activate Multi-Access

- The UE modem informs the application about the presence of multi-access, and the application requests the media session handler (MSH) to request activation of multi-access (MA) mode.

12. Activate MA

- MSH in collaboration with the OS and modem enables multi-access, allowing simultaneous usage of multiple paths.

13. MA Activated

- MSH confirms the app that the Confirmation of activation of multi-access and the application could now attach to the multiple access technologies in order to improve its QoE.

14. Start UL Streaming with MA PON (Primary Operation Node)

- The App reaches the media access function (media streamer) in order for the media streaming to begin over multiple access links for improved performance.

- PHASE 3: Network Assistance Activation

15. Request to Activate Network Assistance (NA)

- Before beginning the uplink session, the app requests for bit rate recommendation (throughput estimation). In order to do this, the app asks the Media AF to enable network assistance for optimizing media delivery. This allows the UE to initiate an uplink streaming session using a bit rate that best matches the current network conditions, or to receive a recommended bit rate from the network. That recommendation remains valid throughout the streaming session unless updated. The suggested bit rate is determined based on the network's estimation or prediction of available uplink bandwidth. This capability serves as a supplementary mechanism to assist the UE, complementing the usual method where the UE estimates the appropriate bit rate based on its own measurements of past uplink traffic.

16. Create Network Assistance Session (M11)

- The MAF reaches the MSH so that a Network Assistance session for network assistance is initiated.

17–18. Estimate Recommended Bitrate

- The MSH now sends the list access points information to the AF. This is possible, thanks to steps 9-11, where the modem informs the app about the possible access points and the app informs this to the MSH. The AF calculates recommended bitrate parameters for each access type (e.g., Wi-Fi vs 5G NR) and reports tis the MSH.

19. Network Assistance Session Request and Response

- The MAF requests session setup and receives bitrate recommendations (e.g., 5 Mbps for LTE, 8 Mbps for Wi-Fi).

20. Set Initial Bitrate / Select Operation Point

- The MAF chooses an optimal bitrate and operational point based on recommendations.

21-22. Establish UL Streaming Using MA Access

- Uplink media streaming starts using multi-access with network assistance.

- PHASE 4: Subscription for Bitrate Updates

23. Uplink MA PDU Session Continues

- Session continues with network assistance active.

24. Subscribe for Bitrate Recommendation Updates

- The UE subscribes for ongoing bitrate recommendations as network conditions change. This is done via the MSH informing the AF.

25. Bitrate Update Notification

- The AF sends updated bitrate recommendations (e.g., due to congestion or throughput improvement).

26-27. MAF Adapts Bitrate

- This information is shared by the MSH to the MAF. The MAF adjusts encoding rate or representation according to new bitrate.

28. Continue Streaming

- Uplink streaming session continues with adapted bitrate under updated network assistance.

- PHASE 5: Delivery Boost (Handling QoE Drop)

29. Detect Sudden Drop in QoE

- The application detects a sudden drop in QoE; detects degraded QoE (e.g., high latency, jitter, or packet loss). This can be detected by comparing the on-going session QoE with the reported/collected QoE metrics in step 8.

30. Request to Activate Delivery Boost

- The app requests a delivery boost to the MAF. The 5GMSu Client uses this function to indicate to the network that a temporary boost, i.e., a temporary increase of network throughput for this client is needed, for example in order to prevent the uplink media streaming buffer in the Media Streamer from overflowing.

31. Request Delivery Boost (M11)

- MAF sends a request to activate a temporary performance boost to the MSH. A Delivery Boost session is created.

32. Detecting Sudden Drop

- The MSH confirms QoE drop detection event. The MSH also detects which access point needs a delivery boost based on access availability information, thanks to steps 9-11.

33–34. Invoke and Respond to Delivery Boost

- The MSH request the AF to give a temporary boost to that specific access point which requires a temporary boost. The Delivery Boost API is triggered; the network temporarily enhances QoS or bandwidth on that particular access point.

35. Notify Boost Result

- MAF receives confirmation that boost has been applied from MSH.

36. MAF Adjusts Bitrate/QoS

- MAF increases bitrate or quality level temporarily based on the above received information.

- PHASE 6: Session Termination

38. End Network Assistance Session

- The UE ends the network assistance session after streaming completes.

39. Delete Network Assistance Session

- The Media AF releases network resources.

40. Continue Without Network Assistance

- The session continues normally without network assistance (if still active) or ends fully.

## End of change