3GPP TSG-S4 Meeting  #134	S4-251924

# Dallas, USA, 17th November – 21st November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0016 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for Key issue on Client-driven switching from multipath to single path media delivery based on energy information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A key issue on client-driven management of media delivery service energy optimization was agreed during S4-MBS SWG AH. No solutions exists for this key issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of solution to key issue#6 on client-driven management of media delivery service energy optimization. The solution proposes to use the energy consumption information received from the network to make a decision on switching between multipath and single path transport sessions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study on energy management for media streaming would be incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | .. |  |  |  |  |  |  |  |  |

CHANGE 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[26804]	3GPP TR 26.804: "Study on 5G media streaming extensions".

[26512]	3GPP TS 26.512: "5G Media Streaming(5GMS); Protocols".

CHANGE 2

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #X |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

CHANGE 3 (AllS NEW TEXT)

## 7.X	Solution #X: Client-driven switching from multipath to single path media delivery based on energy information

### 7.X.1	Key Issue mapping

This Candidate Solution addresses Key Issue #6.

### 7.X.2	Functional description

#### 7.X.2.1	Introduction

Clause 5.18 of TR 26.804 [26804] describes the feature for multi-access media delivery. With this feature, the Media Stream Handler of a 5GMS Client may use multipath transport protocols such as MPTCP and MPQUIC to set up multipath transport session with the 5GMS AS for delivery of 5G Media Streaming content. Clause 13.2.4 of TS 26.512 [26512] specifies the Configuration and Settings API exposed by the Media Stream Handler to the 5GMS-Aware Application and Media Session Handler at reference points M7 and M11 respectively for configuration of the underlying transport session.

The multipath transport session may span one or more access network available to the UE. The multiple paths may be used to increase the resilience of the transport connection to packet loss or to increase data throughput as specified in clause 4.6.2 of TS 26.512 [26512].

The energy consumption in the UE and the network to support multi-access media delivery is often higher than is the case when the UE uses a single path for media delivery at reference point M4. At the UE, to support a multipath transport session over multiple access networks, the 5GMS Client requires activation and usage of multiple access network endpoints, potentially at the same time. In the network, a multi-access delivery session might be using multiple access network nodes such as one or more gNodeBs, zero or more Non-3GPP nodes such as N3IWF etc. for transport of media application session content. The benefit of increased resilience with multipath transport often comes at the cost of higher energy and power consumption.

This Candidate Solution introduces a mechanism for switching from a multipath transport session, potentially over one or more access networks, to a single-path session based on energy consumption information sent from the network to the UE.

#### 7.X.2.2	UE client APIs for network selection

##### 7.X.2.2.1	Android

Editor’s Note:	TODO.

##### 7.X.2.2.2	iOS

Editor’s Note:	TODO.

#### 7.X.2.3	Access Network Energy Cost Information

To determine the relative cost of establishing a transport connection over one or other type of Access Network available to the UE, it needs to be provided with at least the information outlined in table 7.X.2.3-1.

Table 7.X.2.3-1: Baseline Access Network Energy Cost Information

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

#### 7.X.2.4	Reference architecture for client-driven switching from multipath to single path based on energy information from the network

Based on the reference architecture instantiated in clause 7.6.2.3, figure 7.X.2.4-1 depicts a reference architecture that realises this candidate solution in the 5G Media Streaming architecture defined in TS 26.501 [23]. No new interfaces between the 5GMS entities in the network and the UE are defined in this solution. This solution uses existing media streaming reference points in TS 26.501 [23].

NOTE:	The Energy Information Collector and reference point E5 are not instantiated in this reference architecture because they are not required by the Candidate Solution.

Figure 7.X.2.4-1: Reference architecture for client driven switching from multipath transport session to a single path transport session

The Energy Information Function performs collection of energy consumption information for the application session (a multipath transport session carrying 5G Media Streaming content in this case) as specified in clause 5.51.2.2 of TS 23.501 [72]. This solution delivers this energy consumption information to the 5GMS AF, which then forwards the information to the 5GMS Client over E5. The 5GMS Client and the 5GMS-Aware Application then decide whether to switch to a single path transport session from an ongoing multipath transport session.

The solution reuses an existing function already defined for collection and exposure of energy-related information:

-	The Energy Information AF instantiated in the 5GMS AF.

### 7.X.3	Procedures

Figure 7.X.3-1 details the steps for the procedure on client-driven switching from a multipath transport session to a single-path transport session as outlined in clause 7.X.2.

Figure 7.X.3-1: Procedures for client-driven switching of a 5G Media Streaming session from multipath to a single path session

1.	A 5G Media Streaming session is established between the 5GMS entities in the UE and those in the network over a multipath transport session. The multipath transport session may use multiple access networks (via a 3GPP access and 3GPP/Non-3GPP access) as described in clause 5.18.4 of TR 26.804 [26804].

2.	The Media Session Handler configures the EIC to subscribe to energy events from the EIAF

3.	The EIC subscribes to receive energy events over reference point E5 from the EIAF.

4.	M4 media flows are exchanged between the Media Stream Handler in the UE’s 5GMS Client and the 5GMS AS, over one or more access network paths.

5.	The Energy Information Function provides energy information related to the 5G Media Streaming session to the EIAF inside the 5GMS AF at different granularities as specified in clause 5.51.2.2 of TS 23.501[23501]. The energy information includes estimates of energy consumption for each of the access networks that are being used by the 5G Media Streaming session. This information may be relayed via NEF if the 5GMS AF is untrusted.

6.	The EIAF forwards the energy consumption information for the 5G Media Streaming session to the EIC in the Media Session Handler over reference point E5.

7.	The energy information for the 5G Media Streaming session is forwarded to the Media Session Handler

8.	The Media Session Handler and the 5GMS-Aware Application decide whether to switch from the multipath transport session over multiple access networks to a single-path transport session. In making this decision, the 5GMS-Aware Application and the Media Session Handler may consider the energy consumption inside the UE to support the multipath session over multiple access networks along with the information received from the 5GMS AF.

9.	The Media Session Handler may update the multipath transport session parameters as described in clause 13.2.4 of TS 26.512 [26512].

10.	The Media Stream Handler applies the updated configuration to the 5G Media Streaming transport session.

11.	The Media Stream Handler in the UE and the 5GMS AS switch to a single-path transport session for carrying the M4 application flows of the 5G Media Streaming session.

end of CHANGEs