3GPP TSG-S4 Meeting  #134	S4-251930

# Dallas, USA, 17th November – 21st November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0036 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] Network Assistance for multi-access media delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G-MAG requested support for network assistance with uplink streaming use cases when using multi-access media delivery. Rel-19 TR 26.804 also documents gaps on this topic. Please see attached discussion document for more information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a key issue description on supporting network assistance for multi-access media delivery over multiple access networks |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-access media delivery feature is incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1.4A (new), 5.18.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | .. |  |  |  |  |  |  |  |  |

CHANGE-1
(ALL NEw TEXT)

#### 5.18.1.4A	Network Assistance with multi-access media delivery

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedures for the Network Assistance feature in 5G Media Streaming. Network Assistance enables the 5GMS Client in the UE to interrogate the network Quality of Service (QoS) for an ongoing media streaming session or to request a temporary boost in network QoS. Two methods for obtaining network assistance are defined in TS 26501 [15]:

-	AF-based Network Assistance, where in the 5GMS Client requests the network-side component of the 5GMS System, the 5GMS AF, for assistance, and the 5GMS AF interacts with 5G System network components, such as the PCF, to provide the requested assistance to the 5GMS Client.

-	ANBR-based Network Assistance, which is based on signalling interactions between the UE modem and RAN control/user plane entities. The RAN control plane entities interact with 5G System network components to provide Network Assistance to the requesting UE

With either of the above two methods above, the following network assistance facilities may be available to the 5GMS Client:

-	Bit rate recommendation, where in a 5GMS Client requests an estimate, from 5GMS AF, of a bit rate that can be offered to the current 5G Media Streaming session. The 5GMS AF, based on this request from the 5GMS Client, queries 5G System network components such as the PCF to obtain this information, and informs the 5GMS Client of the same. The 5GMS Client may use this information to adjust its own session parameters – e.g., the streaming bit rate to fit within the QoS that the network is able to offer.

-	Delivery boost, where in a 5GMS Client requests for temporary boost of bit rate for the current media streaming session from the network, and the 5GMS AF interacts with 5G System components such as the PCF to facilitate the boost

When a 5G Media Streaming session is conveyed over a Single Access PDU Session, the 5GMS Client may receive network assistance using the above assistance procedures from the network. However, when a 5G Media Streaming session is conveyed over a Multi-Access PDU Session, it is not clear whether the information procured using currently specified network assistance procedures is useful or enough for the 5GMS Client.

CHANGE-3

##### 5.18.1.5.2	Key Issue objectives

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a 5G Media Streaming session, it is not clear how the Dynamic Policy feature specified in TS 26.501 [15] and TS 26.510 [108] is activated and implemented for application flows over multiple access networks.

Specifically, the following issues need to be studied:

-	If M4 application flows are carried over two access networks, what does "activate dynamic policy with QoS requirements" mean – whether the requested network QoS is applicable to one, or more, or all access paths.

-	Is it feasible to request QoS for a subset of access paths over specific access networks?

-	Are any enhancements to the ApplicationFlowDescription type described in TS 26.510 [108] needed to support identification of M4 application flows over multiple access networks?

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a 5G Media Streaming session, the following issues need to be studied to specify network assistance for multi-access media delivery:

-	If M4 application flows are carried over multiple access networks, and the 5GMS Client intends to use network assistance for adapting its behaviour, whether the current bit rate recommendation and delivery boost network assistance facilities are enough to support the 5GMS Client, or enhancements to assistance information is required?

-	Whether and how the 5GMS Client use current bit rate recommendation information from the 5GMS AF to distribute application flows at reference point M4 over one or more access networks.

-	Whether the 5GMS Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4d are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document?

-	Whether the 5GMS Client is able to request, or receive, network assistance for application flows at reference point M4d over a specific access network?

-	Whether and how the network assistance information from the network to the 5GMS Client include the information such as below:

-	Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access and 70% over another access)

-	Split rate recommendation of application flows at reference point M4 over one or more access networks based on type of media being sent (e.g., audio over one access, video on another access)

-	Split recommendation of application flows at reference point M4 over one or more access networks based on stream priority

-	Initial recommended throughput with associated latency over one or more access networks

-	Other information such as upcoming network conditions, upcoming network handover etc.

CHANGE-4

#### 5.18.2.3	Network Assistance and Delivery Boost for Multi-Access uplink media streaming using ATSSS

##### 5.18.2.3.1	Introduction

This clause provides details on how to enable a User Equipment (UE) device to determine the use of multi-access PDU Sessions (e.g., Wi-Fi and cellular) under Access Traffic Steering, Switching, and Splitting (ATSSS) frameworks during uplink media streaming sessions, through the use of the AF-based Network Assistance feature defined in clause 5.5.1.5.2 of the present document. It elaborates on how a 5GMSu-Aware Application or 5GMSu Client is required connect to multiple access paths to positively improve network Quality of Service in the uplink direction with the goal of improving service Quality of Experience (QoE) while leveraging the Network Assistance feature.

According to clause 4.0.5 of TS 26.501 [15] and clause 5.4.4 of TS 26.510 [108], a 5GMSu Client can request bit rate recommendations or delivery boosts from the 5GMSu AF. The 5GMSu AF interacts with PCF/SMF to adjust the network QoS or to provide throughput guidance in the form of bit rate recommendations. This takes into account dynamic session conditions and is especially tailored for content delivery between a UE and an Application Server (AS) in the uplink direction.

CHANGE-5

##### 5.18.5.2.5	Gaps in multi-access delivery path Network Assistance and Delivery Boost for multi-Access media delivery for uplink direction

##### In the context of multi-access PDU sessions, where traffic is distributed across both 3GPP and non-3GPP accesses using ATSSS, the specifications do not clarify:

-	Whether a Client can request or receive Network Assistance specific to a given access network (e.g., 3GPP NR vs. Wi-Fi).

-	How bit rate recommendations or boosts are applied per access when the session spans multiple access links.

-	How Network Assistance interacts with ATSSS steering/switching rules managed by PCF.

As a result:

-	The 5GMS Client receives only a session-level (aggregate) bitrate recommendation, not per-access insight.

-	Delivery boosts are applied at the PDU session level, not selectively to one access leg.

-	Uplink media delivery (e.g., live streaming, user-generated content) cannot dynamically exploit multiple paths.

-	There is no standardized signaling linkage between Network Assistance (M5 interface) and ATSSS policy control (Npcf, Nsmf).

In this clause these gaps are addressed, which calls for clarification of Network Assistance behaviour in multi-access scenarios in uplink direction. As stated above, in the current 5GMS architecture:

-	AF-based Network Assistance (per clause 4.0.5 of TS 26.501 [15] and clause 5.4.4 of TS 26.510 [108]) allows a 5GMS Client to:

-	Request bit rate recommendations, and

-	Request delivery boosts, both via the 5GMS Application Function (AF) using the M5 API.

These functionalities assume that the media streaming session runs over a single access (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current Network Assistance procedures lack clarity on a few key points. The definition of Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

-	AF-based Network Assistance is only available when the system is configured for it; it may not be universally enabled.

end of CHANGEs