3GPP SA4-e (AH) MBS SWG post 134	S4aI25XXXX

# Online, 18 December 2025	   is revision of S4-252084

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0006 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI6 Client-driven management of media delivery service energy optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-12-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.11 adding a a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

2nd change
(All new text)

## 7.11	Solution #10: Client-driven selection of stream variants or delivery paths in the generalised Media Delivery System based on energy characteristics

### 7.11.1	Key Issue mapping

This Candidate Solution addresses Key Issue #6.

### 7.11.2	Functional description

#### 7.11.2.1	Introduction

Currently, media players lack the capability to select stream variants or delivery paths based on their environmental impact, in addition to conventional network Quality of Service (QoS) considerations. This limitation hinders the ability to optimize energy efficiency during media playback while still maintaining an acceptable Quality of Experience.

The proposed solution introduces a mechanism for sharing energy-related characteristics throughout the technical delivery chain. This enables more energy-efficient media playback by allowing client devices to make informed decisions about stream selection and delivery paths, considering their environmental impact alongside QoS.

This solution specifically addresses the mechanism for providing energy-related characteristics to the UE. The decision-making process for optimizing media delivery is not detailed here, either because it is managed directly by the Media Access Function's heuristics for optimizations such as selecting stream variants or because it is already supported by the Media Delivery System. For example, procedures for downlink streaming to Media Players with different presentation manifests (clause 5.2.4 in TS 26.501 [23]) or for streaming from multiple service locations (clause 5.2.6 in TS 26.501 [23]) are already part of the existing 5GMS downlink media streaming procedures.

This solution also takes into consideration end-user energy preferences configured in the UE Application. Three different modes are available in the UE Application to simplify the configuration:

-	Eco mode: Prioritizes energy efficiency, even at the slight expense of Quality of Experience.

-	Standard mode: Balance between energy efficiency and Quality of Experience.

-	Performance mode: Prioritizes Quality of Experience over energy consumption.

#### 7.11.2.2	Reference architecture

Figure 7.11.2.2-1 depicts a reference architecture that realises this Candidate Solution in the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59]. The core principle of this architecture is to leverage the existing method for collecting and exposing energy-related characteristics, as already outlined in Solution #5 (see clause 7.6.2.4). Additionally, it extends this approach by enabling the Media Client to select stream variants or delivery paths based on this information.

Figure 7.11.2.2-1: Reference architecture for client-driven selection of stream variants or delivery paths based on energy characteristics in the generalised Media Delivery System

The core principle of this architecture is to leverage the existing method for collecting and exposing energy-related characteristics, as already outlined in Solution #5 (see clause 7.6). Additionally, it extends this approach by enabling the Media Client to select stream variants or delivery paths based on this information.

The solution is reusing functions already defined for collection and exposure of energy-related information in clause 7.6.2.4:

-	The Energy Information AF instantiated in the Media AF.

-	The Energy Information Collector.

-	Media Client: A UE-internal function dedicated to Media Delivery comprising:

-	Media Session Handler: An entity on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media delivery session.

-	Media Access Function: An entity on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

-	The Energy Information Collector instantiated in the Media Session Handler of the Media Client communicates with the Media Access Function of the Media Client via existing reference point M11..

The solution also reuses reference points M1, M3, E3, M4, M5, E5, M6, M8, M11 and E12 already defined for collection and exposure of energy-related information in the instantiation in generalised media delivery architecture presented in clause 7.6.2.4.

### 7.11.3	Procedures

Based on the procedures outlined in clause 7.6.3, figure 7.11.3-1 below details additional steps suppporting client-driven selection of stream variants or delivery paths based on energy characteristics outlined in clause 7.11.2.2.

Figure 7.11.3-1: Procedures for client-driven selection of stream variants or delivery paths based on energy characteristics in the generalised Media Delivery System

Steps 1 to 16 are not detailed since they are already described in the baseline procedure in clause 7.6.

17.	The Media Session Handler processes energy information received from the different UE components and aggregates them. As this energy information is not specific to this solution, it is out of scope of this solution. an illustrative example could include:

-		The Network Energy consumption per hour of media delivery.

-	The Network Carbon footprint per hour of media delivery.

Step 18 (reconfiguration of the Media Access Function by the Media Session Handler) is omitted.

Step 19 is as described in the baseline procedure.

The steps particular to this Candidate Solution are as follows:

19a.	The Media Session Handler provides the Media Entry Points to the Media-aware Application. The Media Entry Point descriptions are embellished by the Media AF with energy information based on the information it received in steps 15.

19b.	The Media-aware Application selects the appropriate Media Entry Point according to Media Application Provider configuration and/or the energy mode configured by the end-user.

19c.The Media Access Function is invoked with the selected Media Entry Point to start media delivery, per the baseline call flow for the Media Delivery System in question.

19d.	The Media Access Function acquires the Media Entry Point from one or more Media AS service locations according to the current configuration of the Media Access Function.

19e.	Media content is delivered between the Media Access Function and one or more Media AS service locations according to the current configuration of the Media Access Function.

Steps 11 to 19 inclusive are repeated in parallel with media delivery (step 19e). In this case, step 18 may not be omitted:

18.	If the configuration selected by the Media Session Handler in the previous step is different, the Media Session Handler reconfigures the Media Access Function.

19.	The Media Session Handler may instantiate a new Dynamic Policy (or modify an existing one) on the Media AF.

Steps 20 and 21 (exposing energy-related information) are identical to the baseline procedure.

### 7.11.4	Summary

The proposed solution enables the integration of energy-related characteristics into multimedia content delivery systems. It empowers Media Clients to choose stream variants or delivery paths based on their environmental impact, while also considering user preferences that balance the trade-off between Quality of Experience and environmental sustainability.

This solution is based on Network Energy Information available from the Energy Information Function as well as the two new functional entities defined in Solution #5 (see clause 7.6): the Energy Information AF and the Energy Information Collector. Leveraging existing mechanisms of the media delivery system, the Media Client is able to make use of energy-related information provided by these functions to optimize energy efficiency during media delivery sessions, while maintaining the trade-off between Quality of Experience and environmental sustainability configured by the user.

This solution offers several advantages:

1.	Reduced environmental impact: The energy consumption and consequent CO2 emissions associated with multimedia content delivery may be minimised.

2.	User transparency: The environmental impact of content consumption is made visible without impacting privacy.

3.	Flexibility: The approach is compatible with existing 5G System optimizations.

4.	Customization: Thanks to the energy optimization modes, End users can be given the freedom to choose their level of environmental commitment.

5.	Global optimization: The entire delivery chain, from distribution to consumption, is taken into account.

End of changes