3GPP TSG-S4 Meeting  #134	S4-252099

# Dallas, USA, 17th November – 21st November 2025	revision of S4-252078

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0036 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] Network Assistance for multi-access media delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G-MAG requested support for network assistance with uplink streaming use cases when using multi-access media delivery. Rel-19 TR 26.804 also documents gaps on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a key issue description on supporting network assistance for multi-access media delivery over multiple access networks. Also updates the gaps and adds a candidate solution to this Key Issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-access media delivery feature is incomplete. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2., 5.18.1.4A (new), 5.18.1.5.2, 5.18.2.3 (new), 5.18.4.3 (new), 5.18.5.2.5 (new), 5.18.6.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-251930: Initial contribution to propose overview and key issue description on network assistance with multi-access media delivery. S4-252078: Updated previous version with comments during SA4#134 meeting. S4-252099: Merge of candidate solution from CR0034r1 [S4-252060]. |  |  |  |  |  |  |  |  |

First Change

## 2	References

[26512]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[28405]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

Key Issue Description
(ALL NEw TEXT)

#### 5.18.1.4A	Network Assistance with multi-access media delivery

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedures for the Network Assistance feature in 5G Media Streaming. Network Assistance enables the Media Client in the UE to interrogate the network Quality of Service (QoS) for an ongoing media delivery session or to request a temporary boost in network QoS. Two methods for obtaining network assistance are defined in clause 4.0.5 of TS 26.501 [15]:

-	AF-based Network Assistance, where in the Media Client requests the network-side component of the media delivery System, the Media AF, for assistance, and the Media AF interacts with 5G System network components, such as the PCF, to provide the requested assistance to the Media Client. Media Client procedures are specified in clause 5.4.4 of TS 26.510 [108].

-	ANBR-based Network Assistance, which is based on signalling interactions between the UE modem and RAN control/user plane entities. The RAN control plane entities interact with 5G System network components to provide Network Assistance to the requesting UE.

With either of the above two methods above, the following network assistance facilities may be available to the Media Client:

-	Bit rate recommendation, where in a Media Client requests an estimate, from Media AF, of a bit rate that can be offered to the current media delivery session. The Media AF, based on this request from the Media Client, queries 5G System network components such as the PCF to obtain this information, and informs the Media Client of the same. The Media Client may use this information to adjust its own session parameters (e.g., the streaming bit rate) to fit within the QoS that the network is able to offer.

-	Delivery boost, where in a Media Client requests for temporary boost of bit rate for the current media delivery session from the network, and the Media AF interacts with 5G System components such as the PCF to facilitate the boost.

When a media delivery session is conveyed over a Single Access PDU Session, the Media Client may receive network assistance using the above assistance procedures from the network. However, when a media delivery session is conveyed over a Multi-Access PDU Session, it is not clear whether the information procured using currently specified network assistance procedures is useful or enough for the Media Client.

Current specifications assume that the media delivery session runs over a single access network (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current AF-based Network Assistance procedures lack clarity on a few key points. The current specification of AF-based Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

Key Issue Objectives

##### 5.18.1.5.2	Key Issue objectives

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a media delivery session, it is not clear how the Dynamic Policy feature specified in TS 26.501 [15] and TS 26.510 [108] is activated and implemented for application flows over multiple access networks.

Specifically, the following issues need to be studied:

-	If M4 application flows are carried over two access networks, what does "activate dynamic policy with QoS requirements" mean – whether the requested network QoS is applicable to one, or more, or all access paths.

-	Is it feasible to request QoS for a subset of access paths over specific access networks?

-	Are any enhancements to the ApplicationFlowDescription type described in TS 26.510 [108] needed to support identification of M4 application flows over multiple access networks?

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a media delivery session, the following issues need to be studied to specify Network Assistance for multi-access media delivery:

-	If M4 application flows are carried over multiple access networks, and the Media Client intends to use Network Assistance for adapting its behaviour, whether the current bit rate recommendation and delivery boost Network Assistance facilities are enough to support the Media Client, or enhancements to assistance information is required.

-	Whether and how the Media Client uses current bit rate recommendation information from the Media AF to distribute application flows at reference point M4 over one or more access networks.

-	Whether the Media Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4 are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document.

-	Whether the Media Client is able to request, or receive, network assistance for application flows at reference point M4 over a specific access network.

-	Whether and how the network assistance information from the network to the Media Client include the information such as below:

-	Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access network and 70% over another access network).

-	Split rate recommendation of application flows at reference point M4 over one or more access networks based on type of media being sent (e.g., audio over one access network, video on another access network).

-	Split recommendation of application flows at reference point M4 over one or more access networks based on stream priority.

-	Initial recommended throughput with associated latency over one or more access networks.

-	Other information such as upcoming network conditions, upcoming network handover etc.

Collaboration Scenarios
(All New text)

#### 5.18.2.3	AF-based Network Assistance for multi-access media delivery using ATSSS

Extending the collaboration scenario outlined in clause 5.18.2.2, this clause describes a scenario in which a User Equipment (UE) device can determine the use of multi-access PDU Sessions (e.g., Wi-Fi and cellular) under ATSSS during a media delivery sessions, through the use of the AF-based Network Assistance feature described in clause 5.5.1.5.2. It elaborates on how a Media-aware Application or Media Client is required to connect to multiple access paths to positively improve network Quality of Service with the goal of improving service Quality of Experience (QoE) while leveraging the Network Assistance feature.

Architecture MappingS

Editor’s Note: Assume mapping in clause 5.18.3.2 (ATSSS mapping into 5GMS architecture) is applicable.

High-level Call FlowS
(All new text)

##### 5.18.4.3	Access-specific Network Assistance and Delivery Boost extensions to support multi-access

Based on the architecture mapping in clause 5.18.3.2, figure 5.18.4.3-1 below illustrates a call flow for access-specific Network Assistance extensions to the Media Delivery System based on the use of ATSSS procedures. The extensions highlighted in blue and boldface enable a Media Client and a Media AF to perform per-access QoS interrogation and manipulation during ongoing multi-access media delivery sessions. This allows to optimize per-access bit rate and path selection to maximize end-user media delivery quality subject to dynamic network assistance and ATSSS constraints.

Figure 5.18.4.4.2-1: Procedures for AF-based Network Assistanceextensions to support multi-access

In the initial phase, a media streaming session is established over a single access network:

1.	Provisioning: The Media Client in the UE is provisioned with configuration data for the media service, including Service Access Information, etc.

2.	Acquire Service Access Information: The Media Session Handler in the Media Client retrieves access info for media services (e.g., via Mb or Ms interface).

3.	Start media delivery using single access network: The Media Client initiates streaming using a single network interface (e.g., Wi-Fi or cellular).

4–7.	Establish  transport session for  media delivery session: Transport and streaming sessions are set up using a single access path.  Media data is transmitted using a single access path.

8.	QoE metrics collection and reporting: The Media Client collects Quality of Experience (QoE) metrics and reports them. QoE metrics are monitored between the Media Client and the Media AS.

Multi-access is activated for the media streaming session:

9.	Detect multiple access networks: The UE modem detects the presence of multiple access networks (e.g., 4G and Wi-Fi available).

10–11.	Request to activate multi-access: The UE modem informs the Media-aware Application about the presence of multi-access, and the application requests the Media Access Client to activate multi-access mode. In the case of downlink media streaming, this is achieved implicitly by setting a preference for mutlipath transport and by specifying a multipath-capable transport protocol in the Media Player settings, as specified in clause 13.2.4 of TS 26.512 [16].

12.	Activate multi-access: The Media Session Handler, in collaboration with the Operating System (OS) and modem, enables multi-access, allowing simultaneous usage of multiple paths.

13.	Multi-access activated notification: The Media Session Handler confirms to the Media-aware Application that the application could now attach to the multiple access technologies in order to improve its QoE.

14.	?

15.	?

16.	Start media delivery with MA PON (Primary Operation Node): The Media-aware Application reaches the media access function (media streamer) in order for the media streaming to begin over multiple access links for improved performance.

AF-based Network Assistance is activated:

17.	Request to Activate Network Assistance: The app requests assistance from the network in order to improve bit rate recommendation (throughput estimation) for each access type it is attaached. The identification of access type by the Media AF, non-3GPP and 3GPP is as per clause 6.1.of TS 23.503 [41]. The identification of same access types, for instance same 3GPP access types is for further study.

18.	Create Network Assistance Session (M11): The Media Access Function reaches the MSH so that a Network Assistance session for network assistance is initiated.

17–18.	Estimate recommended bit rate: The Media Session Handler now sends the list access types information to the Media AF. (This is possible, thanks to steps 9-11, where the modem informs the Media-aware Application about the possible access points and the app informs this to the Media Session Handler.)

The Media AF calculates recommended bit rate parameters for each access network type (e.g., non-3GPP, e.g.Wi-Fi vs 3GPP e.g.5G NR) and reports this to the Media Session Handler. Alternatively, the Media Session Handler may receive the access type information from the Media AF as per clause 6.1 of TS 23.503 [41]. This information is known to the Media AF by the “Access network information reporting” as per clause 6.6.1 of TS 23.503 [41].

19.	Network Assistance Session request and response:The Media Access Function requests session setup and receives bitrate recommendations (e.g., 5 Mbps for LTE, 8 Mbps for Wi-Fi).

20:	?

21:	?

22.	Set initial bit rate: The Media Access Function chooses an optimal bit rate and operational point based on recommendations.

23-24. Establish media delivery using multi-access: Media delivery starts using multiple access access networks with network assistance.

The Media Client subscribes to receive bit rate recommendations:

25.	 Multi-access PDU Session continues: Session continues with network assistance active.

26.	Subscribe to bit rate recommendations: The Media Session Handler subscribes to receive ongoing bit rate recommendations from the Media AF as network conditions change.

27.	Bit rate update notification: The Media AF sends updated bit rate recommendations (e.g., due to congestion or throughput improvement) to the Media Session Handler.

28–29.	Media Access Function adapts bit rate: This information is shared by the Media Session Handler with the Media Access Function via reference point M11. The Media Access Function adjusts encoding rate or representation according to new bit rate.

30.	Continue streaming: Media streaming session continues with the adapted bit rate recommended by the Media AF.

The Media Client requests a delivery boost:

31. UE detects a sudden drop in QoE: The Media-aware Application detects degraded QoE (e.g., high latency, jitter, or packet loss). This can be detected by comparing the ongoing session QoE with the historic QoE metrics collected in step 8, using for instance QMC framework as per TS 28.405 [28405].

32. Request to activate delivery boost: The Media-aware Application requests a delivery boost to the Media Access Function. The Media Client uses this function to indicate to the network that a temporary boost, i.e., a temporary increase of network throughput is needed for this media delivery session, for example in order to prevent the uplink  media streaming buffer in the Media Access Function from overflowing.

33.	Request delivery boost: The Media Access Function sends a request to the Media Session Handler at reference point M11 to activate a temporary performance boost, for example using the requestDeliveryBoost() method specified in clause 11.4.1.2 of TS 26.510 [26510] for downlink media streaming. For uplink media streaming, no equivalent API is specified. For RTC sessions, ????????.

34.	Detecting sudden drop: The Media Session Handler confirms QoE drop detection event. The Media Session Handler also determines which access network type needs a delivery boost, based on access availability information, thanks to steps 9–11.

35–36.	Invoke and respond to delivery boost: The Media Session Handler requests the Media AF to give a temporary boost to the specific access point requiring this. The requestDeliveryBoost operation of the Maf_SessionHandling_NetworkAssistance API specified in clause 9.4.2 of TS 26.510 [26510] is triggered; the network temporarily enhances QoS or bandwidth on that particular Data Network.

37.	Notify boost result: TheMedia Access Function receives confirmation that boost has been applied from the Media Session Handler at reference point M11. For downlink media streaming, for example, the response to the requestDeliveryBoost() method (step 33) is a Boolean value confirming success or failure. For uplink media streaming, no equivalent API is specified. For RTC sessions, ????????

36.	Media Access Function adjusts bit rate: The Media Access Function increases bit rate temporarily based on the above received information. For example, it may attempt to replenish its media segment buffer (downlink media streaming).

Finally, the media streaming session is terminated:

39.	End Network Assistance Session: When media streaming completes, the UE application ends the Network Assistance Session.

40.	Destroy Network Assistance Session: The Media Session Handler destroys the Network Assistance Session by invoking the destroyNetworkAssistanceSession operation of the Maf_SessionHandling_NetworkAssistance service on the Media AF at reference point M5. The Media AF releases network resources.

41.	Continue without Network Assistance: If still active, the session continues normally without Network Assistance or ends fully.

Gap Analysis
(All New TeXT)

##### 5.18.5.2.5	Gaps in AF-based Network Assistance for multi-access media delivery using ATSSS

A number of gaps are highlighted in the call flow presented in clause 5.18.4.3. These are summarised below to motivate normative changes to the Media Delivery specifications.

In the context of multi-access PDU sessions, where traffic is distributed across both 3GPP and non-3GPP access networks using ATSSS, the media delivery specifications do not clarify:

-	Whether a Media Client can request or receive AF-based Network Assistance specific to a given access network (e.g., 3GPP NR vs. Wi-Fi).

-	How bit rate recommendations are obtained or boosts applied for different access networks when the media delivery session spans multiple access links.

-	How Network Assistance interacts with ATSSS steering/switching rules managed by PCF.

As a result:

-	Depending on implementation, the Media Client currently receives only a session-level (aggregate) bit rate recommendation, not per-access insight.

-	Delivery boosts are applied at the PDU Session level, not selectively to one access leg.

-	Uplink media delivery (e.g., live streaming, user-generated content) cannot dynamically exploit multiple paths.

-	There is no standardized signalling linkage between AF-based Network Assistance at reference point M5 and ATSSS policy control (Npcf followed by Nsmf).

These functionalities assume that the media delivery session runs over a single access (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current AF-based Network Assistance procedures lack clarity on a few key points. The current specification of AF-based Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

Candidate Solutions
(All new text)

#### 5.18.6.3	AF-based Network Assistance for multi-access media delivery using ATSSS

To address the gaps identified in clause 5.18.5.5, it is proposed to

Summary and Conclusions

### 5.18.7	Summary and conclusions

Multi-access media delivery enables media streaming applications to efficiently access content over multiple access networks. This Key Issue has examined existing specification relating to the ATSSS (Access Traffic Steering Switching and Splitting) architecture in TS 23.501 [23] and TS 23.502 [24] to identify its impact on 5GMS. Topics relating to application awareness and influence on multi-access delivery, potential enhancements to dynamic policy feature of 5GMS to support multiple access paths, and network assistance with multi-access delivery have been studied. The Key Issue has documented collaboration scenarios and the mapping of the ATSSS architecture into the 5GMS architecture.

The MPTCP and MPQUIC link-specific multipath IP addresses are not routable via N6 as of current release, and therefore identification of specific paths for any 5G Media Streaming procedures is not supported. Further, traffic splitting for GBR QoS Flows is not supported. If M4 media flows are transported as GBR QoS Flows, then traffic splitting of M4 media flows using ATSSS is not supported in this release, and the study is to be revisited in a future release.

The following recommendations were adopted in Release 19 deliverables:

1.	An informative annex is added to TS 26.501 [23] documenting:

a.	A brief description of multi-access media delivery, based on clause 5.18.1 of the present document.

b.	The mapping of the ATSSS architecture into the 5GMS architecture, as described in clause 5.18.3.2 of the present document.

2.	Changes to the Configuration Settings API and to the Dynamic Status Information API as described in clause 5.18.6.2 of the present document are implemented in TS 26.510 [108] to allow for application configuration and status information exchange for multi-access media delivery.

Based on the candidate solution described in clause 5.18.6.3, the following modifications to 3GPP specifications are recommended:

3.	.

end of CHANGEs