3GPP TSG-SA WG4 Meeting #134 Meeting	S4-251824r02 is revision of S4-251403

Dallas, Texas 17th – 23rd November 2025

# Title:	Reply LS on clarification on reply SDP for standalone DC

# Response to:

# Release:	Release 19

# Work Item:	NG_RTC_Ph2

Source:	SA4

To:	SA2

Cc:

Contact Person:

Name:	Daniel VENMANI

E-mail Address:	daniel (DOT) venmani (AT) nokia (DOT) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S4-251401

Question from SA2: SA2 has discussed one scenario of standalone IMS data channel session in which an originating UE initiates SIP INVITE request towards terminating UE with an initial SDP offer including BDC media, ADC media, and APP ID associated with the ADC. In this scenario, if the DC application is not available, i.e., needs to be downloaded by the terminating UE, it needs to indicate in the SDP answer to the initial SDP offer that the ADC is not able to be established due to downloading required.

SA2 would like to ask SA4 to provide feedback on how to specify the required response as depicted above, in the SDP response to the received SDP offer.

Answer from SA4: SA4 kindly responds to SA2 to the above question as follows:

In this particular case when the DC application is not available at the terminating UE, i.e., needs to be downloaded by the terminating UE, the terminating UE needs to indicate in the SDP answer to the initial SDP offer that the ADC is not able to be established due to downloading required; the SDP offer shall indicate this via the newly defined "a=3gpp-req-app" line "app-dc-status" parameter with an "app-dc-status-value" of "pending" shall be included in an SDP offer. , as defined by clause 6.2.10 of TS 26.114, to indicate the offering UE by the terminating UE as part of the media description creating application data channels for that application before the data channels are to be created. The terminating UE could re-send an offer to the originating UE once the app is downloaded.

A detailed description of the above is included as CR in attachment.

2. Actions:

To SA2:

ACTION:	SA4 kindly asks SA2 to take the above reply into account in response to the feedback on how to specify the required response in the SDP response to the received SDP offer.

3. Date of Next TSG SA WG4 Meetings:

SA4#134	17th -21st November 2025			Dallas, Texas

SA4#135	9th -13th February 2025			             Goa, India