| CR-Form-v9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TR 26.9de |  |  |  | rev |  | - | Current version: | 0.0.1 |  |  |  |
| Title: |  |  | Pseudo-CR on Stereoscopic Video Formats |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | ETRI, NOKIA Corporation |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  | 17/08/2010 |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  |  | Text proposal for IVCS TR |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  | This P-CR includes text proposal for stereoscopic video formats. |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 3D Stereoscopic Video Formats

There are 2 major ways of formatting the views of a stereoscopic video: spatial compression and temporal interleaving. Other formats such color shifting and 2D+Depth are possible but are either outdated or still subject to research and development.

Finally, the left and right views may also be encoded as separate views, possibly exploiting redundancies between the two views to enhance the compression efficiency. This technique is standardized by MPEG as part of the H.264/AVC standard.

In the following, the most relevant formats are described. SVAF[2] includes ISO based file format for the formats.

#### Frame Compatible Video

This technique uses spatial compression to pack the two views of the stereoscopic video into a single frame (thus the name frame compatible). This allows the usage of deployed encoding and transport infrastructure and keeping similar bandwidth requirements at the cost of information loss. The two views are first down-sampled and then packed. The down-sampling may be performed horizontally, vertically, or diagonally. The packing may use a side-by-side, top-bottom, interleaved, or checkerboard format. The different alternatives are illustrated in the following figures.

Figure 1 Spatial Packing Formats

#### Temporal Interleaving

In temporal interleaving, the video is encoded at double the frame rate of the original video. Each pair of subsequent pictures constitutes a stereo pair (left and right view). The rendering of the time interleaved stereoscopic video is typically performed at the high frame rate, where active (shutter) glasses are used to blend the incorrect view at each eye. This requires accurate synchronization between the glasses and the screen.

Figure 2 Temporal Interleaving

#### Multi-view Video

MVC [1] has recently been standardized for the compression of multiple view video as an addition to the H.264/AVC standard family. In MVC, the views from different cameras are encoded into a single bit-stream that is backwards compatible with single view H.264/AVC. MVC introduces new coding tools to exhibit the spatial redundancy among the different views.

MVC is able to efficiently compress stereoscopic video in a backwards compatible manner and without compromising the view resolutions. The NAL units from the secondary view are ignored by legacy decoders as the NAL unit type will not be recognized. If the server is aware of the UE capabilities, it can omit sending NAL units from the secondary view to a device that does not support 3D or does not have enough bitrate to deliver both views.

The following figure depicts a possible prediction chain for a stereoscopic video.

Figure 3 MVC Encoding with Inter-view Prediction

References

[1] ISO/IEC 14496-10, Annex H, Advanced Video Coding for Audiovisual Services

[2] ISO/IEC 23000-11, “Information technology — Multimedia application format (MPEG-A) — Part 11: Stereoscopic video application format,” Nov. 2009.