3GPP TSG-SA4 Meeting #60 	S4-100692 update of S4-100619,

# Erlangen, Germany, 16th-20th August 2010                S4-100514 and S4-100461

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.234 | CR | 0173 | rev | 3 | Current version: | 9.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on update to device capability profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NOKIA Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | PMA-MBS_Ext |  |  |  |  |  |  | Date: |  | 10/08/2010 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | The capability exchange functionality in PSS does not reflect all the capabilities of PSS, which makes the capability exchange unreliable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This change proposes the addition of capability indication for the support of adaptive HTTP streaming. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No possibility to indicate support for adaptive HTTP streaming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.1, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

##### 5.2.3.2.1	PssCommon component

Attribute name:		AudioChannels

Attribute definition:	This attribute describes the stereophonic capability of the natural audio device.

Component:			PssCommon

Type:					Literal

Legal values:			“Mono”, “Stereo”

Resolution rule:		Locked

EXAMPLE:	<AudioChannels>Mono</AudioChannels>

Attribute name:		MaxPolyphony

Attribute definition:	The MaxPolyphony attribute refers to the maximal polyphony that the synthetic audio device supports as defined in [44].

NOTE:	The MaxPolyphony attribute is used to signal the maximum polyphony capabilities supported by the PSS client. This is a complementary mechanism for the delivery of compatible SP-MIDI content and thus by setting the MaxPolyphony attribute the PSS client is required to support Scalable Polyphony MIDI i.e. Channel Masking defined in [44].

Component:			PssCommon

Type:					Number

Legal values:			Integer between 5 and 24

Resolution rule:		Locked

EXAMPLE:		<MaxPolyphony>8</MaxPolyphony>

Attribute name:		NumOfGM1Voices

Attribute definition:	The NumOfGM1Voices attribute refers to the maximum number of simultaneous GM1 voices that the synthetic audio engine supports.

Component:			PssCommon

Type:					Number

Legal values:			Integer greater or equal than 5

Resolution rule:		Locked

EXAMPLE:		<NumOfGMIVoices>24</NumOfGMIVoices>

Attribute name:		NumOfMobileDLSVoicesWithoutOptionalBlocks

Attribute definition:	The NumOfMobileDLSVoicesWithoutOptionalBlocks attribute refers to the maximum number of simultaneous Mobile DLS [70] voices without optional group of processing blocks that the synthetic audio engine supports.

Component:			PssCommon

Type:					Number

Legal values:			Integer greater or equal than 5

Resolution rule:		Locked

EXAMPLE:		<NumOfMobileDLSVoicesWithoutOptionalBlocks>24
</NumOfMobileDLSVoicesWithoutOptionalBlocks>

Attribute name:		NumOfMobileDLSVoicesWithOptionalBlocks

Attribute definition:	The NumOfMobileDLSVoicesWithOptionalBlocks attribute refers to the maximum number of simultaneous Mobile DLS voices with optional group of processing blocks that the synthetic audio engine supports. This attribute is set to zero for devices that do not support the optional group of processing blocks.

Component:			PssCommon

Type:					Number

Legal values:			Integer greater than or equal to 0

Resolution rule:		Locked

EXAMPLE:		<NumOfMobileDLSVoicesWithOptionalBlocks>24
</NumOfMobileDLSVoicesWithOptionalBlocks>

Attribute name:		PssVersion

Attribute definition:	Latest PSS version supported by the client.

Component:			PssCommon

Type:					Literal

Legal values:			"3GPP-R4", "3GPP-R5", "3GPP-R6", "3GPP-R7", "3GPP-R8", "3GPP-R9" and so forth.

Resolution rule:		Locked

EXAMPLE:	<PssVersion>3GPP-R6</PssVersion>

Attribute name:		RenderingScreenSize

Attribute definition:	The rendering size of the device’s screen in unit of pixels available for PSS media presentation. The horizontal size is given followed by the vertical size.

Component:			PssCommon

Type:					Dimension

Legal values:	Two integer values equal or greater than zero. A value equal “0x0”means that there exists no possibility to render visual PSS presentations.

Resolution rule:		Locked

EXAMPLE:	<RenderingScreenSize>70x15</RenderingScreenSize>

##### 5.2.3.2.2	Streaming component

Attribute name:		StreamingMethod

Attribute definition:	List of streaming methods supported by the PSS application. The client may support RTP streaming, HTTP streaming, Progressive Download, or all.

Component:			Streaming

Type:					Literal (Bag)

Legal values:			"RTP", "HTTP", “Progressive”

Resolution rule:		Append

EXAMPLE:		<StreamingMethod>
		<rdf:Bag>
			<rdf:li>RTP</rdf:li>
			<rdf:li>HTTP</rdf:li>
		</rdf:Bag>
	</StreamingMethod>

Attribute name:		StreamingAccept

Attribute definition:	List of content types (MIME types) relevant for streaming over RTP or HTTP supported by the PSS application. Content types listed shall be possible to stream over RTP. For each content type a set of MIME parameters can be specified to signal receiver capabilities. A content type that supports multiple parameter sets may occur several times in the list.

Component:			Streaming

Type:					Literal (Bag)

Legal values:			List of MIME types with related parameters.

Resolution rule:		Append

EXAMPLE 1:	<StreamingAccept>
  <rdf:Bag>
    <rdf:li>audio/AMR-WB; octet-alignment=1</rdf:li>
    <rdf:li>video/H263-2000; profile=0; level=45</rdf:li>
  </rdf:Bag>
</StreamingAccept>

EXAMPLE 2:	<StreamingAccept>
  <rdf:Bag>
    <rdf:li>audio/AMR-WB+</rdf:li>
    <rdf:li>video/H264; profile-level-id=42e00a</rdf:li>
    <rdf:li>video/richmedia+xml; Version-profile=10</rdf:li>
  </rdf:Bag>
</StreamingAccept>

Attribute name:		StreamingAccept-Subset

Attribute definition:	List of content types for which the PSS application supports a subset. MIME types can in most cases effectively be used to express variations in support for different media types. Many MIME types, e.g. AMR-WB have several parameters that can be used for this purpose. There may exist content types for which the PSS application only supports a subset and this subset cannot be expressed with MIME-type parameters. In these cases the attribute StreamingAccept-Subset is used to describe support for a subset of a specific content type. If a subset of a specific content type is declared in StreamingAccept-Subset, this means that StreamingAccept-Subset has precedence over StreamingAccept. StreamingAccept shall always include the corresponding content types for which StreamingAccept-Subset specifies subsets of.

Subset identifiers and corresponding semantics shall only be defined by the TSG responsible for the present document.

Component:			Streaming

Type:					Literal (Bag)

Legal values:			No subsets defined.

Resolution rule:		Append

Attribute name:		ThreeGPPLinkChar

Attribute definition:	Indicates whether the device supports the 3GPP-Link-Char header according to clause 10.2.1.1.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Override

EXAMPLE:	<ThreeGPPLinkChar>Yes</ThreeGPPLinkChar>

Attribute name:		AdaptationSupport

Attribute definition:	Indicates whether the device supports client buffer feedback signaling according to clause 10.2.3.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Locked

EXAMPLE:	<AdaptationSupport>Yes</AdaptationSupport>

Attribute name:         QoESupport

Attribute definition:   Indicates whether the device supports QoE signaling according to clauses 5.3.2.3, 5.3.3.6, and 11.

Component:			Streaming

Type:			Literal

Legal values: 			"Yes", "No"

Resolution rule:			Locked

EXAMPLE:      	<QoESupport>Yes</QoESupport>

Attribute name:		ExtendedRtcpReports

Attribute definition:	Indicates whether the device supports extended RTCP reports according to clause 6.2.3.1.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Locked

EXAMPLE:	<ExtendedRtcpReports>Yes</ExtendedRtcpReports>

Attribute name:		RtpRetransmission

Attribute definition:	Indicates whether the device supports RTP retransmission according to clause 6.2.3.3.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Locked

EXAMPLE:	<RtpRetransmission>Yes</RtpRetransmission>

Attribute name:		MediaAlternatives

Attribute definition:	Indicates whether the device interprets the SDP attributes "alt", "alt-default-id", and "alt-group", defined in clauses 5.3.3.3 and 5.3.3.4.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Override

EXAMPLE:	<MediaAlternatives>Yes</MediaAlternatives>

Attribute name:		RtpProfiles

Attribute definition:	List of supported RTP profiles.

Component:			Streaming

Type:					Literal (Bag)

Legal values:	Profile names registered through the Internet Assigned Numbers Authority (IANA), www.iana.org.

Resolution rule:		Append

EXAMPLE:	<RtpProfiles>
  <rdf:Bag>
    <rdf:li>RTP/AVP</rdf:li>
    <rdf:li>RTP/AVPF</rdf:li>
  </rdf:Bag>
</RtpProfiles>

Attribute name:		ThreeGPPPipelined

Attribute definition:	Indicates whether the device supports fast content start-up with pipelining according to clause 5.5.3.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Override

EXAMPLE:	<ThreeGPPPipelined>Yes</ThreeGPPPipelined>

Attribute name:		ThreeGPPSwitch

Attribute definition:	Indicates whether the device supports fast content switching with known SDP according to clause 5.5.4.3.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Override

EXAMPLE:	<ThreeGPPSwitch>Yes</ThreeGPPSwitch>

Attribute name:		ThreeGPPSwitchReqSDP

Attribute definition:	Indicates whether the device supports fast content switching without SDP according to clause 5.5.4.4.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Override

EXAMPLE:	<ThreeGPPSwitchReqSDP>Yes</ThreeGPPSwitchReqSDP>

Attribute name:		ThreeGPPSwitchStream

Attribute definition:	Indicates whether the device supports the fast switching of media streams  according to clause 5.5.4.5.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Override

EXAMPLE:	<ThreeGPPSwitchStream>Yes</ThreeGPPSwitchStream>

Attribute name:		AcceptRanges

Attribute definition:	List of range indications that are accepted by the client. The client may support UTC or NPT or both.

Component:			Streaming

Type:					Literal (Bag)

Legal values:			"NPT", "UTC"

Resolution rule:		Append

EXAMPLE:		<AcceptRanges>
		<rdf:Bag>
			<rdf:li>NPT</rdf:li>
			<rdf:li>UTC</rdf:li>
		</rdf:Bag>
	</AcceptRanges>

Attribute name:		StreamingOmaDrm

Attribute definition:	Indicates whether the device supports streamed OMA DRM protected content as defined by OMA and Annex K.

Component:			Streaming

Type:					Literal (Bag)

Legal values:			OMA Version numbers supported as a floating number. 0.0 indicates no support.

Resolution rule:		Locked

EXAMPLE:	<StreamingOmaDrm>
  <rdf:Bag>
    <rdf:li>2.0</rdf:li>
  </rdf:Bag>
</StreamingOmaDrm>

Attribute name:		ISMACryp

Attribute definition:	Indicates whether the device supports streamed content in ISMACryp format as defined in ISMACryp and Annex R.

Component:			Streaming

Type:					Literal (Bag)

Legal values:			ISMACryp Version numbers supported as a floating number. 0.0 indicates no support.

Resolution rule:		Locked

EXAMPLE:	<StreamingOmaDrm>
  <rdf:Bag>
    <rdf:li>2.0</rdf:li>
  </rdf:Bag>
</StreamingOmaDrm>

Attribute name:		PSSIntegrity

Attribute definition:	Indicates whether the device supports integrity protection for streamed content as defined by Annex K.2.

Component:			Streaming

Type:					Literal

Legal values:			"Yes", "No"

Resolution rule:		Locked

EXAMPLE:	<PSSIntegrity>Yes</PSSIntegrity>

Attribute name:		VideoDecodingByteRate

Attribute definition:	If Annex G is not supported, the attribute has no meaning. If Annex G is supported, this attribute defines the peak decoding byte rate the PSS client is able to support. In other words, the PSS client fulfils the requirements given in Annex G with the signalled peak decoding byte rate. The values are given in bytes per second and shall be greater than or equal to 16000. According to Annex G, 16000 is the default peak decoding byte rate for the mandatory video codec profile and level (H.263 Profile 0 Level 45).

Component:			Streaming

Type:					Number

Legal values:			Integer value greater than or equal to 16000.

Resolution rule:		Locked

EXAMPLE:	<VideoDecodingByteRate>16000</VideoDecodingByteRate>

Attribute name:		VideoInitialPostDecoderBufferingPeriod

Attribute definition:	If Annex G is not supported, the attribute has no meaning. If Annex G is supported, this attribute defines the maximum initial post-decoder buffering period of video. Values are interpreted as clock ticks of a 90-kHz clock. In other words, the value is incremented by one for each 1/90 000 seconds. For example, the value 9000 corresponds to 1/10 of a second initial post-decoder buffering.

Component:			Streaming

Type:					Number

Legal values:			Integer value equal to or greater than zero.

Resolution rule:		Locked

EXAMPLE:		<VideoInitialPostDecoderBufferingPeriod>9000
</VideoInitialPostDecoderBufferingPeriod>

Attribute name:		VideoPreDecoderBufferSize

Attribute definition:	This attribute signals if the optional video buffering requirements defined in Annex G are supported. It also defines the size of the hypothetical pre-decoder buffer defined in Annex G. A value equal to zero means that Annex G is not supported. A value equal to one means that Annex G is supported. In this case the size of the buffer is the default size defined in Annex G.  A value equal to or greater than the default buffer size defined in Annex G means that Annex G is supported and sets the buffer size to the given number of octets.

Component:			Streaming

Type:					Number

Legal values:	Integer value equal to or greater than zero. Values greater than one but less than the default buffer size defined in Annex G are not allowed.

Resolution rule:		Locked

EXAMPLE:	<VideoPreDecoderBufferSize>30720</VideoPreDecoderBufferSize>

##### 5.2.3.2.3	ThreeGPFileFormat component

Attribute name:		Brands

Attribute definition:	List of supported 3GP profiles identified by brand.

Component:			ThreeGPFileFormat

Type:					Literal (Bag)

Legal values:			Brand identifiers according to 5.3.4 and 5.4 in [50].

Resolution rule:		Append

EXAMPLE: 	<Brands>
  <rdf:Bag>
    <rdf:li>3gp4</rdf:li>
    <rdf:li>3gp5</rdf:li>
    <rdf:li>3gp6</rdf:li>
    <rdf:li>3gr6</rdf:li>
    <rfd:li>3gp7</rdf:li>
    <rfd:li>3gr7</rdf:li>
    <rfd:li>3ge7</rdf:li>
  </rdf:Bag>
</Brands>

Attribute name:		ThreeGPAccept

Attribute definition:	List of content types (MIME types) that can be included in a 3GP file and handled by the PSS application. The content types included in this attribute can be rendered in a 3GP file or a presentation contained therein.  If the identifier "Streaming-Media" is included, streaming media can be included within a contained presentation (e.g. in DIMS). Details on the streaming support can then be found in the Streaming component. For each content type a set of supported parameters can be given. A content type that supports multiple parameter sets may occur several times in the list.

Component:			ThreeGPFileFormat

Type:					Literal (Bag)

Legal values:			List of MIME types with related parameters and the "Streaming-Media" identifier.

Resolution rule:		Append

EXAMPLE1: 		<ThreeGPAccept>
  <rdf:Bag>
    <rdf:li>video/H263-2000; profile=0; level=45</rdf:li>
    <rdf:li>audio/AMR</rdf:li>
  </rdf:Bag>
</ThreeGPAccept>

EXAMPLE 2: 		<ThreeGPAccept>
  <rdf:Bag>
    <rdf:li>audio/AMR</rdf:li>
    <rdf:li>audio/AMR-WB+</rdf:li>
    <rdf:li>video/H263-2000; profile=0; level=45</rdf:li>
    <rdf:li>video/H264; profile-level-id=42e00a</rdf:li>
    <rdf:li>image/jpeg</rdf:li>
    <rdf:li>video/richmedia+xml; Version-profile=10</rdf:li>
    <rdf:li>Streaming-Media</rdf:li>
  </rdf:Bag>
</ThreeGPAccept>

Attribute name:		ThreeGPAccept-Subset

Attribute definition:	List of content types for which the PSS application supports a subset. MIME types can in most cases effectively be used to express variations in support for different media types. Many MIME types have several parameters that can be used for this purpose. There may exist content types for which the PSS application only supports a subset and this subset cannot be expressed with MIME-type parameters. In these cases the attribute ThreeGPAccept-Subset is used to describe support for a subset of a specific content type. If a subset of a specific content type is declared in ThreeGPAccept-Subset, this means that ThreeGPAccept-Subset has precedence over ThreeGPAccept. ThreeGPAccept shall always include the corresponding content types for which ThreeGPAccept-Subset specifies subsets of.

Subset identifiers and corresponding semantics shall only be defined by the TSG responsible for the present document.

Component:			ThreeGPFileFormat

Type:					Literal (Bag)

Legal values:			No subsets defined.

Resolution rule:		Append

Attribute name:		ThreeGPOmaDrm

Attribute definition:	List of the OMA DRM versions that is supported to be used for DRM protection of content present in the 3GP file format.

Component:			ThreeGPFileFormat

Type:					Literal (Bag)

Legal values:			OMA DRM version numbers as floating point values. 0.0 indicates no support.

Resolution rule:		Locked

EXAMPLE: 	<3gpOMADRM>
  <rdf:Bag>
    <rdf:li>2.0 </rdf:li>
  </rdf:Bag>
</3gpOMADRM>

##### 5.2.3.2.4	PssSmil component

Attribute name:		SmilAccept

Attribute definition:	List of content types (MIME types) that can be part of a SMIL presentation. The content types included in this attribute can be rendered in a SMIL presentation. If video/3gpp (or audio/3gpp) is included, downloaded 3GP files can be included in a SMIL presentation. Details on the 3GP file support can then be found in the ThreeGPFileFormat component. If the identifier "Streaming-Media" is included, streaming media can be included in the SMIL presentation. Details on the streaming support can then be found in the Streaming component. For each content type a set of supported parameters can be given. A content type that supports multiple parameter sets may occur several times in the list.

Component:			PssSmil

Type:					Literal (Bag)

Legal values:			List of MIME types with related parameters and the "Streaming-Media" identifier.

Resolution rule:		Append

EXAMPLE 1: 	<SmilAccept>
  <rdf:Bag>
    <rdf:li>image/gif</rdf:li>
    <rdf:li>image/jpeg</rdf:li>
    <rdf:li>Streaming-Media</rdf:li>
  </rdf:Bag>
</SmilAccept>

Attribute name:		SmilAccept-Subset

Attribute definition:	List of content types for which the PSS application supports a subset. MIME types can in most cases effectively be used to express variations in support for different media types. Many MIME types have several parameters that can be used for this purpose. There may exist content types for which the PSS application only supports a subset and this subset cannot be expressed with MIME-type parameters. In these cases the attribute SmilAccept-Subset is used to describe support for a subset of a specific content type. If a subset of a specific content type is declared in SmilAccept-Subset, this means that SmilAccept-Subset has precedence over SmilAccept. SmilAccept shall always include the corresponding content types for which SmilAccept-Subset specifies subsets of.

The following values are defined:

-	"JPEG-PSS": Only the two JPEG modes described in clause 7.5 of the present document are supported.

-	"SVG-Tiny"

-	"SVG-Basic"

Subset identifiers and corresponding semantics shall only be defined by the TSG responsible for the present document.

Component:			PssSmil

Type:					Literal (Bag)

Legal values:			"JPEG-PSS", "SVG-Tiny", "SVG-Basic"

Resolution rule:		Append

EXAMPLE 2:	<SmilAccept-Subset>
  <rdf:Bag>
    <rdf:li>JPEG-PSS</rdf:li>
    <rdf:li>SVG-Tiny</rdf:li>
  </rdf:Bag>
</SmilAccept-Subset>

Attribute name:		SmilBaseSet

Attribute definition:	Indicates a base set of SMIL 2.0 modules that the client supports.

Component:			Streaming

Type:					Literal

Legal values:	Pre-defined identifiers. "SMIL-3GPP-R4" and "SMIL-3GPP-R5" indicate all SMIL 2.0 modules required for SMIL scene description support according to clause 8 of Release 4 and Release 5, respectively, of TS 26.234. "SMIL-3GPP-R6"and "SMIL-3GPP-R7" indicate all SMIL 2.0 modules required for SMIL scene-description support according to Release 6 and Release 7, respectively, of clause 8 of the present document (TS 26.234) and of TS 26.246 [52].

Resolution rule:		Locked

EXAMPLE 3:	<SmilBaseSet>SMIL-3GPP-R6</SmilBaseSet>

Attribute name:		SmilModules

Attribute definition:	This attribute defines a list of SMIL 2.0 modules supported by the client. If the SmilBaseSet is used those modules do not need to be explicitly listed here. In that case only additional module support needs to be listed.

Component:			Streaming

Type:					Literal (Bag)

Legal values:	SMIL 2.0 module names defined in the SMIL 2.0 recommendation [31], section 2.3.3, table 2.

Resolution rule:		Append

EXAMPLE 4:	<SmilModules>
  <rdf:Bag>
    <rdf:li>BasicTransitions</rdf:li>
    <rdf:li>MulitArcTiming</rdf:li>
  </rdf:Bag>
</SmilModules>

Second Change

Annex F (normative):
RDF schema for the PSS base vocabulary

<?xml version="1.0"?>

<!--

This document is the RDF Schema for Packet-switched Streaming

Service (PSS)-specific vocabulary as defined in 3GPP TS 26.234

Release 7 (in the following "the specification").

The URI for unique identification of this RDF Schema is

http://www.3gpp.org/profiles/PSS/ccppschema-PSS7#

This RDF Schema includes the same information as the respective

chapter of the specification. Greatest care has been taken to keep

the two documents consistence. However, in case of any divergence

the specification takes presidence.

All reference in this RDF Schmea are to be interpreted relative to

the specification. This means all references using the form

[ref] are defined in chapter 2 "References" of the specification.

All other references refer to parts within that document.

Note: This Schemas has been aligned in structure and base

vocabulary to the RDF Schema used by UAProf [40].

-->

<rdf:RDF xmlns:rdf="http://www.w3.org/1999/02/22-rdf-syntax-ns#"

xmlns:rdfs="http://www.w3.org/2000/01/rdf-schema#" >

<!-- ****************************************************************** -->

<!-- ***** Properties shared among the components***** -->

<rdf:Description rdf:ID="defaults">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:domain rdf:resource="#ThreeGPFileFormat"/>

<rdfs:domain rdf:resource="#PssSmil"/>

<rdfs:comment>

An attribute used to identify the default capabilities.

</rdfs:comment>

</rdf:Description>

<!-- ****************************************************************** -->

<!-- ***** Component Definitions ***** -->

<rdf:Description rdf:ID="PssCommon">

<rdf:type rdf:resource="http://www.w3.org/2000/01/rdf-schema#Class"/>

<rdfs:subClassOf rdf:resource="http://www.wapforum.org/profiles/UAPROF/ccppschema-20010330#Component"/>

<rdfs:label>Component: PssCommon</rdfs:label>

<rdfs:comment>

The PssCommon component specifies the base vocabulary common for all

PSS applications, in contrast to application-specific parts of the PSS

base vocabulary which are described by the Streaming, ThreeGPFileFormat and

PssSmil components defined below.

PSS servers supporting capability exchange should understand the attributes

in this component as explained in detail in 3GPP TS 26.234 Release 7..

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="Streaming">

<rdf:type rdf:resource="http://www.w3.org/2000/01/rdf-schema#Class"/>

<rdfs:subClassOf rdf:resource="http://www.wapforum.org/profiles/UAPROF/ccppschema-20010330#Component"/>

<rdfs:label>Component: Streaming</rdfs:label>

<rdfs:comment>

The Streaming component specifies the base vocabulary for pure RTSP/RTP-

based streaming in PSS.

PSS servers supporting capability exchange should understand the attributes

in this component as explained in detail in 3GPP TS 26.234 Release 7.

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="ThreeGPFileFormat">

<rdf:type rdf:resource="http://www.w3.org/2000/01/rdf-schema#Class"/>

<rdfs:subClassOf rdf:resource="http://www.wapforum.org/profiles/UAPROF/ccppschema-20010330#Component"/>

<rdfs:label>Component: ThreeGPFileFormat</rdfs:label>

<rdfs:comment>

The ThreeGPFileFormat component specifies the base vocabulary for 3GP file

download or progressive download in PSS.

PSS servers supporting capability exchange should understand the attributes

in this component as explained in detail in 3GPP TS 26.234 Release 7.

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="PssSmil">

<rdf:type rdf:resource="http://www.w3.org/2000/01/rdf-schema#Class"/>

<rdfs:subClassOf rdf:resource="http://www.wapforum.org/profiles/UAPROF/ccppschema-20010330#Component"/>

<rdfs:label>Component: PssSmil</rdfs:label>

<rdfs:comment>

The PssSmil component specifies the base vocabulary for SMIL presentations

in PSS. Note that capabibilites regarding streaming and 3GP files that are

part of a SMIL presentation are expressed by the vocabularies specified by

the Streaming and ThreeGPFileFormat components, respectively.

PSS servers supporting capability exchange should understand the attributes

in this component as explained in detail in 3GPP TS 26.234 Release 7.

</rdfs:comment>

</rdf:Description>

<!-- **

** In the following property definitions, the defined types

** are as follows:

**

** Number: A positive integer

** [0-9]+

** Boolean: A yes or no value

** Yes|No

** Literal: An alphanumeric string

** [A-Za-z0-9/.\-_]+

** Dimension: A pair of numbers

** [0-9]+x[0-9]+

**

-->

<!-- ****************************************************************** -->

<!-- ***** Component: PssCommon ***** -->

<rdf:Description rdf:ID="AudioChannels">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: This attribute describes the stereophonic capability of the

natural audio device. The only legal values are "Mono" and "Stereo".

Type: Literal

Resolution: Locked

Examples: "Mono", "Stereo"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="MaxPolyphony">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: The MaxPolyphony attribute refers to the maximal polyphony

that the synthetic audio device supports as defined in [44]. Legal values

are integer between 5 to 24.

NOTE: MaxPolyphony attribute can be used to signal the maximum polyphony

capabilities supported by the PSS client. This is a complementary

mechanism for the delivery of compatible SP-MIDI content and thus

the PSS client is required to support Scalable Polyphony MIDI i.e.

Channel Masking defined in [44].

Type: Number

Resolution: Locked

Examples: 8

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="NumOfGM1Voices">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: The NumOfGM1Voices attribute refers to the maximum number

of simultaneous GM1   voices that the synthetic audio engine supports.

Legal values are integers greater or equal than 5.

Type: Number

Resolution: Locked

Examples: 24

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="NumOfMobileDLSVoicesWithoutOptionalBlocks">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: The NumOfMobileDLSVoicesWithoutOptionalBlocks attribute

refers to the maximum number of simultaneous voices without optional

group of processing blocks that the synthetic audio engine supports.

Legal values are integers greater or equal than 5.

Type: Number

Resolution: Locked

Examples: 24

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="NumOfMobileDLSVoicesWithOptionalBlocks">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: The NumOfMobileDLSVoicesWithOptionalBlocks attribute refers

to the maximum number of simultaneous voices with optional group of

processing blocks that the synthetic audio engine supports. This attribute

is set to zero for devices that do not support the optional group of

processing blocks. Legal values are integers greater or equal than 0.

Type: Number

Resolution: Locked

Examples: 24

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="PssVersion">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: Latest PSS version supported by the client. Legal

values are "3GPP-R4", "3GPP-R5", "3GPP-R6", "3GPP-R7" and so forth.

Type: Literal

Resolution: Locked

Examples: "3GPP-R5", "3GPP-R6"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="RenderingScreenSize">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssCommon"/>

<rdfs:comment>

Description: The rendering size of the device's screen in unit of

pixels available for PSS media presentation. The horizontal size is

given followed by the vertical size. Legal values are pairs of integer

values equal or greater than zero. A value equal "0x0"means that there

exists no display or just textual output is supported.

Type: Dimension

Resolution: Locked

Examples: "160x120"

</rdfs:comment>

</rdf:Description>

<!-- ****************************************************************** -->

<!-- ***** Component: Streaming ***** -->

<rdf:Description rdf:ID="StreamingMethod">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: List of streaming methods supported by the PSS application. The client may 							support RTP streaming, HTTP streaming, or both.

Type: Literal (bag)

Resolution: Append

Examples: "RTP,HTTP"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="StreamingAccept">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: List of content types (MIME types) relevant for streaming

over RTP supported by the PSS application. Content types listed shall be

possible to stream over RTP. For each content type a set of MIME parameters
      can be specified to signal receiver capabilities. A content type that

supports multiple parameter sets may occur several times in the list.

Legal values are lists of MIME types with related parameters.

Type: Literal (bag)

Resolution: Append

Examples: "audio/AMR-WB;octet-alignment=1,application/smil"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="StreamingAccept-Subset">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: List of content types for which the PSS application supports

a subset. MIME types can in most cases effectively be used to express

variations in support for different media types. Many MIME types, e.g.

AMR-WB has several parameters that can be used for this purpose. There

may exist content types for which the PSS application only supports a

subset and this subset cannot be expressed with MIME-type parameters.

In these cases the attribute StreamingAccept-Subset is used to describe

support for a subset of a specific content type. If a subset of a specific

content type is declared in StreamingAccept-Subset, this means that

StreamingAccept-Subset has precedence over StreamingAccept.

StreamingAccept shall always include the corresponding content types for

which StreamingAccept-Subset specifies subsets of.

No legal values are currently defined.

Type: Literal (bag)

Resolution: Locked

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="LinkChar">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports the

3GPP-Link-Char header according to clause 10.2.1.1 of the specification.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Override

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="AdaptationSupport">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports

client buffer feedback signaling according to clause 10.2.3 of the

specification. Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="QoESupport">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports

QoE signaling according to clauses 5.3.2.3, 5.3.3.6, and 11 of the

specification. Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="ExtendedRtcpReports">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports

extended RTCP reports according to clause 6.2.3.1 of the specification.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="RtpRetransmission">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports RTP

retransmission according to clause 6.2.3.3 of the specification.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="MediaAlternatives">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device interprets the

SDP attributes "alt", "alt-default-id", and "alt-group", defined in

clauses 5.3.3.3 and 5.3.3.4 of the specification.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Override

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="RtpProfiles">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute lists the supported RTP profiles. Legal

values are profile names registered through the Internet Assigned Numbers

Authority (IANA), www.iana.org.

Type: Literal (bag)

Resolution: Append

Examples: "RTP/AVP,RTP/AVPF"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="3GPPPipelined">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports fast content start-up with pipelining according to clause 5.5.3.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="3GPPSwitch">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports fast content switching with known SDP according to clause 5.5.4.3..

Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="3GPPSwitchReqSDP">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports fast content switching without SDP according to clause 5.5.4.4.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="3GPPSwitchStream">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute indicates whether the device supports the fast switching of media streams  according to clause 5.5.4.5.

Legal values are "Yes" and "No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="AcceptRanges">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: List of range indications that are accepted by the client. The client may support UTC or NPT or both.

Type: Literal (bag)

Resolution: Append

Examples: "NPT,UTC"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="StreamingOmaDrm">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: Indicates whether the device supports streamed OMA DRM

protected content, as defined by OMA and Annex K. Legal values are OMA

Version numbers supported as a floating number. 0.0 indicates no support.

Type: Literal (bag)

Resolution: Locked

Examples: "2.0"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="ISMACryp">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: Indicates whether the device supports streamed protected

content in ISMACryp format, as defined by ISMACryp and Annex R. Legal values are ISMACryp

Version numbers supported as a floating number. 0.0 indicates no support.

Type: Literal (bag)

Resolution: Locked

Examples: "2.0"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="PSSIntegrity">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: Indicates whether the device supports integrity protection

for streamed content as defined by Annex K.2. Legal values are "Yes" and

"No".

Type: Literal

Resolution: Locked

Examples: "Yes"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="VideoDecodingByteRate">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: If Annex G is not supported, the attribute has no meaning.

If Annex G is supported, this attribute defines the peak decoding byte

rate the PSS client is able to support. In other words, the PSS client

fulfils the requirements given in Annex G with the signalled peak decoding

byte rate. The values are given in bytes per second and shall be greater

than or equal to 16000. According to Annex G, 16000 is the default peak

decoding byte rate for the mandatory video codec profile and level

(H.263 Profile 0 Level 45). Legal values are integer values greater than

or equal to 16000.

Type: Number

Resolution: Locked

Examples: "16000"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="VideoInitialPostDecoderBufferingPeriod">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: If Annex G is not supported, the attribute has no

meaning. If Annex G is supported, this attribute defines the

maximum initial post-decoder buffering period of video. Values are

interpreted as clock ticks of a 90-kHz clock. In other words, the

value is incremented by one for each 1/90 000 seconds. For

example, the value 9000 corresponds to 1/10 of a second initial

post-decodder buffering. Legal values are all integer values equal

to or greater than zero.

Type: Number

Resolution: Locked

Examples: "9000"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="VideoPreDecoderBufferSize">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#Streaming"/>

<rdfs:comment>

Description: This attribute signals if the optional video

buffering requirements defined in Annex G are supported. It also

defines the size of the hypothetical pre-decoder buffer defined in

Annex G. A value equal to zero means that Annex G is not

supported. A value equal to one means that Annex G is

supported. In this case the size of the buffer is the default size

defined in Annex G.  A value equal to or greater than the default

buffer size defined in Annex G means that Annex G is supported and

sets the buffer size to the given number of octets. Legal values are all

integer values equal to or greater than zero. Values greater than

one but less than the default buffer size defined in Annex G are

not allowed.

Type: Number

Resolution: Locked

Examples: "0", "4096"

</rdfs:comment>

</rdf:Description>

<!-- ****************************************************************** -->

<!-- ***** Component: ThreeGPFileFormat ***** -->

<rdf:Description rdf:ID="Brands">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#ThreeGPFileFormat"/>

<rdfs:comment>

Description: This attribute lists the supported 3GP profiles identified

by brand. Legal values are brand identifiers according to 5.3.4 and 5.4

in [50].

Type: Literal (bag)

Resolution: Append

Examples: "3gp4,3gp5,3gp6,3gr6,3gp7,3gr7,3ge7"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="ThreeGPAccept">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#ThreeGPFileFormat"/>

<rdfs:comment>

Description: List of content types (MIME types) that can be included

in a 3GP file and handled by the PSS application. If the identifier

"Streaming-Media" is included, streaming media can be included in the

presentation, e.g. in DIMS. Details on the streaming support can then be

found in the Streaming component. For each content

type a set of supported parameters can be given. A content type that

supports multiple parameter sets may occur several times in the list.

Type: Literal (bag)

Resolution: Append

Examples: "video/H263-2000;profile=0;level=45,audio/AMR"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="ThreeGPAccept-Subset">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#ThreeGPFileFormat"/>

<rdfs:comment>

Description: List of content types for which the PSS application

supports a subset. MIME types can in most cases effectively be used

to express variations in support for different media types. Many MIME

types have several parameters that can be used for this purpose. There

may exist content types for which the PSS application only supports a

subset and this subset cannot be expressed with MIME type parameters.

In these cases the attribute ThreeGPAccept-Subset is used to describe

support for a subset of a specific content type. If a subset of a

specific content type is declared in ThreeGPAccept-Subset, this means that

ThreeGPAccept-Subset has precedence over ThreeGPAccept. ThreeGPAccept shall always

include the corresponding content types for which ThreeGPAccept-Subset

specifies subsets of. No legal values are currently defined.

Type: Literal (bag)

Resolution: Locked

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="ThreeGPOmaDrm">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#ThreeGPFileFormat"/>

<rdfs:comment>

Description: List of the OMA DRM versions that is supported to be used

for DRM protection of content present in the 3GP file format. Legal values

are OMA DRM version numbers as floating values. 0.0 indicates no support.

Type: Literal (bag)

Resolution: Locked

Examples: "2.0"

</rdfs:comment>

</rdf:Description>

<!-- ****************************************************************** -->

<!-- ***** Component: PssSmil ***** -->

<rdf:Description rdf:ID="SmilAccept">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#PssSmil"/>

<rdfs:comment>

Description: List of content types (MIME types) that can be part of a

SMIL presentation. The content types included in this attribute can be

rendered in a SMIL presentation. If video/3gpp (or audio/3gpp) is

included, downloaded 3GP files can be included in a SMIL presentation.

Details on the 3GP file support can then be found in the ThreeGPFileFormat

component. If the identifier "Streaming-Media" is included, streaming

media can be included in the SMIL presentation. Details on the

streaming support can then be found in the Streaming component.

For each content type a set of supported parameters can be given.

A content type that supports multiple parameter sets may occur several

times in the list. Legal values are lists of MIME types with related

parameters and the "Streaming-Media" identifier.

Type: Literal (bag)

Resolution: Append

Examples: "image/gif,image/jpeg,Streaming-Media"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="SmilAccept-Subset">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#PssSmil"/>

<rdfs:comment>

Description: List of content types for which the PSS application

supports a subset. MIME types can in most cases effectively be used to

express variations in support for different media types. Many MIME types

have several parameters that can be used for this purpose. There may

exist content types for which the PSS application only supports a subset

and this subset cannot be expressed with MIME-type parameters. In these

cases the attribute SmilAccept-Subset is used to describe support for a

subset of a specific content type. If a subset of a specific content type

is declared in SmilAccept-Subset, this means that SmilAccept-Subset has

precedence over SmilAccept. SmilAccept shall always include the

corresponding content types for which SmilAccept-Subset specifies subsets

of.

The following values are defined:

- "JPEG-PSS": Only the two JPEG modes described in clause 7.5 of the

specifictaion are supported.

- "SVG-Tiny"

- "SVG-Basic"

Subset identifiers and corresponding semantics shall only be defined by

the TSG responsible for the present document.

Type: Literal (bag)

Resolution: Append

Examples: "JPEG-PSS,SVG-Tiny"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="SmilBaseSet">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:domain rdf:resource="#PssSmil"/>

<rdfs:comment>

Description: Indicates a base set of SMIL 2.0 modules that the client

supports. Leagal values are the following pre-defined identifiers:

"SMIL-3GPP-R4" and "SMIL-3GPP-R5" indicate all SMIL 2.0 modules required

for SMIL scene-description support according to clause 8 of Release 4 and

Release 5, respectively, of TS 26.234. "SMIL-3GPP-R6" and "SMIL-3GPP-R7"

indicate all SMIL 2.0 modules required for SMIL scene description support

according to Release 6 and Release 7, respectively, of clause 8 of the

specification and of TS 26.246 [52].

Type: Literal

Resolution: Locked

Examples: "SMIL-3GPP-R4", "SMIL-3GPP-R5"

</rdfs:comment>

</rdf:Description>

<rdf:Description rdf:ID="SmilModules">

<rdf:type rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Property"/>

<rdfs:range rdf:resource="http://www.w3.org/1999/02/22-rdf-syntax-ns#Bag"/>

<rdfs:domain rdf:resource="#PssSmil"/>

<rdfs:comment>

Description: This attribute defines a list of SMIL 2.0 modules

supported by the client. If the SmilBaseSet is used those modules

do not need to be explicitly listed here. In that case only

additional module support needs to be listed. Legal values are all

SMIL 2.0 module names defined in the SMIL 2.0 recommendation [31],

section 2.3.3, table 2.

Type: Literal (bag)

Resolution: Locked

Examples: "BasicTransitions,MulitArcTiming"

</rdfs:comment>

</rdf:Description>

</rdf:RDF>