Recommendation ITU-T P.10/G.100 Vocabulary for performance and quality of service Amendment 4 New definitions for inclusion in Recommendation ITU-T P.10/G.100

Summary Amendment 4 to Recommendation ITU-T P.10/G.100 provides new definitions for narrowband, wideband, super-wideband and fullband signals. The existing definitions for normal-band, wideband, super-wideband and fullband telephony remain unchanged. Therefore, it was agreed to include these definitions in Recommendation ITU-T P.10/G.100.

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T P.10; 1980-11-21. 2.0; ITU-T P.10; 1984-10-19. 3.0; ITU-T P.10; 1988-11-25. 4.0; ITU-T P.10; 1993-03-12; XII. 5.0; ITU-T P.10; 1998-12-03; 12. 5.1; ITU-T P.10 (1998) Amd. 1; 2003-11-13; 12. 6.0; ITU-T P.10/G.100; 2006-07-14; 12. 6.1; ITU-T P.10/G.100 (2006) Amd. 1; 2007-01-25; 12. 6.2; ITU-T P.10/G.100 (2006) Amd. 2; 2008-07-14; 12. 6.3; ITU-T P.10/G.100 (2006) Amd. 3; 2011-12-14; 12. 6.4; ITU-T P.10/G.100 (2006) Amd. 4; 2015-05-14; 12.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure, e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU 2012

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Recommendation ITU-T P.10/G.100

Vocabulary for performance and quality of service

Amendment 4

Updated definitions for inclusion in Recommendation ITU-T P.10/G.100

# Introduction

The following definitions are to be included in Recommendation ITU-T P.10/G.100.

Narrowband signal

A narrowband signal has no significant signal components outside of the range 20 Hz to 4000 Hz. A narrowband signal can be created by applying the filter from Table 1 to a fullband signal as defined in this Amendment 4, the upper cut-off frequency of 3800 Hz is due to a realistic interpolation low-pass when re-sampling to 8000 Hz sampling frequency.

The narrowband  signal can also be derived by applying the filter in Table 1 to a wideband or super-wideband signal as defined in this Amendment 4.

Note: A real narrowband transmission system may limit the passband further. The low-pass filter definition does not emulate a narrowband transmission system, channel, device or codec.

Table 1 – Minimum bandwidth filter definition to derive a narrowband  signal from a fullband signal

| Frequency (Hz) | Narrowband gain (dB) |
| --- | --- |
| 20 | 0 |
| 3'700 | 0 |
| 3'800 | 0 to –3 |
| 4'000 | –40 (max) |

Wideband  signal

A wideband signal has no significant signal components outside of the range 20 Hz to 8000 Hz. A wideband signal can be generated by applying the filter from Table 2 to a fullband signal, the upper cut-off frequency of 7600 Hz is due to a realistic interpolation low-pass when re-sampling to 16000 Hz sampling frequency.

The wideband  signal can also be derived by applying the filter in Table 1 to a super-wideband signal as defined in this Amendment 4.

Note: A real wideband transmission system may limit the passband further. The low-pass filter definition does not emulate a wideband transmission system, channel, device or codec.

Table 2 – Minimum bandwidth filter definition to derive a wideband  signal from a fullband signal

| Frequency (Hz) | Wideband gain (dB) |
| --- | --- |
| 20 | 0 |
| 7400 | 0 |
| 7'600 | 0 to –3 |
| 8'000 | –40 (max) |

Super-wideband  signal

A super-wideband signal has no significant signal components outside of the range 20 Hz to 14’000 Hz. A super-wideband signal can be generated by applying the filter from Table 3 to a fullband signal.

Note: A real super-wideband transmission system may limit the passband further. The low-pass filter definition does not emulate a super-wideband transmission system, channel, device or codec.

Table 3 – Minimum bandwidth filter definition to derive a super-wideband  signal from a fullband signal

| Frequency (Hz) | Super-wideband gain (dB) |
| --- | --- |
| 20 | 0 |
| 13’500 | 0 |
| 14'000 | 0 to –3 |
| 15'000 | –40 (max) |

Fullband  signal

A fullband signal has no significant signal components outside of the range 10 Hz to 20’000 Hz. A fullband signal can be generated by applying the filter from Table 4 to an unfiltered source  signal recorded without band limitation in a high quality recording environment. The application of sub-sonic- and rumble-filters to the source  before filtering to fullband is allowed.

Note: A real fullband transmission system may limit the passband further. The low-pass filter definition does not emulate a fullband transmission system, channel, device or codec.

Table 4 – Minimum bandwidth filter definition to derive a fullband  signal from a unfiltered source signal

| Frequency (Hz) | Fullband gain (dB) |
| --- | --- |
| 10 | –20 (max) |
| 20 | 0 to –3 |
| 30 | 0 |
| 19500 | 0 |
| 20'000 | 0 to –3 |
| 21'000 | –40 (max) |
| 24'000 | –50 (max) |

-----------

____________________