Source:	Qualcomm Inc.

Title:	EVSoCS Codec Type, Code Point, and Mode Sets

Document for:	Approval

Agenda Item:	6

- Introduction

Contribution S4-150685 proposed certain basic parameters of EVSoCS for approval. The project plan in S4-150247 schedules decision on codec mode sets, codec type, code point for the past SA4#83 (April) meeting which could not be accomplished at that time.

In this contribution, we summarize the items for approval and request SA4 to agree on them, in order to limit the delay in the work item.

- Proposals for Agreement

(1) The modified Table 4.1 of TS 26.103 shows the inclusion of UMTS_EVS codec type.

Modified Table 4.1: Support of Codec Types in Radio Access Technologies

|  | TDMA EFR | UMTS AMR 2 | UMTS AMR | (GSM) HR AMR | (GSM) FR AMR | GSM EFR | GSM HR | GSM FR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CoID | 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| GERAN GMSK | not defined | not possible | not possible | yes, 1..4 modi | yes, 1..4 modi | yes | Yes | Yes |
| GERAN 8PSK | not defined | not possible | not possible | not defined | not defined | not defined | not defined | not defined |
| UTRAN | not defined | yes, 1..8 modi 1..4 modi recomm. | R99, UTRAN-only UEs | not defined | not defined | not defined | not defined | not defined |

|  | Codec Extension | UMTS EVS | OHR AMR-WB | OFR AMR-WB | OHR AMR | UMTS AMR-WB | FR AMR-WB | PDC EFR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CoID | 0x0F | 0x0E | 0X0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| GERAN GMSK | reserved | not defined | not defined | not defined | not defined | not possible | yes3 modi | not defined |
| GERAN 8PSK | reserved | not defined | yes, 3 modi | yes, 3 modi | yes, 1..4 modi | not possible | not defined | not defined |
| UTRAN | reserved | Yes [TBD] modi | not defined | not defined | not defined | yes 3..4 modi | not defined | not defined |

(2) The modified Table 4.2 of TS 26.103 shows the code point for EVS on UTRAN radio interface.

Modified Table 4.2: Defined Code Points

| Hexadecimal Notation | Binary Notation | Codec Name | Remark |
| --- | --- | --- | --- |
| 0x00h | 0x0000.0000 | GSM_FR |  |
| 0x01h | 0x0000.0001 | GSM_HR |  |
| 0x02h | 0x0000.0010 | GSM_EFR |  |
| 0x03h | 0x0000.0011 | (GSM) FR_AMR |  |
| 0x04h | 0x0000.0100 | (GSM) HR_AMR |  |
| 0x05h | 0x0000.0101 | UMTS_AMR |  |
| 0x06h | 0x0000.0110 | UMTS_AMR2 |  |
| 0x07h | 0x0000.0111 | TDMA_EFR |  |
| 0x08h | 0x0000.1000 | PDC_EFR |  |
| 0x09h | 0x0000.1001 | (GSM) FR_AMR-WB |  |
| 0x0Ah | 0x0000.1010 | UMTS_AMR-WB |  |
| 0x0Bh | 0x0000.1011 | OHR_AMR |  |
| 0x0Ch | 0x0000.1100 | OFR_AMR-WB |  |
| 0x0Dh | 0x0000.1101 | OHR_AMR-WB |  |
| 0x0Eh | 0x0000.1110 | UMTS_EVS |  |
| 0x0Fh | 0x0000.1111 | Codec Extension | For AoIP and TFO |
| 0x10h … 0xFCh | 0x0001.0012 … 0x1111.1100 | Spare, for future use |  |
| 0xFDh | 0x1111.1101 | CSData | For AoIP only |
| 0xFEh | 0x1111.1110 | MuMe2 | For OoBTC only |
| 0xFFh | 0x1111.1111 | MuMe | For OoBTC only |

(3) The mode sets are summarized in the following Table X to be included in TS 26.103, together with the notes below.

Table X: Allowed Configurations for the UMTS_EVS Codec Type

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 |
| --- | --- | --- | --- |
| EVS Primary 24.4 |  |  | 1 |
| EVS Primary 16.4 |  |  | 1 |
| EVS Primary 13.2 | 1 |  | 1 |
| EVS Primary 9.6 | 1 |  | 1 |
| EVS Primary 8 | 1 | 1 | 1 |
| EVS Primary 7.2 | 1 | 1 | 1 |
| EVS Primary 5.9VBR | 1 | 1 | 1 |
| AMR-WB IO 12.65 | 1 |  | 1 |
| AMR-WB IO 8.85 | 1 |  | 1 |
| AMR-WB IO 6.60 | 1 | 1 | 1 |

Notes:

- Configuration 0 is mandatory.

- Requirement for UE: use the maximum possible audio bandwidth at a given bit rate.

- Typical SF for configurations 0, 1, 2 on DL are respectively 128, 256, 64.

- From a transmission point of view, 5.9 VBR mode (5.9 kb/s average gross bit rate) includes bit rates of 2.8, 7.2, 8.0 kb/s.

There was a support for the mode sets in Table X above at the EVS SWG session so we propose them for agreement. Further mode sets may be included later on need basis, upon showing that they work technically correctly.

(4) Send a liaison to 3GPP system groups on the decisions made by SA4.