TSG-SA4#87 meeting	Tdoc S4 (16)0013

25 – 29 January, 2016, Sophia Antipolis, France		Agenda Item: 5.3

3GPP TSG CT4 Meeting #71	C4-152306

Anaheim, USA; 16th – 20th November 2015

Title:	LS on RTCP control flow of the RTP "FECC mode" stream

Response to:	-

Release:	Rel-13

Work Item:	ROI-CT

Source:	3GPP TSG CT WG4

To:	3GPP TSG SA WG4

Cc:	-

Contact Person:

#### Name:	Albrecht Schwarz

E-mail Address:	Albrecht.Schwarz@alcatel-lucent.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

CT4 specifies the handling of RTP and RTCP information, related to Region-of-Interest information signalling, by the IMS-AGW (as part of the H.248 Iq profile 23.334 / 29.334). The required IMS-AGW behaviour for the "Predefined ROI" and "Arbitrary ROI" is obvious because the IMS-AGW is required to transparently forward the RTCP packet/feedback type which carries feedback-based ROI information.

However, the RTCP handling related to the "Far End Camera Control (FECC)" mode is still unclear.

Situation:

The FECC mode is based on a second RTP stream for H.281-over-H.224 information (besides the main RTP stream for video). Each RTP stream has its own SSRC value (NOTE 1). The RTP video stream has its associated RTCP control flow as usual. The RTP H.224 stream could also use RTCP, if there is a reason at all for RTCP. Both RTP streams are mapped on separate H.248 streams in the IMS-AGW. The LS is only about the RTP H.224 stream.

The LS is essentially about "RTP topology"-dependent RTCP handling (NOTE 2).

NOTE 1 - Both RTP streams belong typically to the same RTP session (as well as additional RTP audio stream(s)).

NOTE 2 - See IETF RFC 5117 in general and Recommendation ITU-T H.248.88 in particular as framework for H.248 media gateways with (RTP, RTP) connection models.

CT4 would like to ask SA4 to provide clarification on following questions:

- Does the RTP source/sink (for terminology see RFC 7656) for FECC information use RTCP or not?
If RTCP is not used, there wouldn't be any RTCP resources reserved and allocated in the IMS-AGW.

- If RTCP is used, could you please detail the used RTCP packet/report types (e.g., only RTCP packet types SR, RR and BYE)?
If such level of information would not be available, then we would just consider a general RTCP control flow without further evaluation of RTCP Packet Type codepoints.

- Is it important that the RTCP control flow is end-to-end between H.224 stream endpoints, or is it also acceptable that RTCP and RTP are handled separately at the transport connections at both sides of a gateway?
If there is an important end-to-end significance of RTCP, then the IMS-AGW would need to provide a transparent forwarding behaviour for RTCP (as for H.224-over-RTP packets), which relates to an RTP topology related to transparent forwarding.
If not, the IMS-AGW required RTCP handling would be open, e.g. could also terminate/originate RTCP according to an RTP topology "Back-to-Back RTP endsystem" (because such an RTP handling behaviour is typically applied for RTP audio and RTP video streams in the IMS-AGW).

If the RTCP handling by the IMS-AGW could not be explicitly specified for FECC mode, then there might be different interpretations. Resulting in the fact that an RTP FECC endpoint could observe differences with regards to received RTCP SDES information, round-trip time measurements, etc.

2. Actions:

To SA4 group.

ACTION: 	CT4 kindly asks SA4 to provide clarifications on the above questions.

3. Date of Next CT and CT4 Meetings:

CT4#72	15th – 19st February 2016		Jeju Island, Republic of Korea

CT#70	07th – 08th December 2015	Sitges, Spain