TSG SA4#88 meeting	Tdoc S4(16)0397

18-22 Apr, 2016, Memphis, TN, USA	 rev. of S4-151492

Source:	Qualcomm, Incorporated

Title:	Revision of DESUDAPS-1: Common subjective testing framework for training and validation of SWB and FB P.835 test predictors

## Document for:	Agreement

Version:		1.2

## Agenda Item:	9.7

# [Editor’s note: This contribution updates the subjective test framework with the correct gain factors to achieve 0dB(A)SNR for generation of the fullband and superwideband reference set.]

# Introduction

This document describes a common framework for conducting subjective testing for training and validation of P.835 objective predictors. Such a framework is seen as necessary in order to minimize variations between subjective tests performed in different listening laboratories. The framework can be used for conducting further subjective tests necessary for conclusion of the DESUDAPS WI.

# Subjective test plan

## Traceability

The subjective test method is described in ITU-T Rec. P.835 (1) and the ITU-T Handbook of subjective testing practical procedures (2), with the following observations:

## Speech  database requirements

The source speech database (near end signal) to be used for data collection and listening tests shall consist of at least 8 samples (2 male and 2 female talkers, 2 samples per talker).

The speech material shall conform to the guidelines specified in the ITU-T handbook of subjective testing practical procedures, section 5, and section B.3 of ITU-T P.501 (3). Each sample shall be constructed according to the guidelines described in ITU-T P.835 section 5.1.4 (including 1s of leading and 1s of trailing silence) and normalized to an active speech level (4) of -26dBov. The source speech material shall be 16bit / 48kHz.

## Reference Conditions

Reference conditions shall follow the proposal in (6), which incorporates a spectral subtraction based distortion instead of the MNRU-based distortion typically used in subjective tests. The implementation of the processing for the SIG reference system and specification for NS Levels listed in Table 1 and 2 are found in Appendix A. An example batch processing script is also provided in Appendix A for generation of the reference condition set.

## Test Conditions

Test conditions shall be recorded from real handset devices or from mock-up terminals for offline processing as described in clause 3. Table 1 lists the recommended test conditions used for the recordings and listening tests. At least 6 out of the 8 noise types described should be included in the test to provide similarity of context between different labs. 2 of the 8 noise types can be replaced by either a clean speech transmission scenario (i.e. the background noise reproduction is disabled) or other noise types taken from the ETSI ES 202.396-1 / TS 103 224 databases.

Either handset, headset or handheld hands-free usage modes are acceptable.  The inclusion of hands-free and headset test cases are intended to span a larger range of degradations for the purposes of training of the objective predictor model. Some of the test conditions will have insufficient listening quality for practical voice communication.

Table 1 –Test and Reference conditions for narrowband subjective evaluation of noise reduction

| Reference Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| File | SIG. | SNR | Noise Type |  |
| i01 | Source (filtered) | No Noise | - |  |
| i02 | Source (filtered) | 0dB | Fullsize_Car1_130Kmh_binaural |  |
| i03 | Source (filtered) | 12dB | Fullsize_Car1_130Kmh_binaural |  |
| i04 | Source (filtered) | 24dB | Fullsize_Car1_130Kmh_binaural |  |
| i05 | Source (filtered) | 36dB | Fullsize_Car1_130Kmh_binaural |  |
| i06 | NS Level 1 | No Noise | - |  |
| i07 | NS Level 2 | No Noise | - |  |
| i08 | NS Level 3 | No Noise | - |  |
| i09 | NS Level 4 | No Noise | - |  |
| i10 | NS Level 3 | 24dB | Fullsize_Car1_130Kmh_binaural |  |
| i11 | NS Level 2 | 12dB | Fullsize_Car1_130Kmh_binaural |  |
| i12 | NS Level 1 | [0dB] | Fullsize_Car1_130Kmh_binaural |  |
| Test Conditions |  |  |  |  |
| File | Speech level @ MRP Handset/(handsfree or headset) | Noise level @ HATS ear simulators with ID correction | Noise Type | Description of Noise from EG 202 396-1 |
| i13 | -1.7/+1.3dBPa | L: 75,0 dB(A) / R: 73,0 dB(A) | Pub_Noise_binaural_V2 | Recording in a pub |
| i14 | -1.7/+1.3dBPa | L: 74,9 dB(A) / R: 73,9 dB(A) | Outside_Traffic_Road_binaural | Recording at pavement |
| i15 | -1.7/+1.3dBPa | L: 69,1 dB(A) / R: 69,6 dB(A) | Outside_Traffic_Crossroads_binaural | Recording at pavement |
| i16 | -1.7/+1.3dBPa | L: 68.2dB(A) / R:69.8dB(A) | Train_Station_binaural | Recording at departure platform |
| i17 | -1.7/+1.3dBPa | L: 69,1 dB(A) / R: 68,1 dB(A) | Fullsize_Car1_130Kmh_binaural | Recording in passenger cabin |
| i18 | -1.7/+1.3dBPa | L: 68,4 dB(A) / R: 67,3 dB(A) | Cafeteria_Noise_binaural | Recording at sales counter |
| i19 | -1.7/+1.3dBPa | L: 63,4 dB(A) / R: 61,9 dB(A) | Mensa_binaural | Recording in a cafeteria |
| i20 | -1.7/+1.3dBPa | L: 56,6 dB(A) / R: 57,8 dB(A) | Work_Noise_Office_Callcenter_binaural | Recording in a business office |

## Pre-processing of reference conditions

For the reference conditions, the clean speech and noise signals shall be filtered with the 20KBP filters available from ITU-T G.191 (9). The clean speech is then processed with the spectral subtraction algorithm in Annex A at the appropriate settings and, prior to mixing, normalized to an active speech level of -26dBov. The mixing shall be performed with the appropriate ITU-T G.191 tool to obtain the SNRs described in Table 1. The SNR is defined as the ratio between active speech levels to A-weighted noise level. Appendix A provides a batch script tool that can be used to generate the reference condition set and Annex B a block diagram of the necessary processing steps.

## Post-processing of test conditions

The uplink recordings of processed speech materials shall be normalized for use in the subjective test.  For the test conditions, the normalization gain is the gain necessary to obtain a recorded active speech level of -26dBov with a clean speech condition (no noise applied in the room). This normalization gain shall then be applied to all other test conditions for the same device (noise suppressed speech signals). In this way, the effect of level changes introduced by terminals in the presence of noise shall be part of the quality measurement.

## Calibration and equalization of headphones for presentation

Headphones used for presentation of the test material to the listening panel should be calibrated and equalized using a HATS conforming to ITU-T Recommendation P.58 and an artificial ear type 3.3 according to ITU-T Recommendation P.57 (10). The HATS is diffuse field equalized. The resulting one-third octave frequency response characteristic of the headphones used in the subjective experiments should be within the mask given in ITU-R BS.708, Annex 1, Figure 1.

Alternatively, equalization can be made using a subjective method as in IEC 62800-7, ensuring that all frequencies for full-band listening are satisfactorily reproduced.

The presentation of the test and reference conditions to listeners shall be diotic. The system gain is adjusted so that a speech segment of -26dBov corresponds to a presentation level of 73 dB SPL measured at the DRP with diffuse-field equalization.

## Requirements on the listening laboratory

Listening laboratory facilities shall comply with the recommendations provided in ITU-T P.800 (12).

## Experimental design

The use of the Balanced Blocks experimental design described in (2), section 3.3.2 is recommended. The experimental design shall include the 12 reference conditions and 8 test conditions per device under test, described in Table 1 (alternatively, for wideband, Table 2 can be used). A minimum of two and a maximum of six devices shall be included in any one test.

### The test and reference conditions should be reported for a total of 32 naïve listeners. The listeners shall be native speakers of the language used for the test. An example subjective test presentation sequence (i.e. “randomizations”) is provided in Annex A for a test with 32 listeners, 4 talkers, 4 samples per talker, 12 reference conditions and 48 test conditions (6 devices and 8 noise types). Each of the 4 presentation sequences in Annex A are presented to 8 of the 32 listeners.

128 votes per condition should be obtained, in order to achieve adequately low variance per condition. The number of votes per sample will depend on the number of samples per talker chosen (see section 2.2). A minimum of 2 samples per talker and 12 votes per sample shall be used in order to achieve adequately low variance per sample for model training purposes

## Training session

Prior to administration of the test, subjects shall be provided with written instructions on the test procedures. The use of training materials (e.g. videos, presentations) is encouraged to ensure the participants fully understand the task being requested. The training session shall be followed by a practice session containing 16 trials. The practice session shall include conditions representative of those presented in the test. An example is provided below:

| Trial | Sample | Condition |
| --- | --- | --- |
| 1 | m1s3.r01 | Reference - Source/No noise |
| 2 | f2s1.x06 | Test - Cafeteria |
| 3 | m2s4.r11 | Reference - NS-L2/12dB SNR |
| 4 | f1s1.r02 | Reference - Source/0dB SNR |
| 5 | m2s3.x03 | Test - Traffic-crossroads |
| 6 | f1s1.x05 | Test - Fullsize car |
| 7 | m2s1.r07 | Reference - NS-L2/No noise |
| 8 | f2s2.x02 | Traffic-road |
| 9 | m2s2.r03 | Reference - Source/12dB SNR |
| 10 | f2s2.r06 | Reference - NSL1/No noise |
| 11 | m2s4.x01 | Pub |
| 12 | f2s3.x08 | Test - Call-center |
| 13 | m2s4.r04 | Reference - Source/24dB SNR |
| 14 | f2s1.x04 | Test - Train station |
| 15 | m2s3.r12 | Reference - NS-L1/0dB SNR |
| 16 | f2s3.x07 | Test - Mensa babble |

Note: x is a device outside the set of DUTs.

# Set-up for acquisition of test conditions

## Terminal positioning and HATS calibration

For reproduction of the near-end signal, a HATS conforming to ITU-T P.58 (8) is used. The mouth simulator shall be equalized to achieve the reproduction accuracy described in 3GPP TS 26.132 clause 5.3.

For handset and headset mode testing, the mouth sensitivity gain shall be adjusted to produce an active speech level of -1.7 dBPa at MRP for a -26dBov input speech signal.

The handset terminals or mock-ups under test shall be set-up on HATS and the handset mounting position documented as described in 3GPP TS 26.132 clause 5.1.1 (7).

Headsets shall be set-up on HATS as described in 3GPP TS 26.132 clause 5.1.2.

For handheld hands-free mode the device is set-up using HATS as described in 3GPP TS 26.132 clause 5.1.3.3.

For handheld hands-free and headset mode testing, the mouth sensitivity gain shall be adjusted to produce an active speech level of +1.3dBPa at MRP for a -26dBov input speech signal.

## Background Noise reproduction

The background noise reproduction system shall be setup and equalized according to ETSI EG 202 396-1 (5) and/or ETSI TS 103 224. Noise types shall be reproduced at their realistic levels according to ETSI EG 202 396-1 clause 8 and ETSI TS 103 224 clause 7.2. The test conditions and noise files are specified in Table 1.

## Noise and speech playback synchronization

The noise and speech playback must be time aligned and synchronized. This is generally the case when playing the noise and speech files out of multiple channels of a same hardware interface but appropriate synchronization must be ensured when using separate hardware for noise and speech playback

## Convergence sequence

For proper convergence of terminal noise suppression the following time sequencing should be applied:

- The terminal is set-up and a call is established in noise free conditions,

- 2 seconds of noise only is applied in the test room with a linear amplitude fade-in from 0 to 2 seconds (noise ramp-up period), immediately followed by

- 6 seconds of noise only, immediately followed by

- 16 seconds (4 samples) of simultaneous speech and noise, immediately followed by

- actual test material to be used for listening panel presentation.

To allow for proper convergence of noise suppressors, an additional four sentences shall be placed immediately prior to the 32 sentences used in subjective testing.

## Example of noise and speech playback sequence including convergence period

Figure 1 illustrates an example of a playback time history for speech and one particular noise signal (Fullsize car1 130km/h binaural). The following applies to the example in figure 1:

- The speech signal is constructed by concatenating 8 seconds of silence with 36 speech samples of 4s each. The total length is therefore 152 seconds. The first 24 seconds are used for convergence of the noise suppression algorithm and not used for the purposes of listening panel presentation.

- The noise signal is constructed by concatenating 6 repetitions of a noise sample and the first 8 seconds of the 7th repetition. The noise sample is cut out, or generated from, the original noise files in the ETSI EG 202.396-1 / ETSI TS 103 224 databases to be 24s in length and fade-in and fade-out processing is applied to the first and last 50 samples (assuming noise at 48kHz sampling rate) to ensure zero-crossing of the signal amplitude at beginning and end of the sample. A linear fade-in is applied to the first 2 seconds of the concatenated noise signal, as this was found necessary for proper convergence of some terminals.

It is noted that by looping the noise every 24 seconds (a multiple of the speech sample length of 4s) the sharp transitions in the noise amplitude at the looping point coincide with the location of sample cutting for listening panel presentation. This avoids audible sharp transitions to fall during a speech segment.

Figure 1 - Noise and speech playback sequence, including convergence period.

# Annex A – Subjective test presentation sequence randomization

Provided as an attachment

# References

[1]	ITU-T Recommendation P.835 – Subjective test methodology for evaluation speech communication systems that include noise suppression algorithm

[2]	ITU-T TD 477 (GEN/12) – Handbook of subjective test practical procedures (temporary document) – Geneva, 18-27 January 2011

[3]	ITU-T Recommendation P.501 – Test signals for use in telephonometry

[4]	ITU-T Recommendation P.56 – Objective measurement of active speech level

[5]	ETSI EG 202.396-1 - Speech Quality performance in the presence of background noise; Part 1

[6]	AH-11-029, Better Reference System for the P.835 SIG Rating Scale, Q7/12 Rapporteur’s meeting, 20-21 June 2011, Geneva, Switzerland

[7]	ETSI TS 26.132 - Universal Mobile Telecommunications System (UMTS); LTE; Speech and video telephony terminal acoustic test specification (3GPP TS 26.132 version 10.1.0 Release 10)

[8]	ITU-T Recommendation P.58 – Head and Torso simulator for telephonometry

[9]	ITU-T Recommendation G.191 – Software tools for speech and audio coding standardization

[10]	ITU-T Recommendation P.57 – Artificial ears

[11]	3GPP TS 26.131 –  Terminal acoustic characteristics for telephony; Requirements (3GPP TS 26.131 version 12.4.0)

[12]	ITU-T Recommendation P.800 – Methods for subjective determination of transmission quality

[13]	VOICEBOX: Speech Processing Toolbox for MATLAB

http://www.ee.ic.ac.uk/hp/staff/dmb/voicebox/voicebox.html

[14]	ETSI TS 103 224 – A sound field reproduction method for terminal testing including a background noise database

# Appendix A:  MATLAB code and batch processing script for generation of reference conditions.

## NSLevel.m Matlab function

function []=NSLevels(noisemix,speech,nsparameter,output)

p=importdata(nsparameter);

[s,fs] = wavread(noisemix);

[s2] = wavread(speech);

[ss]=specsubm_mod(s,fs,p,s2);

wavwrite(ss,fs,output);

end

## Specsubm_mod.m Matlab function

function [ss,po]=specsubm_mod(s,fs,p, s2)

%SPECSUBM performs speech enhancement using spectral subtraction [SS,PO]=(S,FS,P,S2)

%

% S is the input mixture, speech + noise

% S2 is the clean speech (modification of original SPECSUBM)

% FS is the sampling rate of S and S2

% P is the parameter vector with details noted below:

%

% implementation of spectral subtraction algorithm by R Martin (rather slow)

% algorithm parameters: t* in seconds, f* in Hz, k* dimensionless

% 1: tg = smoothing time constant for signal power estimate (0.04): high=reverberant, low=musical

% 2: ta = smoothing time constant for signal power estimate

%        used in noise estimation (0.1)

% 3: tw = fft window length (will be rounded up to 2^nw samples)

% 4: tm = length of minimum filter (1.5): high=slow response to noise increase, low=distortion

% 5: to = time constant for oversubtraction factor (0.08)

% 6: fo = oversubtraction corner frequency (800): high=distortion, low=musical

% 7: km = number of minimisation buffers to use (4): high=waste memory, low=noise modulation

% 8: ks = oversampling constant (4)

% 9: kn = noise estimate compensation (1.5)

% 10:kf = subtraction floor (0.02): high=noisy, low=musical 0.01 < sub

% floor < 0.05

% 11:ko = oversubtraction scale factor (4): high=distortion, low=musical

%

% Refs:

%    (a) R. Martin. Spectral subtraction based on minimum statistics. In Proc EUSIPCO, pages 1182-1185, Edinburgh, Sept 1994.

%    (b) R. Martin. Noise power spectral density estimation based on optimal smoothing and minimum statistics.

%        IEEE Trans. Speech and Audio Processing, 9(5):504-512, July 2001.

%

%      Copyright (C) Mike Brookes 2004

%      Version: $Id: specsubm.m,v 1.4 2007/05/04 07:01:39 dmb Exp $

%

%   VOICEBOX is a MATLAB toolbox for speech processing.

%   Home page: http://www.ee.ic.ac.uk/hp/staff/dmb/voicebox/voicebox.html

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%   This program is free software; you can redistribute it and/or modify

%   it under the terms of the GNU General Public License as published by

%   the Free Software Foundation; either version 2 of the License, or

%   (at your option) any later version.

%

%   This program is distributed in the hope that it will be useful,

%   but WITHOUT ANY WARRANTY; without even the implied warranty of

%   MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

%   GNU General Public License for more details.

%

%   You can obtain a copy of the GNU General Public License from

%   http://www.gnu.org/copyleft/gpl.html or by writing to

%   Free Software Foundation, Inc.,675 Mass Ave, Cambridge, MA 02139, USA.

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if nargin<3 | isempty(p), po=[0.04 0.1 0.032 1.5 0.08 400 4 4 1.5 0.02 4].'; else po=p; end

if nargin < 4

s2 = s;

end

ns=length(s);

s2 = s2(1:ns);

ts=1/fs;

ss=zeros(ns,1);

ni=pow2(nextpow2(fs*po(3)/po(8))); % po(3): fft window length

ti=ni/fs;

nw=ni*po(8); % po(8): oversampling constant

nf=1+floor((ns-nw)/ni);

nm=ceil(fs*po(4)/(ni*po(7))); % po(4): length min filter, po(7) minBufferCount

%win=0.5*hamming(nw+1)/1.08;win(end)=[];

%win=sqrt(hamming(nw+1)); win(end)=[]; % for now always use sqrt hamming window

win=sqrt(hann(nw)); % for now always use sqrt hamming window

win=win/sqrt(sum(win(1:ni:nw).^2));       % normalize to give overall gain of 1

zg=exp(-ti/po(1)); % po(1) smoothing time constant

za=exp(-ti/po(2)); % po(2) smoothing time constant in noise estimate

zo=exp(-ti/po(5)); % po(5) time constant for oversubtraction factor

px=zeros(1+nw/2,1);

pxs = px;

pxn=px;

os=px;

pn=px;

mb=ones(1+nw/2,po(7))*nw/2; % po(7) minBufferCount

im=0;

osf=po(11)*(1+(0:nw/2).'*fs/(nw*po(6))).^(-1); % po(6) oversubtraction corner frequency, po(11) oversubtraction scale factor

imidx=[13 21]';

x2im=zeros(length(imidx),nf);

osim=x2im;

pnim=x2im;

pxnim=x2im;

qim=x2im;

q = px;

for is=1:nf

idx=(1:nw)+(is-1)*ni;

x=rfft(s(idx).*win);

xs=rfft(s2(idx).*win);

x2=x.*conj(x);

xs2=xs.*conj(xs);

pxn=za*pxn+(1-za)*x2; % signal power estimate

im=rem(im+1,nm);

if im % always if im ~= 0

mb(:,1)=min(mb(:,1),pxn);

else % only if im == 0 (add to MinBuffers)

mb=[pxn,mb(:,1:po(7)-1)]; % po(7) minBufferCount

end

pn=po(9)*min(mb,[],2); % po(9) noise estimate compensation

%os : oversubtraction factor

os=zo*os+(1-zo)*(1+osf.*pn./(pn+pxn));

px= zg*px+(1-zg)*x2;

pxs= zg*pxs+(1-zg)*xs2;

if any(x2 == 0) | any(pn < 0) | any(px == 0) | any(os.*pn < 0)

q = 0;

else

q=max(po(10)*sqrt(pn), 1-sqrt(os.*pn./px)); % po(10) subtraction floor

%        q(1)=0;

%         q=filter(1/3*[1 1 1],1,q);%q=filter(0.8,[1 -0.2],q);

%        q=filter(1/3*[1 1 1],1,q(end:-1:1));

%        q=q(end:-1:1);

%        q(1)=0;

%        plot(q);drawnow;

end

%D = irfft(x.*q);

D = irfft(xs.*abs(q));

%   Y= xs.*(1-sqrt(os.*pn./xs2));

%   Y(Y<sqrt(po(10)*pn))=sqrt(po(10)*pn);

%    D = irfft(Y);

ss(idx)=ss(idx)+D.*win;

end

ss = ss./max(ss)*0.9;

if nargout==0

soundsc([s; ss],fs);

end

Parameters for derivation of the noise suppression levels in Table 1 and Table 2 (AmEng speech):

| NS Level | tg | ta | tw | tm | to | fo | km | ks | kn | kf | ko | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 0.04 | 0.1 | 0.032 | 1.6 | 0.64 | 400 | 4 | 4 | 1.5 | 0.02 | 4 | 12.0 |
| 3 | 0.04 | 0.2 | 0.032 | 1.2 | 0.64 | 400 | 4 | 4 | 1.5 | 0.01 | 4 | 9.0 |
| 2 | 0.04 | 0.8 | 0.032 | 0.8 | 0.32 | 400 | 4 | 4 | 1.5 | 0.01 | 4 | 6.0 |
| 1 | 0.08 | 1.6 | 0.032 | 0.4 | 0.16 | 800 | 4 | 4 | 2.0 | 0.02 | 4 | 3.0 |

Another similar set of parameters is provided below. These parameters prevent noise reduction features from being too aggressive and were found to be appropriate if using the reference set from Table 2 with French samples:

| NS Level | tg | ta | tw | tm | to | fo | km | ks | kn | kf | ko | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 0.04 | 0.1 | 0.032 | 3.0 | 0.64 | 400 | 4 | 4 | 1.5 | 0.02 | 4 | 12.0 |
| 3 | 0.04 | 0.3 | 0.032 | 3.0 | 0.64 | 400 | 4 | 4 | 1.5 | 0.01 | 4 | 9.0 |
| 2 | 0.04 | 0.6 | 0.032 | 3.0 | 0.32 | 400 | 4 | 4 | 1.5 | 0.01 | 4 | 6.0 |
| 1 | 0.08 | 0.9 | 0.032 | 3.0 | 0.16 | 800 | 4 | 4 | 2.0 | 0.02 | 4 | 3.0 |

## 6.3	Batch processing script for generation of reference condition set

@echo off

: Reference condition set generator for P.835 testing

: v1.0 16/01/2012, V1.5 13/02/2015, V2.0 11/13/2015

: Andre Schevciw, Fredrik Stenmark

: Multimedia R&D and Standards

: Qualcomm, Inc.

: 5775 Morehouse Drive

: San Diego, CA 92121

: CURRENT LIMITATIONS

: * 1 - Source speech files must be in 48 kHz, 16 bit, mono, wav format and have an

: * exact : length of 6912000 samples (144 seconds)!!

: * 2 - Source noise files must be in 48kHz, 16 bit, stereo, wav format and have a

: * length : of 30 seconds. File name must contain no spaces.

: * 3 - Default noise type is Fullsize_Car1_130kmh_binaural from ETSI database,

: * please read section 3 comments below.

:

: NECESSARY EXECUTABLES

: * From ITU-T Rec. G.191 (03/10) (Available from http://www.itu.int/rec/T-REC-G.191/en)

: * astrip.exe, sv56demo.exe, stereoop.exe, filter.exe (example programs from ITU-T

: * G.191)

: * add.exe (add function from Basop module, demo program not available!)

:

: * From McGill AFSP tools

: * (Available from http://www mmsp.ece.mcgill.ca/documents/Software/)

: * CopyAudio.exe, GenNoise.exe, FiltAudio.exe

:

: * From MATLAB compiling

: * NSLevels.exe in section 5 must be compiled in Matlab by running:

: *

: * mcc -m NSLevels.m

: *

: * with NSLevels.m Matlab script provided in contribution Tdoc S4-AHQ042 - Appendix A

:

: NECESSARY FILES AND DIRECTORY STRUCTURE

: * (Provided as an attachment to contribution Tdoc S4-AHQ042)

: * .\config\NSLVL1.txt

: * .\config\NSLVL2.txt

: * .\config\NSLVL3.txt

: * .\config\NSLVL4.txt

: * .\speech\?.wav (? being the source speech material to be processed)

: * .\noise\Fullsize_Car1_130Kmh_binaural.wav

Set -e

CLS

SETLOCAL

: * source speech material file name (without extension)

SET SOURCE=R14_Dyna-Src_P835_144secs

: * source noise material file name (without extension)

SET NOISE=Fullsize_Car1_130Kmh_binaural

: * initial pause to be added to the beginning of source speech file. Corresponds to 16

: * seconds in 16kHz.

SET /A INITIAL_PAUSE_FB=768000

SET /A INITIAL_PAUSE_SWB=512000

SET /A INITIAL_PAUSE_WB=256000

SET /A INITIAL_PAUSE_NB=128000

: * final pause to be added to the end of source speech file. Corresponds to 8 seconds

: * in 16kHz.

SET /A FINAL_PAUSE_FB=384000

SET /A FINAL_PAUSE_SWB=256000

SET /A FINAL_PAUSE_WB=128000

SET /A FINAL_PAUSE_NB=64000

: * file names for text files containing the specsub_mod.m Matlab parameter vectors

SET NSLEVEL1=NSLVL1.txt

SET NSLEVEL2=NSLVL2.txt

SET NSLEVEL3=NSLVL3.txt

SET NSLEVEL4=NSLVL4.txt

: * gain to normalize noise to account for A-weighting of FB, SWB, WB and NB car Noise. Note Speech levels have changed to -40dBov

SET NORMFACTOR_FB=2.305

SET NORMFACTOR_SWB=1.772

SET NORMFACTOR_WB=1.6816

SET NORMFACTOR_NB=2.891

: * The A-weighting delta for Car is 16.3 dB in FB, 11.5 dB in SWB, 10.5 dB in WB and 7.7 dB in NB. To incorporate this, the Signal level was dropped to -40 dBov to avoid clipping when the A-weighed Level of the Noise file is used.

: * create temporary and output directories

mkdir .\tempFB

mkdir .\%SOURCE%_refset_FB

mkdir .\tempSWB

mkdir .\%SOURCE%_refset_SWB

:---------------------------------------------------------------------------------------

: 1.Speech Filtering For Fullband

:

: * FB

copyaudio -F "noheader" .\speech\%SOURCE%.wav .\tempFB\%SOURCE%.raw

filter -q 20kbp .\tempFB\%SOURCE%.raw .\tempFB\1_%SOURCE%.raw

astrip -q -smooth -sample -wlen 480 .\tempFB\1_%SOURCE%.raw .\tempFB\2_%SOURCE%.raw

GenNoise -d "0.5/32768" -n %INITIAL_PAUSE_FB% -s 48000 -F "noheader" -D "integer16" .\tempFB\initdither.raw

GenNoise -d "0.5/32768" -n %FINAL_PAUSE_FB% -s 48000 -F "noheader" -D "integer16" .\tempFB\finaldither.raw

copyAudio -C -t "noheader" -P "integer16, 0, 48000, native, 1, default" .\tempFB\initdither.raw .\tempFB\2_%SOURCE%.raw .\tempFB\finaldither.raw .\tempFB\3_%SOURCE%.raw

sv56demo -lev -40 -q .\tempFB\3_%SOURCE%.raw .\tempFB\4_%SOURCE%.raw

sv56demo -lev -26 -q .\tempFB\3_%SOURCE%.raw .\tempFB\26_%SOURCE%.raw

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\4_%SOURCE%.raw .\tempFB\40_%SOURCE%.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\4_%SOURCE%.raw .\%SOURCE%_refset_FB\%SOURCE%.wav

:---------------------------------------------------------------------------------------

: 2. Noise filtering and concatenation

: * FB

copyaudio -F "noheader" .\noise\%NOISE%.wav .\tempFB\%NOISE%.raw

stereoop -mono .\tempFB\%NOISE%.raw .\tempFB\2_%NOISE%.raw

filter -q 20kbp .\tempFB\2_%NOISE%.raw .\tempFB\3_%NOISE%.raw

astrip -q -smooth -sample -wlen 4 -start 301 -end 1152300 .\tempFB\3_%NOISE%.raw .\tempFB\4_%NOISE%.raw

copyAudio -C -t "noheader" -P "integer16, 0, 48000, native, 1, default" .\tempFB\4_%NOISE%.raw .\tempFB\4_%NOISE%.raw .\tempFB\4_%NOISE%.raw .\tempFB\4_%NOISE%.raw .\tempFB\4_%NOISE%.raw .\tempFB\4_%NOISE%.raw .\tempFB\4_%NOISE%.raw .\tempFB\5_%NOISE%.raw

:---------------------------------------------------------------------------------------

: 3. Noise level normalization

:

: * The normalization factor found is entered by the variable %NORMFACTOR% below.

: * The default NORMFACTORs are for Fullsize_Car1_130kmh_binaural.wav.

: * If using a different noise file, please update this variable

:

: * FB

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g %NORMFACTOR_FB% .\tempFB\5_%NOISE%.raw .\tempFB\6_%NOISE%.raw

:---------------------------------------------------------------------------------------

: 4. Source Speech and noise mixing at different SNRs

:

: * FB

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 1 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_0.raw

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 0.7071 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_3.raw

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 1/2 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_6.raw

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 0.3535 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_9.raw

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 1/4 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_12.raw

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 1/16 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_24.raw

copyAudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -g 1/64 .\tempFB\6_%NOISE%.raw .\tempFB\6_%NOISE%_36.raw

add -q .\tempFB\6_%NOISE%_0.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_0.raw

add -q .\tempFB\6_%NOISE%_3.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_3.raw

add -q .\tempFB\6_%NOISE%_6.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_6.raw

add -q .\tempFB\6_%NOISE%_9.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_9.raw

add -q .\tempFB\6_%NOISE%_12.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_12.raw

add -q .\tempFB\6_%NOISE%_24.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_24.raw

add -q .\tempFB\6_%NOISE%_36.raw .\tempFB\4_%SOURCE%.raw 480 1 20000 .\tempFB\5_%SOURCE%_36.raw

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_12.raw .\tempFB\%SOURCE%_12.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_3.raw .\tempFB\%SOURCE%_3.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_6.raw .\tempFB\%SOURCE%_6.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_9.raw .\tempFB\%SOURCE%_9.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_0.raw .\%SOURCE%_refset_FB\%SOURCE%_0.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_12.raw .\%SOURCE%_refset_FB\%SOURCE%_12.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_24.raw .\%SOURCE%_refset_FB\%SOURCE%_24.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\5_%SOURCE%_36.raw .\%SOURCE%_refset_FB\%SOURCE%_36.wav

:---------------------------------------------------------------------------------------

: 5. Applies the spectral subtraction algorithm (NSLevels) and re-normalizes synthesized speech to

: -40dBov ASL

:

: * FB

NSLevels .\tempFB\%SOURCE%_3.wav .\tempFB\40_%SOURCE%.wav .\config\%NSLEVEL1% .\tempFB\%SOURCE%_NSLVL1.wav

NSLevels .\tempFB\%SOURCE%_6.wav .\tempFB\40_%SOURCE%.wav .\config\%NSLEVEL2% .\tempFB\%SOURCE%_NSLVL2.wav

NSLevels .\tempFB\%SOURCE%_9.wav .\tempFB\40_%SOURCE%.wav .\config\%NSLEVEL3% .\tempFB\%SOURCE%_NSLVL3.wav

NSLevels .\tempFB\%SOURCE%_12.wav .\tempFB\40_%SOURCE%.wav .\config\%NSLEVEL4% .\tempFB\%SOURCE%_NSLVL4.wav

copyaudio -F "noheader" .\tempFB\%SOURCE%_NSLVL1.wav .\tempFB\%SOURCE%_NSLVL1.raw

copyaudio -F "noheader" .\tempFB\%SOURCE%_NSLVL2.wav .\tempFB\%SOURCE%_NSLVL2.raw

copyaudio -F "noheader" .\tempFB\%SOURCE%_NSLVL3.wav .\tempFB\%SOURCE%_NSLVL3.raw

copyaudio -F "noheader" .\tempFB\%SOURCE%_NSLVL4.wav .\tempFB\%SOURCE%_NSLVL4.raw

sv56demo -q -lev -40 .\tempFB\%SOURCE%_NSLVL1.raw .\tempFB\%SOURCE%_NSLVL1_norm.raw

sv56demo -q -lev -40 .\tempFB\%SOURCE%_NSLVL2.raw .\tempFB\%SOURCE%_NSLVL2_norm.raw

sv56demo -q -lev -40 .\tempFB\%SOURCE%_NSLVL3.raw .\tempFB\%SOURCE%_NSLVL3_norm.raw

sv56demo -q -lev -40 .\tempFB\%SOURCE%_NSLVL4.raw .\tempFB\%SOURCE%_NSLVL4_norm.raw

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL1_norm.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL1.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL2_norm.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL2.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL3_norm.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL3.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL4_norm.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL4.wav

:---------------------------------------------------------------------------------------

: 6. Degraded speech and noise mixing at different SNRs

:

: * FB

add -q .\tempFB\%SOURCE%_NSLVL1_norm.raw .\tempFB\6_%NOISE%_0.raw 480 1 20000 .\tempFB\%SOURCE%_NSLVL1_0.raw

add -q .\tempFB\%SOURCE%_NSLVL2_norm.raw .\tempFB\6_%NOISE%_12.raw 480 1 20000 .\tempFB\%SOURCE%_NSLVL2_12.raw

add -q .\tempFB\%SOURCE%_NSLVL3_norm.raw .\tempFB\6_%NOISE%_24.raw 480 1 20000 .\tempFB\%SOURCE%_NSLVL3_24.raw

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL1_0.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL1_0.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL2_12.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL2_12.wav

copyaudio -t "noheader" -P "integer16, 0, 48000, native, 1, default" -F "WAVE" -D "integer16" .\tempFB\%SOURCE%_NSLVL3_24.raw .\%SOURCE%_refset_FB\%SOURCE%_NSLVL3_24.wav

:

:---------------------------------------------------------------------------------------

: 1.Speech Filtering for Superwideband

:

: * SWB

copyaudio -F "noheader" .\speech\%SOURCE%.wav .\tempSWB\%SOURCE%.raw

filter -q -up HQ2 .\tempSWB\%SOURCE%.raw .\tempSWB\1a_%SOURCE%.raw

filter -q -down HQ3 .\tempSWB\1a_%SOURCE%.raw .\tempSWB\1b_%SOURCE%.raw

filter -q 14kbp .\tempSWB\1b_%SOURCE%.raw .\tempSWB\1c_%SOURCE%.raw

astrip -q -smooth -sample -wlen 320 .\tempSWB\1c_%SOURCE%.raw .\tempSWB\2_%SOURCE%.raw

GenNoise -d "0.5/32768" -n %INITIAL_PAUSE_SWB% -s 32000 -F "noheader" -D "integer16" .\tempSWB\initdither.raw

GenNoise -d "0.5/32768" -n %FINAL_PAUSE_SWB% -s 32000 -F "noheader" -D "integer16" .\tempSWB\finaldither.raw

copyAudio -C -t "noheader" -P "integer16, 0, 32000, native, 1, default" .\tempSWB\initdither.raw .\tempSWB\2_%SOURCE%.raw .\tempSWB\finaldither.raw .\tempSWB\3_%SOURCE%.raw

sv56demo -lev -40 -q .\tempSWB\3_%SOURCE%.raw .\tempSWB\4_%SOURCE%.raw

sv56demo -lev -26 -q .\tempSWB\3_%SOURCE%.raw .\tempSWB\26_%SOURCE%.raw

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\4_%SOURCE%.raw .\tempSWB\40_%SOURCE%.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\4_%SOURCE%.raw .\%SOURCE%_refset_SWB\%SOURCE%.wav

:---------------------------------------------------------------------------------------

: 2. Noise filtering and concatenation

: * SWB

copyaudio -F "noheader" .\noise\%NOISE%.wav .\tempSWB\%NOISE%.raw

stereoop -mono .\tempSWB\%NOISE%.raw .\tempSWB\2_%NOISE%.raw

filter -q -up HQ2 .\tempSWB\2_%NOISE%.raw .\tempSWB\2a_%NOISE%.raw

filter -q -down HQ3 .\tempSWB\2a_%NOISE%.raw .\tempSWB\2b_%NOISE%.raw

filter -q 14kbp .\tempSWB\2b_%NOISE%.raw .\tempSWB\3_%NOISE%.raw

astrip -q -smooth -sample -wlen 4 -start 201 -end 768200 .\tempSWB\3_%NOISE%.raw .\tempSWB\4_%NOISE%.raw

copyAudio -C -t "noheader" -P "integer16, 0, 32000, native, 1, default" .\tempSWB\4_%NOISE%.raw  .\tempSWB\4_%NOISE%.raw .\tempSWB\4_%NOISE%.raw .\tempSWB\4_%NOISE%.raw .\tempSWB\4_%NOISE%.raw .\tempSWB\4_%NOISE%.raw .\tempSWB\4_%NOISE%.raw .\tempSWB\5_%NOISE%.raw

:---------------------------------------------------------------------------------------

: 3. Noise level normalization

:

: * The normalization factor found is entered by the variable %NORMFACTOR% below.

: * The default NORMFACTORs are for Fullsize_Car1_130kmh_binaural.wav.

: * If using a different noise file, please update this variable

:

: * SWB

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g %NORMFACTOR_SWB% .\tempSWB\5_%NOISE%.raw .\tempSWB\6_%NOISE%.raw

:

:---------------------------------------------------------------------------------------

: 4. Source Speech and noise mixing at different SNRs

:

: * SWB

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 1 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_0.raw

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 0.7071 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_3.raw

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 1/2 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_6.raw

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 0.3535 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_9.raw

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 1/4 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_12.raw

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 1/16 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_24.raw

copyAudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -g 1/64 .\tempSWB\6_%NOISE%.raw .\tempSWB\6_%NOISE%_36.raw

add -q .\tempSWB\6_%NOISE%_0.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_0.raw

add -q .\tempSWB\6_%NOISE%_3.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_3.raw

add -q .\tempSWB\6_%NOISE%_6.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_6.raw

add -q .\tempSWB\6_%NOISE%_9.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_9.raw

add -q .\tempSWB\6_%NOISE%_12.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_12.raw

add -q .\tempSWB\6_%NOISE%_24.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_24.raw

add -q .\tempSWB\6_%NOISE%_36.raw .\tempSWB\4_%SOURCE%.raw 320 1 20000 .\tempSWB\5_%SOURCE%_36.raw

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_12.raw .\tempSWB\%SOURCE%_12.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_3.raw .\tempSWB\%SOURCE%_3.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_6.raw .\tempSWB\%SOURCE%_6.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_9.raw .\tempSWB\%SOURCE%_9.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_0.raw .\%SOURCE%_refset_SWB\%SOURCE%_0.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_12.raw .\%SOURCE%_refset_SWB\%SOURCE%_12.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_24.raw .\%SOURCE%_refset_SWB\%SOURCE%_24.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\5_%SOURCE%_36.raw .\%SOURCE%_refset_SWB\%SOURCE%_36.wav

:

:---------------------------------------------------------------------------------------

: 5. Applies the spectral subtraction algorithm (NSLevels) at -26 dBov and re-normalizes speech to

: -40dBov ASL

:

: * SWB

NSLevels .\tempSWB\%SOURCE%_3.wav .\tempSWB\40_%SOURCE%.wav .\config\%NSLEVEL1% .\tempSWB\%SOURCE%_NSLVL1.wav

NSLevels .\tempSWB\%SOURCE%_6.wav .\tempSWB\40_%SOURCE%.wav .\config\%NSLEVEL2% .\tempSWB\%SOURCE%_NSLVL2.wav

NSLevels .\tempSWB\%SOURCE%_9.wav .\tempSWB\40_%SOURCE%.wav .\config\%NSLEVEL3% .\tempSWB\%SOURCE%_NSLVL3.wav

NSLevels .\tempSWB\%SOURCE%_12.raw .\tempSWB\40_%SOURCE%.wav .\config\%NSLEVEL4% .\tempSWB\%SOURCE%_NSLVL4.wav

copyaudio -F "noheader" .\tempSWB\%SOURCE%_NSLVL1.wav .\tempSWB\%SOURCE%_NSLVL1.raw

copyaudio -F "noheader" .\tempSWB\%SOURCE%_NSLVL2.wav .\tempSWB\%SOURCE%_NSLVL2.raw

copyaudio -F "noheader" .\tempSWB\%SOURCE%_NSLVL3.wav .\tempSWB\%SOURCE%_NSLVL3.raw

copyaudio -F "noheader" .\tempSWB\%SOURCE%_NSLVL4.wav .\tempSWB\%SOURCE%_NSLVL4.raw

sv56demo -q -lev -40 .\tempSWB\%SOURCE%_NSLVL1.raw .\tempSWB\%SOURCE%_NSLVL1_norm.raw

sv56demo -q -lev -40 .\tempSWB\%SOURCE%_NSLVL2.raw .\tempSWB\%SOURCE%_NSLVL2_norm.raw

sv56demo -q -lev -40 .\tempSWB\%SOURCE%_NSLVL3.raw .\tempSWB\%SOURCE%_NSLVL3_norm.raw

sv56demo -q -lev -40 .\tempSWB\%SOURCE%_NSLVL4.raw .\tempSWB\%SOURCE%_NSLVL4_norm.raw

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL1_norm.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL1.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL2_norm.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL2.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL3_norm.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL3.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL4_norm.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL4.wav

:

:---------------------------------------------------------------------------------------

: 6. Degraded speech and noise mixing at different SNRs

:

: * SWB

add -q .\tempSWB\%SOURCE%_NSLVL1_norm.raw .\tempSWB\6_%NOISE%_0.raw 320 1 20000 .\tempSWB\%SOURCE%_NSLVL1_0.raw

add -q .\tempSWB\%SOURCE%_NSLVL2_norm.raw .\tempSWB\6_%NOISE%_12.raw 320 1 20000 .\tempSWB\%SOURCE%_NSLVL2_12.raw

add -q .\tempSWB\%SOURCE%_NSLVL3_norm.raw .\tempSWB\6_%NOISE%_24.raw 320 1 20000 .\tempSWB\%SOURCE%_NSLVL3_24.raw

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL1_0.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL1_0.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL2_12.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL2_12.wav

copyaudio -t "noheader" -P "integer16, 0, 32000, native, 1, default" -F "WAVE" -D "integer16" .\tempSWB\%SOURCE%_NSLVL3_24.raw .\%SOURCE%_refset_SWB\%SOURCE%_NSLVL3_24.wav

:

# APPENDIX B – Format for exchange of validation samples and subjective test results.

A spreadsheet template is provided as an attachment to this permanent document. Results of the training and validation subjective tests shall be exchanged in the template format.

The audio data exchanged shall follow these guidelines:

  - Processed, Unprocessed at MRP, Unprocessed at DUT microphone and clean data must be provided for each sample of the LOT.

  - The three signals may be provided in WAV format.

  - The three related signals must be merged into one single file with three channels (order: Proc / Unprocessed at MRP / Unprocessed at DUT / Clean).

  - All 4-channel-files must be within one directory; no directories per device, per talker, etc.

  - The alphabetical sorting of such a directory MUST correspond to the order given in the Excel document with the subjective data! (Numbering the files by the ID-column may help here)