3GPP TSG-SA4 Meeting #88 	S4-160479

# Memphis, USA, 18-22 April 2016                                                        update of S4-160437

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | 0538 | rev | 1 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MooD Proxy Server Load Limitation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MI-MooD |  |  |  |  |  |  | Date: |  | 2016-04-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | MooD is based on the function of gauging the popularity of certain content before initiating broadcast delivery of that content. Currently, all requests for content that is marked as MooD eligible are served through the MooD proxy server, which might cause a huge load on these proxy servers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This change will enable the control of the load that MooD proxy servers experience and will address concerns on the scalability of MooD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All MooD eligible content will have to flow through the proxy server, which may cause it to sustain a large load. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 12.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR … CR … |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service; Stage 1".

[3]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 25.346: "Introduction of Multimedia Broadcast/Multicast Service (MBMS) in the Radio Access Network (RAN); Stage 2".

[6]	IETF STD 0064/RFC 3550 (July 2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick, V. Jacobson.

[7]	IETF STD 0006/RFC 0768 (August 1980): "User Datagram Protocol", J. Postel.

[8]	IETF STD 0005/RFC 0791 (September 1981): "Internet Protocol", J. Postel.

[9]	IETF RFC 3926 (October 2004): "FLUTE - File Delivery over Unidirectional Transport", T. Paila, M. Luby, R. Lehtonen, V. Roca, R. Walsh.

[10]	IETF RFC 3450 (December 2002): "Asynchronous Layered Coding (ALC) Protocol Instantiation", M. Luby, J. Gemmell, L. Vicisano, L. Rizzo, J. Crowcroft.

[11]	IETF RFC 3451 (December 2002): "Layered Coding Transport (LCT) Building Block", M. Luby, J. Gemmell, L. Vicisano, L. Rizzo, M. Handley, J. Crowcroft.

[12]	IETF RFC 5052 (August 2007): "Forward Error Correction (FEC) Building Block", M. Luby, M. Watson, L. Vicisano.

[13]	IETF RFC 3695 (February 2004): "Compact Forward Error Correction (FEC) Schemes", M. Luby, L. Vicisano.

[14]	IETF RFC 4566 (July 2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[15]	IETF RFC 4570 (July 2006): "Session Description Protocol (SDP) Source Filters", B. Quinn, R. Finlayson.

[16]	Void.

[17]	IETF RFC 3048 (January 2001): "Reliable Multicast Transport Building Blocks for One-to-Many Bulk-Data Transfer", B. Whetten, L. Vicisano, R. Kermode, M. Handley, S. Floyd, M. Luby.

[18]	IETF RFC 2616 (June 1999): "Hypertext Transfer Protocol -- HTTP/1.1".

[19]	IETF STD 0066/RFC 3986 (January 2005): "Uniform Resource Identifier (URI)".

[20]	3GPP TS 33.246: "3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)".

[21]	OMG: "Unified Modeling Language (UML), version 1.5" (formal/03-03-01).

[22]	W3C Recommendation 28 October 2004: "XML Schema Part 2: Datatypes Second Edition".

[23]	IETF RFC 5234 (January 2008): "Augmented BNF for Syntax Specifications: ABNF", D. Crocker and P. Overell.

[24]	3GPP TS 26.290: "Audio codec processing functions; Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec; Transcoding functions".

[25]	3GPP TS 26.304: "Floating-point ANSI-C code for the Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec".

[26]	3GPP TS 26.273: "Speech codec speech processing functions; Extended Adaptive Multi-Rate - Wideband (AMR-WB+) speech codec; Fixed-point ANSI-C code".

[27]	Void.

[28]	3GPP TS 26.401: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description".

[29]	3GPP TS 26.410: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Floating-point ANSI-C code".

[30]		3GPP TS 26.411: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Fixed-point ANSI-C code".

[31]	W3C Recommendation 04 February 2004: "Extensible Markup Language (XML) 1.1", T. Bray, J. Paoli, C. Sperberg-McQueen, E. Maler, F. Yergeau and J. Cowan.

[32]		3GPP TS 26.244: "Transparent end-to-end streaming service; 3GPP file format (3GP)".

[33]	IETF RFC 4867 (April 2007): "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs", J. Sjoberg, M. Westerlund, A. Lakaniemi, Q. Xie.

[34]	IETF RFC 4352 (January 2006): "RTP Payload Format for the Extended Adaptive Multi-Rate Wideband (AMR-WB+) Audio Codec", Sjoberg J. et al.

[35]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[36]	Void.

[37]	IETF RFC 2557 (March 1999): "MIME Encapsulation of Aggregate Documents, such as HTML (MHTML)", J. Palme, A. Hopmann, N. Shelness.

[38]	IETF RFC 3890 (September 2004): "A Transport Independent Bandwidth Modifier for the Session Description Protocol (SDP)", M. Westerlund.

[39]		IETF RFC 3556 (July 2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth", S. Casner.

[40]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[41]	IETF RFC 3640 (November 2003): "RTP Payload Format for Transport of MPEG-4 Elementary Streams", J. van der Meer, D. Mackie, V. Swaminathan, D. Singer, P. Gentric.

[42]	IETF RFC 1952 (May 1996): "GZIP file format specification version 4.3", P. Deutsch.

[43]	ITU-T Recommendation H.264 (04/2013): "Advanced video coding for generic audiovisual services".

[44]	Void.

[45]	ITU-T Recommendation H.263 (1998): "Video coding for low bit rate communication".

[46]	ITU-T Recommendation H.263 - Annex X (04/01): "Annex X: Profiles and levels definition".

[47]	3GPP TS 26.234: "Transparent end-to-end streaming service; Protocols and codecs".

[48]	3GPP TS 26.071: "AMR speech codec; General description".

[49]	3GPP TS 26.090: "AMR speech codec; Transcoding functions".

[50]	3GPP TS 26.073: "AMR speech Codec; C-source code".

[51]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi-Rate (AMR) speech codec".

[52]	3GPP TS 26.171: "AMR speech codec, wideband; General description".

[53]	3GPP TS 26.190: "Mandatory Speech Codec speech processing functions AMR Wideband speech codec; Transcoding functions".

[54]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[55]	3GPP TS 26.204: "ANSI-C code for the floating-point Adaptive Multi-Rate Wideband (AMR-WB) speech codec".

[56] 	Scalable Polyphony MIDI Specification Version 1.0, RP-34, MIDI Manufacturers Association, Los Angeles, CA, February 2002.

[57] 	Scalable Polyphony MIDI Device 5-to-24 Note Profile for 3GPP Version 1.0, RP-35, MIDI Manufacturers Association, Los Angeles, CA, February 2002.

[58]	"Standard MIDI Files 1.0", RP-001, in "The Complete MIDI 1.0 Detailed Specification, Document Version 96.1", The MIDI Manufacturers Association, Los Angeles, CA, USA, February 1996.

[59]	Mobile DLS, MMA specification v1.0, RP-41 Los Angeles, CA, USA. 2004.

[60]	Mobile XMF Content Format Specification, MMA specification v1.0, RP-42, Los Angeles, CA, USA. 2004.

[61]	ITU-T Recommendation T.81 (1992) | ISO/IEC 10918-1:1993: "Information technology - Digital compression and coding of continuous-tone still images - Requirements and guidelines".

[62]	C-Cube Microsystems (September 1992): "JPEG File Interchange Format", Version 1.02.

[63]	CompuServe Incorporated (1987): "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA. See at http://www.dcs.ed.ac.uk/home/mxr/gfx/2d/GIF87a.txt.

[64]	CompuServe Incorporated (1990): "Graphics Interchange Format: Version 89a", Columbus, OH, USA.

[65]	IETF RFC 2083 (March 1997): "PNG (Portable Networks Graphics) Specification Version 1.0", T. Boutell.

[66]	W3C Working Draft 27 October 2004: "Scalable Vector Graphics (SVG) 1.2", http://www.w3.org/TR/2004/WD-SVG12-20041027/.

[67]	W3C Working Draft 13 August 2004: "Mobile SVG Profile: SVG Tiny, Version 1.2", http://www.w3.org/TR/2004/WD-SVGMobile12-20040813/.

[68]	Standard ECMA-327 (June 2001): "ECMAScript 3rd Edition Compact Profile".

[69]	Void.

[70]	ISO/IEC 10646-1 (2000): "Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane".

[71]	The Unicode Consortium: "The Unicode Standard", Version 3.0 Reading, MA, Addison-Wesley Developers Press, 2000, ISBN 0-201-61633-5.

[72]	3GPP TS 26.245: "Transparent end-to-end Packet switched Streaming Service (PSS); Timed text format".

[73]	IETF RFC 4646: "Tags for the Identification of Languages".

[74]	ISO 639: "Codes for the representation of names of languages".

[75]	ISO 3166: "Codes for the representation of names of countries and their subdivisions".

[76]	Void.

[77]	3GPP TS 23.003: "Numbering, addressing and identification".

[78]	IETF RFC 5905, "Network Time Protocol Version 4: Protocol and Algorithms Specification", D. Mills, J. Martin, J. Burbank and W. Kasch, June 2010.

[79]	OMA Push OTA Protocol (25-April-2001): WAP-235-PushOTA-20010425-a

http://www.openmobilealliance.org/tech/affiliates/LicenseAgreement.asp?DocName=/wap/wap-235-pushota-20010425-a.pdf

[80]	IETF RFC 3711 (March 2004): "The Secure Real-time Transport Protocol (SRTP)", M. Baugher, D. McGrew, M. Naslund, E. Carrara, K. Norrman.

[81]	Void.

[82]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings", Josefsson S., Ed., October 2006.

[83]	IETF RFC 3023: "XML Media Types", M. Murata, S. St.Laurent, D. Kohn, January 2001.

[84]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification", D. Mills, June 2010.

[85]	OMA OMNA Registered PUSH Application ID list

http://www.openmobilealliance.org/tech/omna/omna-push-app-id.htm

[86]	3GPP TR 26.936: "Performance characterization of 3GPP audio codecs".

[87]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[88]	IETF RFC 2326: "Real Time Streaming Protocol (RTSP)", Schulzrinne H., Rao A. and Lanphier R., April 1998.

[89]	Void.

[90]	"Service Guide for Mobile Broadcast Services", Open Mobile Alliance, OMA-TS-BCAST_ServiceGuide-V1_1, Candidate Version 1.1 –14 Sep 2010.

[91]	IETF RFC 5053 (October 2007): "Raptor Forward Error Correction Scheme for Object Delivery", M. Luby, A. Shokrollahi, M. Watson, T. Stockhammer.

[92]	IETF RFC 5285: "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni, July 2008.

[93]	IETF RFC 4396: "RTP Payload Format for 3rd Generation Partnership Project (3GPP) Timed Text", Rey J. and Matsui Y., February 2006.

[94]	OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Approved Version 1.2".

[95]	3GPP TS 26.430: "Timed Graphics".

[96]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[97]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[98]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[99]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[100]	IETF RFC 6064: "SDP and RTSP Extensions Defined for 3GPP Packet-Switched Streaming Service and Multimedia Broadcast/Multicast Service", M. Westerlund, P. Frojdh, January 2011.

[101]	Void.

[102]	Void.

[103]	Void.

[104]	3GPP TS 36.443: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M2 Application Protocol (M2AP)".

[105]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[106]	IETF RFC6681, "Raptor FEC Schemes for FECFRAME," M. Watson, T. Stockhammer and M. Luby, August 2012.

[107]	IETF RFC6363, "Forward Error Correction (FEC) Framework," M. Watson, A. Begen and V. Roca, October 2011.

[108]	3GPP TS 36.304: "User Equipment (UE) procedures in idle mode".

[109]	"Mobile Location Protocol (MLP) ", Open Mobile Alliance, OMA-LIF-MLP-V3_1, Approved Version 3.1 – 20 Sep 2011.

[110]	3GPP TR 26.946: "Multimedia Broadcast/Multicast Service (MBMS); User service guidelines".

[111]	Void.

[112]	ITU-T Recommendation H.265 (04/2013): "High efficiency video coding".

[113]	IETF Internet-Draft: "RTP Payload Format for High Efficiency Video Coding", draft-ietf-payload-rtp-h265-01.txt ,Wang Y.-K. et al, September 2013.

[114]	IETF RFC 1035 (November 1987): "Domain Names – Implementation and Specification", P. Mockapetris.

[115]	3GPP TR 26.906 Evaluation of HEVC for 3GPP Services (Release 12).

[116]	ISO/IEC 23009-1:2014: Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats.

[117]	3GPP TS 29.214 "Policy and charging control over Rx reference point".

[118]	3GPP TS 23.271 "Functional stage 2 description of Location Services (LCS)".

[119]	IETF RFC 5651: "Layered Coding Transport (LCT) Building Block", M. Luby, M. Watson, L. Vicisano.

[120]	3GPP TS 23.468, "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[121]	3GPP TS 29.468, "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3”.

[122]	3GPP TS 23.179. "Functional architecture and information flows for mission critical communication, services; Stage 2".

[123]	3GPP TS 24.379, "Mission Critical Push to Talk (MCPTT) call control Protocol specification".

[124]	3GPP TS 26.307, "Presentation Layer for 3GPP Services".

[125]	3GPP TS 26.116, "Television (TV) over 3GPP Services; Video Profiles".

[126]		Standard ECMA-262, 5.1 Edition / June 2011, ECMAScript Language Specification

Second Change

### 12.2.2	MooD Configuration Management Object

OMA-DM should be used to specify the MooD configuration information. If such a DM configuration object exists on the UE, the UE shall use it whenever it elects to support MBMS offloading. The OMA DM management object is used to configure offloading for any type of eligible content accessed over the unicast network via HTTP or RTP.

The Management Object Identifier shall be set to: urn:oma:mo:ext-3gpp-mbmsmood:1.0. The MO is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2 [94].

Note: the MO information may be translated into a Proxy Auto-Config (PAC) file that can be used by the UE to automatically pick the proxy server for the eligible content.

Figure 1 depicts the nodes and leaf objects contained under the 3GPP_MBMS MooD MO, if an MBMS client supports the feature described in this clause (information on the DDF for this MO is given below in Annex X):

<X>*

Enabled

ProxyServer

USD

LocationType?

Ext?

<X>+

ContentRestriction+

ContentRestrictionExt+

Address+

URL?

Ext?

Ext?

UrlPattern+

<X>+

Figure 1 - 3GPP MooD MO

Node: /<X>

This interior node specifies the unique object id of a MBMS MooD management object. The purpose of this interior node is to group together the parameters of a single object.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

The following interior nodes shall be contained if the UE supports the “MBMS MooD Management Object”.

/<X>/Enabled

This leaf indicates if MooD is supported by the BM-SC.

-	Occurrence: One

-	Format: bool

-	Minimum Access Types: Get

/<X>/ProxyServer

This node represents the one or more Proxy Servers that the UE shall use for all its unicast requests to resources that it elects to potentially receive over MBMS.

-	Occurrence: One

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

/<X>/ProxyServer/<X>

This interior node acts as a placeholder for one or more instances of ProxyServer information as addresses associated with content restriction identifiers for proxy server selection.  Should more than one proxy server satisfy the conditions of the content restriction, the UE may randomly select one of them.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

/<X>/ProxyServer/<X>/Address

This leaf indicates the one or more address of a ProxyServer in the form of a Fully-Qualified Domain Name (FQDN), and is associated with a set of content restrictions of which at least one must be satisfied in order for a UE to use that/those Proxy Server(s) for all its unicast requests to resources that it elects to potentially receive over MBMS.

-	Occurrence: OneOrMore

-	Format: chr

-	Access Types: Get, Replace

-	Values: FQDN (one or more)

/<X>/ ProxyServer/<X>/ContentRestriction

The ContentRestriction leaf contains one or more domain names for matching against the HTTP(s) or RTSP URL of the resource request issued by the UE to determine whether the requested content is eligible for conversion from unicast access to an MBMS User Service, and if so, the corresponding Proxy Server to use. A match between this value and the requested resource URL indicates that the requested resource may be switched to MBMS delivery, and the associated proxy server shall be used by the UE for unicast access of that resource.

-	Occurrence: OneOrMore

-	Format: chr

-	Access Types: Get, Replace

-	Values: concatenation of URI scheme as defined in RFC 3986 [3] with a domain name as defined in RFC 1035 [114]

/<X>/ ProxyServer/<X>/ContentRestrictionExt/<X>

The ContentRestrictionExt node allows for more refined definition of the MooD eligible content and the rules on when to use the proxy server to query for availability of content over MBMS. If both ContentRestriction and ContentRestrictionExt exist, then the UE shall apply both of them to determine if a particular resource URL is MooD eligible and needs to be sent through the MooD proxy. In particular, if a ContentRestriction element exists and none of the provided FQDNs match, then the UE shall assume that the content is not eligible.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

/<X>/ ProxyServer/<X>/ContentRestrictionExt/<X>/UrlPattern

The UrlPattern leaf provides a regular expression as defined by [126] that the request URL will be matched against. If the request matches, the requested resource is marked as MooD eligible.

-	Occurrence: OneOrMore

-	Format: chr

-	Access Types: Get, Replace

-	Values: a regular expression as defined in [126].

/<X>/ProxyServer/<X>/Ext

The Ext node is an interior node where vendor-specific information can be placed (vendor meaning application vendor, device vendor, etc.), pertaining to UE selection of the proxy server. Usually the vendor extension is identified by a vendor-specific name under the Ext node. The tree structure under the identified vendor is not defined and can therefore include one or more non-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get, Replace

/<X>/USD

The USD node is the starting point of the MBMS User Service Discovery/Announcement information definitions.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get, Replace

/<X>/USD/URL

This leaf provides a URL to an aggregated service announcement document encapsulating all relevant metadata fragments for the demand-based MBMS user service, which the UE can fetch using the unicast channel.  It may also be used by the network when the network redirects the UE to switch MBMS reception.  Should a redirection message provide an alternative redirection link to service announcement information, it shall take precedence over the URL provided by the MO.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

-	Values: <HTTP(S) URL>

/<X> USD/Ext

The Ext node is an interior node where vendor-specific information can be placed ("vendor" may correspond to an application vendor, device vendor, etc.). Usually the vendor extension is identified by a vendor-specific name under the Ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/LocationType

This leaf provides a location type for UE to report in the unicast content request.  Exactly one of the following entries: one or more serving cell-ID(s) (cell-ID in the form of CGI (Cell Global Identification) or ECGI (E-UTRAN Cell Global Identification)) or MBMS SAI may be present.  CGI, ECGI and MBMS SAI are defined in 3GPP TS 23.003 [4]. The serving cell-ID(s) should correspond to all cells from which the UE receives the service, i.e., the Primary Cell or P-Cell and any Secondary Cell(s) or S-Cell(s), if Carrier Aggregation [96] is employed in the E-UTRAN. When present, the UE should send its location as part of the MooD header field together with the requests that it sends to a MooD proxy server. If the LocationType is set to MBMS SAI, the UE includes the SAIs present in SIB15 [97].

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

-	Values: Exactly one of the following location information types: CGI, ECGI, MBMS SAI, and whereby one or more entries of the specified type may be included.

/<X>/Ext

The Ext node is an interior node where vendor-specific information can be placed ("vendor" may correspond to an application vendor, device vendor, etc.). Usually the vendor extension is identified by a vendor-specific name under the Ext node. The tree structure under the identified vendor is not defined and can therefore include one or more non-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get