3GPP TSG-SA4 Meeting #89	S4-160627

Kista, Sweden, June 27 – July 1, 2016

Source:	Fraunhofer HHI

Title:	VR content creation and transport workflow

Document for:	Discussion and agreement

Agenda Item:	10.6

# Introduction

Two of the objectives of the Study Item FS_VR (Virtual Reality) as agreed in S4-160165 are as follows:

- Analysing the different technologies and equipment in place that provide a Virtual Reality experience.

- Analysing and identifying the media formats (including audio and video), interfaces and delivery procedures between client and server required to offer such an experience.

This document provides an overview of the existing capture systems and proposes a delivery method “tile-based streaming” to be studied as part of the FS_VR study item.

# Content creation and transport workflow

Draft report of the DVB CM Study Mission on Virtual Reality [Z] describes the steps for the creation of VR video content as in Figure 1.

Figure 1: VR video content creation [Z]

VR content may be represented in different formats, e.g. cylinders or spheres depending on the capabilities of the capture systems. There are numerous products that produce source content in 360-degrees cylindrical, such as 360Heros-H3Pro7HD, Immersive Media-Quattro or Totavision-Cylindrical Camera (more examples can be found in https://thefulldomeblog.com/category/360-video/#cylindrical-video ). Besides, many systems capture spherical videos covering the full 360x180°.

Capturing of such content is typically done by multiple cameras, frames of which are stitched in real time or during an offline post-processing phase. Multi-camera arrangement for capturing panoramic video requires focal points of all camera views to coincide at a common point so that stitching can easily be performed with minimum parallax effect. The following gives an overview of various capturing systems.

Star-like systems:

Many commercial solutions use star-like camera arrangements (Figure 2). In this configuration, focal points of all cameras are located on a circle. This approach works quite well in scenes with distant objects but leads to visible stitching artefacts in case of close objects due to the parallax effect.

Figure 2: Star-like configuration

Mirror-based systems:

A good approximation to the optimal parallax-free configuration (common focal point for all cameras) can be achieved by using special mirror-rigs. In this case, all cameras look upwards onto mirrors arranged to form an upside down pyramid with a regular convex polygon base depending on the number of cameras, e.g. 6 cameras as shown in Figure 3. With correct alignment and calibration, virtual images of all focal points are superimposed at a common point within the pyramid.

Figure 3: Mirror-based camera system (HHI OmniCam-360 [H]) presented in NAB 2016

Fraunhofer HHI’s OmniCam-360 is such a scalable system with the most recent model consisting of ten 36-degree mirror segments and HD cameras. Arrangement of the cameras allows parallax-free stitching from 1 meter to infinity. The covered Field of View (FoV) is about 60-degree with a resolution up to 10000x2000 pixels. Also, 3D-video content generation is possible by an extension of the system that uses two cameras per mirror segment, OmniCam-360-3D. This amounts to a total of 20 HD cameras for 360-degree 3D panoramic recordings.

Figure 4: A Sample OmniCam3D capture

Depth-aware systems (segmented stereo):

Depth-aware systems can be categorized into systems with minor overlap (left) and extreme overlap (right) of the camera FoV (Figure 5).

Figure 5: Depth-aware systems with minor overlap (left) and extreme overlap (middle). HHI OmniStar 3D (right) is a depth-aware camera system with extreme overlap.

Systems with minor overlap include Samsung Beyond with 6 integrated stereo cameras, Next VR containing 3 stereo rigs with wide-angle lenses, and VOKE VR with 5 integrated stereo cameras. Systems providing extreme overlap rely on fisheye or wide-angle lenses. Such systems include Nokia OZO, comprising 8 cameras with 196 degree fisheyes, Jaunt NEO comprising 24 cameras with wide-angle lenses, and OmniStar3D by Fraunhofer HHI comprising 24 cameras with 36 degree lenses.

Figure 6: A sample of OmniStar3D capture

Depth-enabled light field rendering:

Light field cameras (a.k.a plenoptic cameras) capture in addition to the light intensity also the direction of light rays. Novel views of a scene can be created by extracting appropriate 2D slices from the light field of a scene. This technique is called light field rendering. Light field rendering requires an unrealistically dense camera grid.  However, by using depth information, camera density can be increased (virtual light field cameras can be computed with the depth information). Resulting depth-enabled light field camera systems can be highly flexible and can provide parallax-free rendering with more freedom to move around the scene.

Figure 7: Increasing camera density by using depth information

Lytro Immerge is a prominent example of an end-to-end system for capturing light fields for creating VR content. Immerge is a five-ring globe that captures a “light-field volume”. Unlike most traditional camera rigs capturing 360-degree video, Immerge captures all light rays in a given volume. Subsequently, this data allows rendering many spatially shifted 360-degree videos, each from a given subset of the captured rays. Thus it enables the user to move her head within the boundaries of the captured volume.

Figure 8: Lytro Immerge light-field camera

Stitching and projections:

Stitching captured camera views parallax-free is much less complex for camera arrangements with a common focal-point, e.g., mirror-based systems. Therefore, they are much more suitable than others for live scenarios with high quality video.

Cylindrical content can be encoded as is. However, spherical content can be encoded with different mappings. A common mapping used is equirectangular projection (Figure 9, left). As shown in the figure at the top and bottom parts (poles) of the picture are stretched to provide a rectangular image.

Another common approach is cubic mapping supported for instance in frameworks such as OpenGL. This projection is shown in Figure 9 (right). The sphere is mapped onto the six faces of a cube using e.g. perspective or gnomonic projection and the cube faces are then aligned in two rows.

Figure 9: Equirectangular (left) and cubic projection (right)

Another approach is the cylindrical projection (Figure 10) presented in MPEG Application Formats ad-hoc group regarding Omnidirectional Media Format (OMAF) [T]. Here the planar video to be mapped to a cylinder has a rectangular region and two circular regions; thus top and bottom views are also considered and distortion at poles is reduced compared to the equirectangular mapping.

Figure 10: Cylindrical projection [T]

# Tile-based video streaming

As the data rate of the obtained panorama video is usually too high to be transmitted to an end device, the delivery must be done in a bandwidth-efficient manner. One option is to spatially split the panorama into a number of rectangular regions. These regions could be offered at varying resolution or quality and the client may adaptively request the regions containing the Field-of-View (FoV) of the user.

In many virtual reality applications, only a subregion of the whole content corresponding to the Region-of-Interest (RoI) is displayed to the user. Decoding and bandwidth requirements can be significantly reduced, if only the RoI is decoded and transmitted. This can be achieved by partitioning a video stream into a number of independently decodable video streams and delivering only a stream comprising the RoI to the user.

## HEVC tile sets

In the HEVC standard, such spatial segmentation of the picture plane can be achieved through tiles, which define rectangular region of CTUs by vertical and/or horizontal boundaries. Tile boundaries break in-picture parsing and prediction dependencies such that a tile can be processed independently (with the exception of in-loop filters which can optionally cross tile boundaries to prevent boundary artifacts). Specifically, entropy coding and reconstruction dependencies including intra prediction, motion vector prediction, and context selection are not allowed across a tile boundary. This allows separate tiles to be encoded on different processors with little inter-processor communication.

In order to extend the prediction independency of spatial segments even further, so-called motion-constrained tile sets (MCTS) are defined. In an MCTS, the inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set. In other words, MCTS allow an encoder to guarantee the independence of tiles over time, thereby allowing truly independent subregions within the video stream.

## Tile-based streaming with MPEG-DASH

Figure X1: Tile gathering to assemble the RoI

# The fourth edition of ISO/IEC 14496-15 includes mechanisms that allow extraction of a single HEVC conforming bitstream comprising an MCTS. Thus, a single decoder is required to decode any RoI. This is useful in computationally limited devices that use a single hardware-accelerated decoder and also to save battery power [Y].

The Spatial Representation Description (SRD) descriptor defined in the second amendment of MPEG-DASH standard part 1, 23009-1:2014 describes spatial relationships between parts of the video content. SRD features a set of parameters to describe the position of tiles with respect to a common reference space. Using SRD, a DASH client can select and retrieve different spatial parts of the video. Thus, a client can request only the segments corresponding to the RoI.

[W], which was discussed during the 115th MPEG meeting, shows examples for mapping tiles to AdaptationSet elements in the MPD and how an HEVC conforming bitstream using the mechanisms in ISO/IEC 14496-15 can be used and signaled within the MPD. The idea is to use an extra adaptation set to describe an “aggregation” of tiles (each offered as a separate AdaptationSet) corresponding to an MCTS. Using the same concept as in [W] we show how this approach can work in a straightforward manner with the currently existing attributes. This approach does not require any special processing by the client but it quickly increases the size of the MPD as the number of tiles and resolutions increase. An example MPD for a 8x3 tiling and a FoV of 2x2 is as follows:

<MPD … >

…

<Period >

… (Each of the tiles in a separate AdaptationSet)

<AdaptationSet maxWidth="640" maxHeight="640">

<SupplementalProperty schemeIdUri="urn:mpeg:dash:srd:2014" value="1,0,0,640,640"/>

<Representation id="1_1_q1"

mimeType="video/mp4"

codecs="hev1.1.6.L93.0"

bandwidth=”128000”>

<BaseURL>video-1_1_q1.mp4</BaseURL>

</Representation >

<Representation id="1_1_q2"

mimeType="video/mp4"

codecs="hev1.1.6.L93.0"

bandwidth=”256000”>

<BaseURL>video-1_1_q2.mp4</BaseURL>

</Representation >

</AdaptationSet>

<AdaptationSet maxWidth="640" maxHeight="640">

<SupplementalProperty schemeIdUri="urn:mpeg:dash:srd:2014" value="1,640,0,640,640"/>

<Representation id="1_2_q1"

mimeType="video/mp4"

codecs="hev1.1.6.L93.0"

bandwidth=”128000”>

<BaseURL>video-1_2_q1.mp4</BaseURL>

</Representation >

<Representation id="1_2_q2"

mimeType="video/mp4"

codecs="hev1.1.6.L93.0"

bandwidth=”256000”>

<BaseURL>video-1_2_q1.mp4</BaseURL>

</Representation >

</AdaptationSet>

… (For the rest of the tiles)

<AdaptationSet maxWidth="640" maxHeight="640">

<SupplementalProperty schemeIdUri="urn:mpeg:dash:srd:2014" value="1,4480,1280,640,640"/>

<Representation id="3_8_q1"

mimeType="video/mp4"

codecs="hev1.1.6.L93.0"

bandwidth=”128000”>

<BaseURL>video-3_8_q1.mp4</BaseURL>

</Representation >

<Representation id="3_8_q2"

mimeType="video/mp4"

codecs="hev1.1.6.L93.0"

bandwidth=”256000”>

<BaseURL>video-3_8_q2.mp4</BaseURL>

</Representation >

</AdaptationSet>

… (Aggregation of tiles)

<AdaptationSet  ...>

<SupplementalProperty schemeIdUri="urn:mpeg:dash:srd:2014" value="1,0,0,1280,1280"/>

<Representation id="Aggregation1_q1"

mimeType="video/mp4"

codecs="hev2.1.6.L123.0"

dependencyId="1_1_q1 1_2_q1 2_1_q1 2_2_q1"

bandwidth=”512000”>

<BaseURL>Aggregation1_q1.mp4</BaseURL>

</Representation >

<Representation id="Aggregation1_q2"

mimeType="video/mp4"

codecs="hev2.1.6.L123.0"

dependencyId="1_1_q2 1_2_q2 2_1_q2 2_2_q2"

bandwidth=”1024000”>

<BaseURL> Aggregation1_q2.mp4</BaseURL>

</Representation >

</AdaptationSet>

… (Further Aggregation of tiles)

<AdaptationSet  ...>

<SupplementalProperty schemeIdUri="urn:mpeg:dash:srd:2014" value="1,3840,640,1280,1280"/>

<Representation id="AggregationN_q1"

mimeType="video/mp4"

codecs="hev2.1.6.L123.0"

dependencyId="2_7_q1 2_8_q1 3_7_q1 3_8_q1"

bandwidth=”512000”>

<BaseURL>Aggregation1_q1.mp4</BaseURL>

</Representation >

<Representation id="AggregationN_q2"

mimeType="video/mp4"

codecs="hev2.1.6.L123.0"

dependencyId="2_7_q2 2_8_q2 3_7_q2 3_8_q2"

bandwidth=”1024000”>

<BaseURL>AggregationN_q2.mp4</BaseURL>

</Representation >

</AdaptationSet>

</Period>

</MPD>

# Proposal

It is proposed to include section 3 into the section 7 of TR 26.918 on Virtual Reality.

# References

[X] MPEG#114 (San Diego), m38147, ISO/IEC 14496-15: on extractor design for HEVC files,

Fraunhofer HHI

[Y] Sanchez, Y., Globisch, R., Schierl, T., & Wiegand, T. (2014, January). Low complexity cloud-video-mixing using HEVC. In Consumer Communications and Networking Conference (CCNC), 2014 IEEE 11th (pp. 213-218). IEEE.

[W] MPEG#115 (Geneva), m38648, Usage of HEVC Tile Tracks in MPEG-DASH, Telecom ParisTech

[T] Technologies under Consideration for Omnidirectional Media Application Format, ISO/IEC JTC1/SC29/WG11 N15946

[Z] DVB, CM-VR0013, “Report of the DVB CM Study Mission on Virtual Reality”, draft 11, June 2016.

[H] http://www.hhi.fraunhofer.de/departments/vision-imaging-technologies/products-technologies/capture/panoramic-uhd-video.html