Source:	Samsung Electronics Co., Ltd.

Title:	IMS-based FLUS Solution

Document for:	discussion and agreement

Agenda Item:	11.6

# Introduction

One realization of FLUS [1] relies on IMS and can be implemented as an extension of an MTSI [2] client. In particular, the FLUS sink can be a regular MTSI client or it can be an IMS Application Server (AS) that acts as an MTSI client. The latter will be denoted as FLUS IMS AS in this contribution.

IMS provides tools for the MTSI client to discover the S-CSCF that will serve it using the I-CSCF and DNS queries. When using FLUS, an IMS-based FLUS source needs to discover the FLUS IMS AS that will be able to satisfy the requirements of that particular session. The S-CSCF will route the initial INVITE to the FLUS Sink that is known to satisfy the requirements from the source. Further negotiation with the FLUS sink will then take place.

# FLUS Control in IMS-based Solution

## System Configuration

### Introduction

In an IMS-based FLUS system, the control plane FLUS-C enables the selection and configuration of a FLUS Sink for the reception of media data from a FLUS session.

In order to select the FLUS Sink for a particular FLUS session, the FLUS source may:

- already know the FLUS Sink, e.g. in the case of an end-to-end MTSI session with an MTSI client or through provisioning

- discover the FLUS Sink using DNS SRV and/or NAPTR RRs

- rely on the S-CSCF to select the appropriate FLUS Sink based on the included request headers

In the following, a description of each of these alternatives is provided.

### FLUS Management Object

The FLUS Managmenet Object shall haven a Management Object Identifier: “urn:oma:mo:ext-3gpp-flus:1.0”. The MO shall be compatible with OMA Device Management protocol specification version 1.2 and above as described in [OMA-ERELD_DM-VI].

The following nodes for FLUS configuration are defined:

Node: /<X>

This interior node specifies the unique object id of a FLUS management object. The purpose of this interior node is to group together the parameters of a single object.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

The following interior nodes shall be contained if the FLUS sink in the terminal supports the FLUS Management Object.

/<X>/Sink/<X>

This node is a collection of information about a FLUS Sink

-	Occurrence: OneOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Sink/<X>/SIPURI

This leaf node provides the SIP URI for the FLUS Sink that is described by the parent node.

-	Occurrence: One

-	Format: string

-	Minimum Access Types: Get

/<X>/Sink/<X>/Capabilities

This leaf node provides a URL to the XML document that describes the capabilities of the FLUS Sink. The document shall follow the syntax and semantics described by the data model.

-	Occurrence: ZeroOrOne

-	Format: string

-	Minimum Access Types: Get

### DNS Discovery of FLUS Sink

If discovery is performed using DNS, a DNS SRV RR request shall be sent to the DNS server using the pre-provisioned FQDN: flus.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org.

The response shall contain a list of one or more FLUS sinks that are recommended by the network. A TXT RR record should be included to provide a corresponding capability URL for each of the included FLUS Sinks in the SRV RR response.

### Network-selected FLUS Sink

The UE may be provisioned with a FLUS URI, e.g. sink@flus.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org, in which case, the selection of the FLUS Sink is performed transparently by the S-CSCF.

To assist the S-CSCF with the correct routing of the SIP INVITE request, additional information may be provided as part of the request URI.

The following tag is defined for this purpose: “flus-requirements” and the value shall be a URL that points to a document that describes the data model.

### Session Establishment

An IMS-based FLUS session is established in the same way as a regular MTSI session using a SIP INVITE message. The SDP shall include at least one sendonly media session. Furthermore, to identify that media session as a FLUS session, each FLUS media line shall contain an attribute “a=3gpp-flus” with the following ABNF sytax:

FLUS-attribute = "a=3gpp-flus:" stream-id SP "configuration=" config

stream-id = 1*5DIGIT; provides an identifier of this FLUS component

config = <base64-encoded value>; provides a configuration of the current FLUS component as a base64 encoded json representation of the data model as described in section X.

An example of the initial SIP INVITE with the SDP offer is provided here:

INVITE sip:flusB@flus.mnc12.mcc14.pub.3ggpnetwork.org;flus-requirements=http://flus.example.com/rig/12/datamodel.json SIP/2.0 Via: SIP/2.0/UDP here.com:5060 From: UE <sip:UE@example.com> To: FLUSSink <sip:flus@flus.mnc12.mcc14.pub.3ggpnetwork.org> Call-ID: 12345601@example.com CSeq: 1 INVITE Contact: <sip:UE@100.100.100.100> Content-Type: application/sdp Content-Length: 3457 m=video 48092 RTP/AVPF 104 b=AS:240 b=RS:0 b=RR:2500 a=rtpmap:104 H264/90000 a=fmtp:104 packetization-mode=0; profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== a=sendonly a=3gpp-flus:1 configuration= VGhhbmsgeW91IGZvciBkZWNvZGluZyB0aGlzLiBGTFVTIGlzIGF3ZXNvbWUu m=video 48094 RTP/AVPF 104 b=AS:240 b=RS:0 b=RR:2500 a=rtpmap:104 H264/90000 a=fmtp:104 packetization-mode=0; profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== a=sendonly a=3gpp-flus:2 configuration= VGhpcyBpcyB0aGUgc2Vjb25kIGNvbXBvbmVudCBvZiB0aGlzIEZMVVMgZXhhbXBsZS4gVGhhbmtzLg==

# Proposal

We propose to agree the content of section 2 to the TS as part of section 5.2.

# References

[1]	3GPP SP-170576, Work Item on Framework for Live Uplink Streaming