3GPP TSG SA4#97 Meeting	S4-180131

5-9 February 2018, Fukuoka, Japan

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | xxx | rev | x | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR TS 26.118: Video Operation Points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI, Nokia Corporation, Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRStream |  |  |  |  |  |  | Date: |  | 2018-29-01 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Definition of Video Operation Points to enable VR video delivery over 3GPP services. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Introduction

During SA4#94 the New Work Item “3GPP Virtual Reality Profiles for Streaming Media (VRStream)” in S4-170751 was agreed and afterwards approved in by SA plenary #77 in SP-170612.

The objective of this work item is to define the relevant media and protocol enablers for the set of VR Streaming use cases related to UE consumption of managed and third party VR content as documented in TR26.918. Specifically, the following topics are addressed:

- Define the necessary presentation and media profile(s) to address the objectives of the use cases on VR Streaming in TR 26.918, clauses 5.3 and 5.4, taking into account the recommended assumptions and conclusions in TR 26.918.

- Define the enablers for PSS-based download and streaming of a 3GPP VR Presentation

- Define the enablers for MBMS streaming and MBMS-based download delivery of a 3GPP VR Presentation

- Provide suitable optional extensions (such as metadata) to support an improved VR experience,

- Document content generation and content consumption guidelines, taking into account a reference client architecture that provides: (a) the signalling and processing steps for download delivery, (b) PSS delivery, (c) interfaces between the VR service platform, the VR application (e.g. sensors), and the VR rendering system (displays, GPU, loudspeakers).

There is a preference for solutions that enable harmonization with broader industry consensus, such as MPEG or VR-IF.

During the ad-hoc meeting in Santa Clara, it was considered necessary to define operation points for VR video delivery, taking the structure of TV Video Profiles in TS 26.116 as a basis. As a starting point, initial contributions on video operations by two companies were presented during the Video SWG telco on VRStream on 19 December.

In Section 2 of this document, some considerations for VR video operations are presented discussing the similarities and differences compared to TV Video Profiles. Aligned with the results of this discussion, in Section 3, a text proposal for VR video operations in TS 26.118 is presented.

# Considerations for video operation points

TS 26.116 defines a set of operations points for TV video profiles based on the following definitions:

- Operation Point: A collection of discrete combinations of different content formats including spatial and temporal resolutions, colour mapping, transfer functions, etc. and the encoding format.

- Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

- Bitstream: A media bitstream that conforms to a video encoding format and certain Operation Point.

Operation points are defined on a media codec level and specify a mixture of source and distribution formats.

Source formats define the original formats of the video as provided by the content provider. In TS 26.116, distribution formats are defined as subsampled versions of source formats which may be used in order to address specific device classes or bitrate constraints. Specifically, TS 26.116 defines the following video operation points in Table 4.1.

Table 4.1: TV over 3GPP services Video Profile Operation Points

| Operation Point name | Resolution format | Picture aspect ratio | Scan | Max. frame rate | Chroma format | Chroma sub-sampling | Bit depth | Colour space format |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 [3] |
| H.265/HEVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 [3] |
| H.264/AVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 8 | BT.709 [3]; BT.2020 [4] |
| H.265/HEVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 8; 10 | BT.709 [3]; BT.2020 [4] |
| H.265/HEVC UHD | 3840 × 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 [4] |

In the following, we analyse the parameters defined in Table 4.1 for TV video profiles and discuss which of these are feasible for VR video profiles and what kind of modifications are necessary in the definition of an Operation Point.

Resolution format:

In TS 26.116 for TV video profiles, spatial resolutions typically define operation points and the operation points are named after the main content format (720p, Full HD, UHD). The relevant distribution formats, are defined as the subsampled versions of the main content format.

However, for VR video, distribution formats can be created by combining two or more available source formats (or subsampled versions of them) into a single bitstream by using tools like Region-Wise Packing (RWP). As a result, a packed distribution format is obtained, which may contain several resolutions merged to one bitstream.

This implies that the spatial resolution of the obtained distribution format is not a relevant parameter for determining the presented display resolution to the user. What matters for the Quality-of-Experience experienced by the user is the effective resolution inside the viewport and thus the source format that has the maximum resolution determining this effective resolution. Therefore, the defining factor for a VR operation point should be the source format having the maximum resolution (which we term as “maximum input resolution”) and the decoder constraints of the selected codec profile and level.

A concrete example: Maximum luma picture size of HEVC Level 5.1 is 8,912,896 samples. If we want to achieve the effective resolution of 6K inside the viewport and still cover a sufficiently large region to allow the viewport to change its position without switching the bitstream, we have to cover a larger region than the client’s field of view (FoV). Typical FoV values for the HMDs lie around 90°  90°. Therefore, if we cover for example 130°x110° of 6k resolution video, we would consume 46.71% (2218  1877) of available samples of HEVC Level 5.1 and cover 22.06% of the entire 360° video with the maximum resolution of 6K. The remaining 53.29% of the available samples could be redistributed as the downsampled version(s) of the 6K video in order to cover the remaining 77.94% of the 360° video.

Therefore, we propose defining the operation points under consideration of the maximum input resolution as well as the decoder constraints reflecting the capabilities of the available devices in the market (AVC Progressive High Profile Level 5.1, HEVC Main-10 Profile Main Tier Level 5.1).

Picture aspect ratio:

Picture aspect ratio is an important parameter for distribution formats in TV profiles because the resolution and aspect ratio of the distribution format correspond to the resolution and aspect ratio of the video displayed on the TV screen. For example, for the “H.265/HEVC UHD operation point” in TS 26.116, spatial resolutions of the distribution format are defined as 3840  2160, 3200  1800, 2560  1400, and so on, with a constant aspect ratio of 16:9.

However, for VR video, aspect ratio is not a relevant parameter because the decoded pictures are not displayed as such but undergo a mapping to spherical domain (and possibly region-wise un-packing) first.

Therefore, we propose not defining a specific aspect ratio for VR operation points.

Scan:

Same for VR video: Progressive.

Max frame rate

TV video profiles define 30 fps as maximum frame rate for 720p operation points and 60 fps for Full HD and UHD operation points (H.264/AVC and H.265/HEVC).

It is generally acknowledged that VR video requires higher frame rates than traditional video due to sensitivity of the users against motion-to-photon latency. 60 fps or higher frame rates are considered appropriate for VR.

However, we believe that operation points should primarily address the decoder capabilities of widely available devices on the market while providing the best possible quality. Since H.264/AVC Progressive High Profile Level 5.1 and H.265/HEVC Main-10 Profile Main Tier Level 5.1 are two codec operation points widely employed by the market, we propose to address the constraints of these codec operation points.

Considering those constraints, we propose a maximum frame rate of 30 fps (@4K) for H.264/AVC operation points and 60 fps (@4K) for H.265/HEVC operation points.

Chroma format:

Same for VR video: Y’CbCr.

Chroma sub-sampling:

Same for VR video: 4:2:0.

Bit depth:

Same for VR video: 8 bits for H.264/AVC, 8 or 10 bits for H.265/HEVC.

Colour space format:

Same for VR video: ITU-R BT.709 [3] and ITU-R BT.2020 [4].

As mentioned before, VR video typically undergoes a projection from spherical to planar domain and correspondingly, an inverse projection from planar to spherical domain on the receiver side. Considering this, in addition to the above defined parameters, the following parameter needs to be defined for VR video:

Projection:

For VR video, projection methods may be used for conversion of a spherical image into a two-dimensional rectangular image. Various projection methods are possible. In order to harmonize with OMAF, we propose to adopt Equirectangular Projection (ERP) for all operation points and additionally Cubemap Projection (CMP) for some operation points.

Aligned with the considerations above, we propose the set of operation points in Table 2 for delivery of VR video over 3GPP services.

Table 2: VR Video Operation Points over 3GPP Services.

| Operation Point name | Max input resolution | Projection | Max. frame rate | Chroma format | Chroma sub-sampling | Bit depth | Colour space format | Transfer Characteristics |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC 4K | 4096x2048 | ERP | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] | Mono/Stereo |
| H.264/AVC 5K | 5120x2560 | ERP | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] | Mono/Stereo |
| H.265/HEVC 4K | 4096x2048 | ERP | 60 | Y'CbCr | 4:2:0 | 8, 10 | BT.709 [3] BT.2020 [4] | BT.709 [3], BT.2100 [11] PQ | Mono/Stereo |
| H.265/HEVC 6K | 6144x3072 | ERP, CMP | 60 | Y'CbCr | 4:2:0 | 8, 10 | BT.709 [3] BT.2020 [4] | BT.709 [3], BT.2100 [11] PQ | Mono/Stereo |

# Text proposal for TS 26.118

========================= CHANGES =========================

# 5	Video Enablers

# 5.1	Video Operation Points

<Provides video operation points, describing content formats following the parameter in clause 4.1.2>

## 5.1.1	General requirements on media profile Operation Points

The following requirements apply to VR media profile Operation Points:

-	Y'CbCr (non-constant luminance) as the Chroma Format shall be used.

- 	4:2:0 chroma sub-sampling shall be used.

-	Projection Format: Either the equirectangular projection (ERP) or cubemap projection (CMP) shall be used depending on the Operation Point.

- The following frame rates should be used depending on the Operation Point: 24; 25; 30; 50 and 60Hz. The following fractional frame rates may be used: 24/1.001, 30/1.001, 60/1.001 (Hz). Frame rates are not associated to any particular spatial resolution.

- The following colour space formats may be used depending on the Operation Point: ITU-R BT.709 [3] and ITU-R BT.2020 [4]. If no signal is provided for the colour space, BT.709 [3] should be assumed as default colour space. Receiving devices should support BT.2020 [4] signaling and provide an appropriate mapping of the signal to the supported colour space of the device. Colour spaces are not associated to any particular spatial resolution.

- The following transfer characteristics may be used depending on the Operation Point: BT.709 [3] and BT.2020 [4] non-constant luminance transfer characteristics or the electro-optical transfer function as defined in Recommendation ITU-R BT.2100 [11] for the Perceptual Quantization (PQ) system.

- The Random Access Point period shall be less than or equal to 5 seconds, should be less than or equal to 2 seconds and may be less than or equal to 1 second for some services using a specific Operation Point.

- Bit depth: Either 8 or 10 bits shall be used depending on the operation point.

- Table 5.1 provides an overview of the Operation Points defined in the present document.

Table 5.1: Proposed VR Video Operation Points

| Operation Point name | Max input resolution | Projection | Max. frame rate | Chroma format | Chroma sub-sampling | Bit depth | Colour space format | Transfer Characteristics |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC 4K | 4096x2048 | ERP | 30 | Y’CbCr | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] | Mono/Stereo |
| H.264/AVC 5K | 5120x2560 | ERP | 30 | Y’CbCr | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] | Mono/Stereo |
| H.265/HEVC 4K | 4096x2048 | ERP | 60 | Y’CbCr | 4:2:0 | 8, 10 | BT.709 [3] BT.2020 [4] | BT.709 [3], BT.2100 [11] PQ | Mono/Stereo |
| H.265/HEVC 6K | 6144x3072 | ERP, CMP | 60 | Y’CbCr | 4:2:0 | 8, 10 | BT.709 [3] BT.2020 [4] | BT.709 [3], BT.2100 [11] PQ | Mono/Stereo |

Operation Points are defined including the video codec format.

## 5.1.2	General video codec requirements

The following video codecs and associated Profiles and Levels should be used:

-	H.264/AVC Progressive High Profile Level 5.1

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1

The Table 5.2 presents the mapping of the operation points with the codec type, profile and level.

Table 5.2: Proposed Video codec parameters

| Operation Point name | Codec type, profile and level |
| --- | --- |
| H.264/AVC 4K | AVC/H.264 Progressive High Profile Level 5.1 |
| H.264/AVC 5K | AVC/H.264 Progressive High Profile Level 5.1 |
| H.265/HEVC 4K | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |
| H.265/HEVC 6K | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |

## 5.1.3	H.264/AVC Operation Points

### 5.1.3.1		Common requirements and recommendations

#### 5.1.3.1.1	General

The video Bitstream and Receiver shall conform to Recommendation ITU-T H.264 / ISO/IEC 14496-10 with the restrictions described in this clause. H.264/AVC Bitstreams and Receivers shall support some parts of the "Video Usability Information (VUI)" syntax elements as specified in Recommendation ITU-T H.264 / ISO/IEC 14496-10, annex E, whose values are defined in this clause.

#### 5.1.3.1.2	Random Access point

##### 5.1.3.1.2.1	Definition

An H.264/AVC random access point (RAP) is defined as an access unit in an H.264/AVC Bitstream at which a Receiver can begin decoding the video successfully. This access unit shall include an AU delimiter NAL unit, only one Sequence Parameter Set (that is active) including the VUI and the Picture Parameter Set (PPS) that is required for decoding the associated picture. The access unit shall contain an IDR picture or an I picture.

##### 5.1.3.1.2.2   	Random access point period

RAPs shall be present in the Bitstream at least once every 5 seconds. It is recommended that RAPs occur in the video Bitstream on average at least every 2 seconds. For some services, RAPs may occur with a period of less than or equal to 1 second. The time interval between successive RAPs is measured as the difference between their respective decoding time values.

#### 5.1.3.1.3	Sequence parameter set

The following restrictions apply to the active Sequence Parameter Set (SPS):

-	gaps_in_frame_num_value_allowed_flag value shall be set to 0.

-	The Video Usability Information shall be present in the active Sequence Parameter Set. The vui_parameter_present_flag shall be set to 1.

-	The source video format shall be progressive. frame_mbs_only_flag shall be set to 1 for every picture of the Bitstream.

#### 5.1.3.1.4	Video Usability information

The aspect ratio information shall be present, i.e.

-	Either the aspect_ratio_present_flag value shall be set to 0 or the aspect_ratio_present_flag value shall be set to 1 and aspect_ratio_idc should be equal to 0 (unspecified) or 1 (square).

NOTE:	Note that the sample aspect ratio parameters in VUI do not affect the remapping of samples into their corresponding spherical coordinates.

The colour parameter information shall be present, i.e.

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	The values of colour_primaries, transfer_characteristics and matrix_coefficients are defined in clause 4.4.2.4 for H.264/AVC 720p HD and in clause 4.4.3.4 for H.264/AVC Full HD Operation Points.

The timing information may be present.

-	If the timing information is present, i.e. the value of timing_info_present_flag is set to 1, then the values of num_units_in_tick and time_scale shall be set according to the frame rates allowed in clause 5.1.3.2 for H.264/AVC 4K and in clause 5.1.3.3 for H.264/AVC 5KOperation Points. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

NOTE:	In 3GPP PSS and MBMS User services, the Receiver observes the timing at the system level, and ignores the timing information in the video Bitstream.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value shall be set to 1.

There are no requirements on output timing conformance for H.264/AVC decoding. The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

### 5.1.3.2		H.264/AVC 4K Operation Point

#### 5.1.3.2.1	Introduction

The following restrictions apply for the H.265/AVC 4K Operation Point. The general requirements on media profile Operation Points in clause 5.1.2 shall apply. This Operation Point allows achieving a resolution of around 1Kx1K for the viewport either only for one view (e.g. the resolutions achieved by 4096x2048 ERP) if monoscopic or shared for both eyes if stereoscopic.

#### 5.1.3.2.2	Profile, tier and level

A Bitstream conforming to the H.265/AVC 4K Operation Point shall comply with the following restrictions:

-	The profile_idc shall be set to 100 indicating the High profile.

-	The constrain_set0_flag, constrain_set1_flag, constrain_set2_flag and constrain_set3_flag shall all be set to 0.

-	The value of level_idc shall not be greater than 51 (corresponding to the level 5.1) and should indicate the lowest level to which the Bitstream conforms.

#### 5.1.3.2.3	Maximum input resolution

The maximum resolution of the source format shall be the following:

-	4096 × 2048

The spatial resolution of the distribution format shall be conformant to the decoder requirements in Clause 5.1.3.2.2.

#### 5.1.3.2.4	Colour information

A Bitstream conforming to the H.264/AVC 4K Operation Point shall use Recommendation ITU-R BT.709 colorimetry.

The colour_primaries value, the transfer_characteristics value and the matrix_coefficients value in the Video Usability Information shall all be set to 1.

A Receiver conforming to the H.264/AVC 4K Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 colorimetry.

#### 5.1.3.2.5	Frame rates

A Bitstream conforming to the H.264/AVC 4K Operation Point shall have one of the following frame rates: 24; 25; 30; 50; 60; 24/1.001; 30/1.001; 60/1.001 Hz.

The frame rate may be indicated in the VUI by setting vui_time_scale and vui_num_units_in_tick.

#### 5.1.3.2.6	Projection and SEI messages

Bitstreams conforming to the H.265/AVC 4K Operation Point shall contain the equirectangular projection SEI message.

Region-wise Packing SEI message: TBD

Frame Packing SEI message TBD

#### 5.1.3.2.7	Receiver compatibility

Receivers conforming to the H.265/AVC 5K Operation Point shall support decoding and displaying H.264/AVC 4K mono and stereo Bitstreams.

### 5.1.3.3		H.264/AVC 5K Operation Point

#### 5.1.3.3.1	Introduction

The following restrictions apply for the H.264/AVC 5K Operation Point. The general requirements on media profile Operation Points in clause 5.1.2 shall apply. This Operation Point allows achieving a higher resolution than around 1Kx1K for the viewport either only for one view (e.g. the resolutions achieved by 5120x2560) if monoscopic or shared for both eyes if stereoscopic.

#### 5.1.3.3.2	Profile, tier and level

A Bitstream conforming to the H.265/AVC 4K Operation Point shall comply with the following restrictions:

-	The profile_idc shall be set to 100 indicating the High profile.

-	The constrain_set0_flag, constrain_set1_flag, constrain_set2_flag and constrain_set3_flag shall all be set to 0.

-	The value of level_idc shall not be greater than 51 (corresponding to the level 5.1) and should indicate the lowest level to which the Bitstream conforms.

#### 5.1.3.3.3	Maximum input resolution

The maximum resolution of the source format shall be the following:

-	5120 × 2560

The spatial resolution of the distribution format shall be conformant to the decoder requirements in Clause 5.1.3.3.2.

#### 5.1.3.3.4	Colour information

A Bitstream conforming to the H.264/AVC 5K Operation Point shall use Recommendation ITU-R BT.709 colorimetry.

The colour_primaries value, the transfer_characteristics value and the matrix_coefficients value in the Video Usability Information shall all be set to 1.

A Receiver conforming to the H.264/AVC 5K Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 colorimetry.

#### 5.1.3.3.5	Frame rates

A Bitstream conforming to the H.264/AVC 5K Operation Point shall have one of the following frame rates: 24; 25; 30; 50; 60; 24/1.001; 30/1.001; 60/1.001 Hz.

The frame rate may be indicated in the VUI by setting vui_time_scale and vui_num_units_in_tick.

#### 5.1.3.3.6	Projection and SEI messages

Bitstreams conforming to the H.264/AVC 5K Operation Point shall contain the equirectangular projection SEI message.

Region-wise Packing SEI message: TBD

Frame Packing SEI message TBD

#### 5.1.3.3.7	Receiver compatibility

Receivers conforming to the H.265/HEVC 4K Operation Point shall support decoding and displaying H.265/HEVC 4K mono and stereo Bitstreams and H.265/HEVC 5K mono and stereo Bitstreams.

## 5.1.4	H.265/HEVC Operation Points

### 5.1.4.1		Common requirements and recommendations

#### 5.1.4.1.1	General

The video Bitstream and Receiver shall conform to Recommendation ITU-T H.265 / ISO/IEC 23008-2 with the restrictions described in this clause. H.265/HEVC Bitstreams and Receivers shall support some parts of the "Video Usability Information (VUI)" syntax elements as specified in Recommendation ITU-T H.265 / ISO/IEC 23008-2, annex E, whose values are defined in this clause.

#### 5.1.4.1.2	Random Access point

##### 5.1.4.1.2.1      Definition

An H.265/HEVC random access point (RAP) is defined as an access unit in an H.265/HEVC Bitstream at which a Receiver can begin decoding the video bitstream successfully. This access unit shall include an AU delimiter NAL unit, only one Video Parameter Set (that is active), only one Sequence Parameter Set (that is active) including the VUI and the Picture Parameter Set that is required for decoding the associated picture. The access unit shall contain an IRAP picture or an I picture, as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2.

##### 5.1.4.1.2.1      Random access point period

RAPs shall be present in the Bitstream at least once every 5 seconds. It is recommended that RAPs occur in the video Bitstream on average at least every 2 seconds. For some services, RAPs may occur with a period of less than or equal to 1 second. The time interval between successive RAPs is measured as the difference between their respective decoding time values.

#### 5.1.4.1.3	Video parameter set

H.265/HEVC Receivers should ignore the content of all Video Parameter Sets (VPS) NAL units as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2.

#### 5.1.4.1.4	Sequence parameter set

The following restrictions apply to the active Sequence Parameter Set (SPS):

-	The Video Usability Information (VUI) shall be present in the active Sequence Parameter Set. The vui_parameters_present_flag shall be set to 1.

-	The chroma sub-sampling shall be 4:2:0, chroma_format_idc value shall be set to 1.

-	The source video format shall be progressive, i.e.

-	The general_progressive_source_flag shall be set to 1,

-	The general_interlaced_source_flag shall be set to 0,

-	The general_frame_only_constraint_flag shall be set to 1.

Receivers conforming to any of the H.265/HEVC Operation Points shall only support Bitstreams with the restrictions on the SPS defined above.

#### 5.1.4.1.5	Video Usability information

The aspect ratio information shall be present, i.e.

-	Either the aspect_ratio_present_flag value shall be set to 0 or the aspect_ratio_present_flag value shall be set to 1 and aspect_ratio_idc should be equal to 0 (unspecified) or 1 (square).

NOTE:	Note that the sample aspect ratio parameters in VUI do not affect the remapping of samples into their corresponding spherical coordinates.

The colour parameter information shall be present, i.e.

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	The values of colour_primaries, transfer_characteristics and matrix_coeffs are defined in clause 5.1.4.2 for H.265/HEVC 4K, in clause 5.1.4.3 for H.265/HEVC 6K Operation Points.

The timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed in clause 5.1.4.2 for H.265/HEVC 4K, in clause 5.1.4.3 for H.265/HEVC 6K Operation Points. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

NOTE:	In 3GPP PSS and MBMS User services, the Receiver observes the timing at the system level, and ignores the timing information in the video Bitstream.

There are no requirements on output timing conformance for H.265/HEVC decoding. The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

### 5.1.4.2	H.265/HEVC 4K Operation Point

#### 5.1.4.2.1	Introduction

The following restrictions apply for the H.265/HEVC 4K Operation Point. The general requirements on media profile Operation Points in clause 5.1.2 shall apply. This Operation Point allows achieving a resolution of around 1Kx1K for the viewport either only for one view (e.g. the resolutions achieved by 4096x2048 ERP) if monoscopic or shared for both eyes if stereoscopic.

#### 5.1.4.2.2	Profile, tier and level

A Bitstream conforming to the H.265/HEVC 4K Operation Point shall comply with the following restrictions:

- The general_profile_idc shall be set to 2 indicating the Main-10 profile.

- The general_tier_flag shall be set to 0 indicating the Main tier.

-	   The value of level_idc shall not be greater than 153 (corresponding to the Level 5.1).

#### 5.1.4.2.3	Maximum input resolution

The maximum resolution of the source format shall be the following:

-	4096 × 2048

The spatial resolution of the distribution format shall be conformant to the decoder requirements in Clause 5.1.4.2.

#### 5.1.4.2.4	Colour information

A Bitstream conforming to the H.265/HEVC 4K Operation Point shall use Recommendation ITU-R BT.709 colorimetry.

The colour_primaries value, the transfer_characteristics value and the matrix_coefficients value in the Video Usability Information shall all be set to 1.

A Receiver conforming to the H.265/HEVC 4K Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 colorimetry.

#### 5.1.4.2.5	Frame rates

A Bitstream conforming to the H.265/HEVC 4K Operation Point shall have one of the following frame rates: 24; 25; 30; 50; 60; 24/1.001; 30/1.001; 60/1.001 Hz.

The frame rate may be indicated in the VUI by setting vui_time_scale and vui_num_units_in_tick.

#### 5.1.4.2.6	Projection and SEI messages

Bitstreams conforming to the H.265/HEVC 4K Operation Point shall contain the equirectangular projection SEI message.

When present, a frame packing arrangement SEI message shall be constrained in either of the following ways:

- frame_packing_arrangement_cancel_flag shall be equal to 1.

- All of the following constraints apply:

  - frame_packing_arrangement_type shall be equal to 3, 4, or 5.

  - quincunx_sampling_flag shall be equal to 0.

  - content_interpretation_type shall be equal to 1.

When the video does not cover the entire sphere, for each picture, there shall be a region-wise packing SEI message present in the bitstream that applies to the picture.

When present, the region-wise packing SEI messages shall indicate constraints that comply with the equirectangular projected video scheme type 'erpv' specified in clause 7.4.1.3 [1]

#### 5.1.4.2.7	Receiver compatibility

Receivers conforming to the H.265/HEVC 4K Operation Point shall support decoding and displaying H.265/HEVC 4K mono and stereo Bitstreams.

### 5.1.4.3	H.265/HEVC 6K Operation Point

#### 5.1.4.3.1	Introduction

The following restrictions apply for the H.265/HEVC 6K Operation Point. The general requirements on media profile Operation Points in clause 5.1.2 shall apply. This Operation Point allows achieving a higher resolution than 1Kx1K for the viewport either only for one view (e.g. the resolutions achieved by 6144x3072 ERP) if monoscopic or shared for both eyes if stereoscopic.

#### 5.1.4.3.2	Profile, tier and level

A Bitstream conforming to the H.265/HEVC 6K Operation Point shall comply with the following restrictions:

-	The general_profile_idc shall be set to 2 indicating the Main-10 profile.

-	The general_tier_flag shall be set to 0 indicating the Main tier.

-	The value of level_idc shall not be greater than 153 (corresponding to the Level 5.1).

#### 5.1.4.3.3	Maximum input resolution

The maximum resolution of the source format shall be the following if ERP is used:

-	    6144 x 3072

The maximum resolution of the source format shall be the following if CMP is used:

- 4608 x 3072

The spatial resolution of the distribution format shall be conformant to the decoder requirements in Clause 5.1.4.2.

#### 5.1.3.3.4	Colour information

A Bitstream conforming to the H.265/HEVC 6K Operation Point shall use Recommendation ITU-R BT.709 colorimetry.

The colour_primaries value, the transfer_characteristics value and the matrix_coefficients value in the Video Usability Information shall all be set to 1.

A Receiver conforming to the H.265/HEVC 6K Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 colorimetry.

#### 5.1.3.3.5	Frame rates

A Bitstream conforming to the H.265/HEVC 6K Operation Point shall have one of the following frame rates: 24; 25; 30; 50; 60; 24/1.001; 30/1.001; 60/1.001 Hz.

The frame rate may be indicated in the VUI by setting vui_time_scale and vui_num_units_in_tick.

#### 5.1.3.3.6	Projection and SEI messages

Bitstreams conforming to the H.265/HEVC 6K Operation Point shall contain either the equirectangular projection SEI message or the cubemap projection SEI message.

When present, a frame packing arrangement SEI message shall be constrained in either of the following ways:

- frame_packing_arrangement_cancel_flag shall be equal to 1.

- All of the following constraints apply:

  - frame_packing_arrangement_type shall be equal to 3, 4, or 5.

  - quincunx_sampling_flag shall be equal to 0.

  - content_interpretation_type shall be equal to 1.

When the video does not cover the entire sphere, for each picture, there shall be a region-wise packing SEI message present in the bitstream that applies to the picture.

When present, the region-wise packing SEI messages shall indicate constraints that comply with the equirectangular projected video scheme type 'erpv' specified in clause 7.4.1.3 [1] or the packed equirectangular or cubemap projected video scheme type 'ercm' specified in clause.

#### 5.1.3.3.7	Receiver compatibility

Receivers conforming to the H.265/HEVC 6K Operation Point shall support decoding and displaying H.265/HEVC 4K mono and stereo Bitstreams and H.265/HEVC 6K mono and stereo Bitstreams.

## 5.2	Video Rendering Schemes

<Provides an overview of video rendering schemes, post processing details such as ERP, CMP, packing, rotation>

## 5.3	Media Profiles

### 5.3.1	Viewport-Independent Video Media Profile

<provides a media profile for which the decoding is independent of the viewport>

### 5.3.2	Viewport-Dependent Video Media Profile

<provides a media profile for which the decoding is depends of the viewport>

## 5.4	ISO BMFF Integration

<provides file format integration of the different media profiles>

## 5.5	DASH Integration

<provides DASH integration of the different media profiles>

========================= END OF CHANGES =========================

# 4. Proposal

It is proposed to:

- Take into account the considerations on VR video operation points presented in Section 2.

- Adopt the proposed H.264/AVC and H.265/HEVC video operation points to TS 26.118.

- Modify time plan to move “Completion of Media Profiles” to the Kista meeting (April 2018).