# Multi-SDO Project on Converged Management PM Interface Definitions

# Document number:	S5eCPM0111

# Source:	Ericsson

# Title:		Specification for "mesurementName"

# Meeting date:	2014-10-06

# Objective

# This paper suggests a SDO specific standard for the measurementName property (discussed in [2]). There are two reasons for the suggestion.

- There are vast amounts of measurements today encoded using SDO specific name convention. The expiration dates for these measurements extend well into the future and decoding of these measurements names are required and needs to be maintained well into the future.

- The 3GPP/SA5 measurement naming scheme supports vendor-specific name convention, e.g. for UMTS and combined GSM/UMTS measurement names. This support is needed also well into the future.

# References

# Following is a list of references:

- S5eCPM0107, PM2: Metadefinitions for Measurements and KPIs V0.8

# Detailed Proposal

The following table shows two properties. The TRACKED CHANGES illustrates the paper's suggestion using corresponding text in [2] as base.

| measurementFamily | This property identifies the measurement family (e.g. HHO, RAB, SMS). Note that this family may also be used for measurement administration purpose. Note: when defining a new family name, consult the family names defined by other SDOs so that all the names are unique. It is not allowed to define a new family with the same semantics of an already existing family. | String |
| --- | --- | --- |
| measurementName | This property captures the measurement name. This name does not carry the measurement family name. The measurement name convention is SDO specific. | String |