# 3GPP TSG SA WG5 (Telecom Management) Meeting #103	S5-155082

# 12 - 16 October, 2015, Vancouver, Canada	revision of S5-153abc

# Source:	Intel

# Title:	Add LCM states

# Document for:	Approval

Agenda Item:	6.5.1.5

# 1	Decision/action requested

The group is asked to discuss and agree on the LCM state.

# 2	References

[1]	3GPP TS 28.525 “Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements “

# 3	Rationale

This document proposes the LCM states for TS 28.525 [1].

# 4	Detailed proposal

1st Modified Section

# 4	Concepts and background

## 4.1 	Concepts

### 4.1.1 	VNF lifecycle management states

VNF Package on-boarding and VNF instantiation / scaling / termination are key functions of VNF lifecycle management. VNF Package on-boarding is closely related to the operation of VNF instantiation / scaling / termination. For example,

- VNF can be instantiated only after the VNF package has been on-boarded, and enabled.

- A VNF package can only be deleted, if such VNF package is disabled, and is not in use by any outstanding VNF instance(s).

Figure 4.1.1-1 shows the VNF lifecycle management state transition diagram, describing the flow of VNF lifecycle management operation. It determines the valid VNF lifecycle management opearion in each state.

Figure 4.1.1-1: VNF lifecycle management state transition

Next Modified Section

## 5.4		High-level use cases

### 5.4.1 	FM use cases

### 5.4.2 	CM use cases

### 5.4.3 	PM use cases

#### 5.4.3.1 Use Case of VNF performance measurement monitoring

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable operators to monitor the performance of virtualized network functions in the mobile networks. |  |
| Actors and Roles | NM receives the performance measurments. EM administrates, collects, and reports the performance measurements. |  |
| Telecom resources | NM, EM |  |
| Assumptions | VNF(s) in the mobile networks are in operation. |  |
| Pre-conditions | A measurement job has been scheduled by EM for the periods for which VNF PM data collection is performed. |  |
| Begins when | VNF generates a VNF performance measurement. |  |
| Step 1 (M) | VNF sends the performance measurement to EM. |  |
| Step 2 (M) | NM sends the VNF performance measurement to NM. |  |
| Ends when (*) | NM receives the VNF performance measurement. |  |
| Exceptions |  |  |
| Post-conditions |  |  |
| Traceability (*) | REQ-MAMO-PM-CON-1, REQ-MAMO- PM-FUN -2 |  |

End of Modified Section