3GPP TSG-SA5 Meeting #124 	S5-192338

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192228

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0002 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the DN to URI mapping rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Th rules for mapping a DN to a URI and vice versa is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The rule for mapping a DN to a URI and vice versa is defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A DN cannot be mappded to aURI and vice versa. This mapping is essential for the network management to work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.1, 4.2.1a (new), 4.2.2, 4.2.3, 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CRUD	Create, Retrieve, Update, Delete

DC	Domain Component

DN	Distinguished Name

DNS	Domain Name Service

FQDN	Fully Qualified Doman Name

HTTP	Hypertext Transfer Protocol

JSON	JavaScript Object Notation

LDN	Local Distinguished Name

MnS	Management Service

REST	REpresentational State Transfer

RPC	Remote Procedure Call

TCP	Transmission Control Protocol

URI	Uniform Resource Identifier

End of 1st modified section

2nd modified section

### 4.2.1	Managed object naming

The Distinguished Name (DN) is used in 3GPP as global identifier for managed object instance. The DN is a comma (",") separated list  of Relative Distinguished Names (RDNs). Each managed object instance has an associated RDN. The sequence of RDNs is governed by name containment relationships in the UML class diagram describing the modelled network. The RDN consists of a naming attribute name and value. They are separated by an equal sign ("="). The naming attribute name is the class name of the manged object instance.

In addition to the RDNs associated to a managed object instance the DN has one or multiple RDNs on the left side whose naming attribute name is "DC" (Domain Component). The value of this special RDN is a Fully Qualified Domain Name (FQDN). It is allowed to split the FQDN into its components and use one RDN for each component.

DNs are independent of the used protocol technology and cannot be used for routing purposes.

The DN concept is described in detail in TS 32.300 [3]

The following is an example of a DN for a cell.

DN = "DC=operatorA.com,subNetwork=south,managedElement=a,eNBFunction=1,cell=1"

### 4.2.1a	Global and local namespaces

A DN is a global identifier and starts with the RDN of the global root. A name of a manged object instance may be defined also in a local name space starting with the RDN of the local root. The name in a local namespace is also referred to as Local Distinguished Name (LDN).

The local root can be either an instance of "ManagedElement" or an instance of "SubNetwork". These objects have a DN-prefix defined. The LDN for a "ManagedElement" instance, together with its DN-prefix, is the DN of the "ManagedElement" instance. The LDN for a "SubNetwork" instance, together with its DN-prefix, is the DN of the "SubNetwork" instance.

For example, if the local root is a "ManagedElement" instance, then the LDN of the cell instance is

LDN = "managedElement=a,eNBFunction=1,cell=1"

and the DN prefix of the "ManagedElement" is

DN-prefix = "DC=operatorA.com,subNetwork=south"

If the local root is a "SubNetwork" instance, then the LDN of the cell instance is

LDN = "subNetwork=south,managedElement=a,eNBFunction=1,cell=1"

and the DN prefix of the "SubNetwork" is

DN-prefix = "DC=operatorA.com"

### 4.2.2	Resource identification

HTTP uses a subset of the generic Uniform Resource Identifier (URI) scheme (RFC 3986 [4]) defined in RFC 7230 [5] for target resource identification.

http-URI = "http:" "//" authority path-abempty [ "?" query ] [ "#" fragment ]

The path component is an absolute path (one that starts with a single slash character) or empty.

The origin server is identified by the authority component, which includes a host identifier and an optional path TCP port. The hierarchical path component and optional query component serve as an identifier for a potential target resource within that origin server’s name space. The optional fragment component allows for indirect identification of a secondary resource.

The host identifier is either an IP address or an indirect identifier such as a FQDN to be resolved with DNS.

### 4.2.3	Mapping of DNs to URIs

DNs are used as the universal and protocol technology independent identifier for managed object instances. DNs have no meaning on HTTP protocol level, where resources representing managed object instances are identified using the URI.

A HTTP client wishing to interact with a resource on some HTTP server needs to know the URI of this resource. In many cases the DN of the managed object instance is known but not the URI of the corresponding resource. For this reason, it is beneficial if the DN can be mapped to an URI based on a standard rule thus avoiding the need to maintain mapping tables.

This mapping rule is as follows:

The DN prefix is mapped semantically to the authority component of the URI. The syntax of the DN prefix is modified to match the syntax of the authority component.

The LDN is mapped semantically to the path component of the URI. The syntax of the LDN is modified to match the syntax of the path component.

When mapping a LDN the equal sign "=" shall be used as delineator between the naming attribute name and naming attribute value when constructing a RDN.

URI-RDN = {namingAttributeName} "=" {namingAttributeValue}

The URI-LDN is the concatenation of URI-RDNs separated by a slash "/".

URI-LDN = *( "/" RDN )

For example, the LDN

LDN = "subNetwork=south,managedElement=a,eNBFunction=1,cell=1"

maps to

URI-LDN = "/subNetwork=south/managedElement=a/eNBFunction=1/cell=1"

and the LDN

LDN = "managedElement=a,eNBFunction=1,cell=1"

to

URI-LDN = "/managedElement=a/eNBFunction=1/cell=1"

When constructing the authority part from the DN prefix, it shall be reformatted according to the name conventions applying to FQDNs. For example, the DN prefix

DN-prefix = "DC=operatorA.com"

maps to

URI-DN-prefix = "operatorA.com"

and the DN prefix

DN-prefix = "DC=operatorA.com,subNetwork=south"

to

URI-DN-prefix = "south.subNetwork.operatorA.com"

The complete URIs for the examples are

http://operatorA.com/subNetwork=south/managedElement=a/eNBFunction=1/cell=1

http://south.subNetwork.operatorA.com/managedElement=a/eNBFunction=1/cell=1

The constructed URI-DN-prefix is a FQDN that can be registered into a name resolution service such as DNS. The sole presence of a constructed FQDN does not mean it can be resolved to an IP address and there is a server listening at that address.

This framework for contructing a URI from the DN assumes that the registered FQDN of the HTTP server supporting the resource identified by the URI is the domain name constructed from the DN prefix. If the registered domain name is different, then the authority component of constructed URI cannot be resolved into an IP address.

The entity mapping the DN to a URI needs to know which part of the DN shall be mapped into the URI, i.e. the DN prefix of the "ManagedElement" instance or the DN prefix of the "SubNetwork" instance.

The leftmost part of the path component may include one or more path segments ("label")

http://operatorA.com/{label}/subNetwork=south/.../cell=1

allowing to structure the resource hierarchy, for example

http://operatorA.com/3GPPmanagemen/ProvMnS/v1500/subNetwork=south/.../cell=1

The character set allowed in DNs is much bigger than the character set allowed in the path component and authority component of a URI. Care needs to be taken when selecting the naming attribute names und values that the mapping from a DN to a URI does not become impossible as a consequence of not mappable characters.

End of 2nd modified section

3rd modified section

## 4.4	URI structure

URIs identifying resources representing managed object instances shall follow a common structure given by

http://{FQDN}/{root}/{MnSName}/{MnSVersion}/{URI-LDN}

where:

{FQDN}	indicates the FQDN constructed from the DN prefix.

{root}	indicates an optional root for structuring the resource hierarchy.

{MnSName}	indicates the MnS name.

{MnSVersion}	indicates the version of the MnS.

{URI-LDN}	indicates the resource path constructed from the LDN.

End of 3rd modified section