3GPP TSG-SA5 Meeting #127	S5-196049

# Sophia-Antipolis (FR), 14 - 18 October 2019 									Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0056 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RESTful performance data file reporting management service notifications for integration with ONAP VES. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 3 for the integration of performance data file reporting management service notifications with ONAP VES is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Stage 3 enabling the integration of performance data file reporting management service notifications with ONAP VES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Integration of performance data file reporting management service notifications with ONAP VES would not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.X (New), A, A.3.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

## 12.3	Generic performance assurance management service

### 12.3.X	RESTful HTTP-based solution set for integration with ONAP VES API

#### 12.3.X.1	Mapping of operations

NOTE: this mapping is not part of the present document.

#### 12.3.X.2	Mapping of notifications

##### 12.3.X.2.1	Introduction

###### 12.3.X.2.1.1	General

The 3GPP IS notifications are mapped to SS equivalents according to table 12.3.X.2.1.1-1.

Table 12.3.X.2.1.1-1: Mapping of 3GPP IS notifications to SS equivalents

| 3GPP IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyFileReady | POST | /eventListener | M |
| notifyFilePreparationError | POST | /eventListener | M |

###### 12.3.X.2.1.2	Notification parameter mapping principles

3GPP performance assurance notification parameters are mapped either to:

- ONAP VES API commonEventHeader fields [21] – see Table 12.3.X.2.1.2-1, or to

- ONAP VES API perf3gpp fields – see mapping tables in the following clauses.

Table 12.3.X.2.1.2-1: Mapping of 3GPP IS notification parameters to ONAP VES common event header fields

| 3GPP IS notification parameter name | SQ | Corresponding ONAP VES common event header field name | SQ in ONAP VES common event header | Remark |
| --- | --- | --- | --- | --- |
| no corresponding IS parameter | N/A | sourceId | O |  |
| notificationId | M | eventId | M |  |
| no corresponding IS parameter | N/A | eventName | M | See [23]. |
| eventTime | M | lastEpochMicrosec | M | Type conversion needed [21]. |
| no corresponding IS parameter | N/A | reportingEntityName | M | See [24]. |
| eventTime | M | startEpochMicrosec | M | Same value as lastEpochMicrosec. Type conversion needed [21]. |
| no corresponding IS parameter | N/A | domain | M | “perf3GPP” |
| no corresponding IS parameter | N/A | priority | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the performance data file reporting MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | sequence | M | Field not used by this solution set. Actual value out of scope of this document. Value ‘0’ [21] to be provided by the performance data file reporting MnS producer. |
| no corresponding IS parameter | N/A | sourceName | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the performance data file reporting MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | version | M | Version of the event header [21]. |
| no corresponding IS parameter | N/A | vesEventListenerVersion | M | Version of the VES Event Listener API [21]. |

The ONAP VES API commonEventHeader datatype consists of fields common to all events, as specified in [21].

NOTE 1: Table 12.3.X.2.1.2-1 has the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 11.3.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 11.3.1 of the present document;

- Corresponding VES common event header field name: name of the field of the VES common event header [21];

- SQ in ONAP VES common event header: specified in [21], where ‘M’ corresponds to ‘Required = Yes’ in [21], and ‘O’ corresponds to ‘Required = No’ in [21];

NOTE 2: In Table 12.3.X.2.1.2-1, some VES common event header fields have no corresponding IS parameter. In case those VES common event header field Support Qualifier is ‘M’ (as specified in [21]), the producer of the notification shall provide a value to these parameters. See column ‘Remark’ in Table 12.3.X.2.1.2-1.

NOTE 3: ONAP VES common event header fields which are both optional [21] and have no corresponding IS parameter are not listed in Table 12.3.X.2.1.2-1.

NOTE 4: The mapping tables in following clauses of clause 12.3.X have the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 11.3.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 11.3.1 of the present document;

- SS parameter name (Corresponding VES ‘perf3GPP’ domain field): name of the field in ‘perf3gppFields’;

- SQ: Support Qualifier in ‘perf3gppFields’. When the IS notification parameter Support Qualifier is ‘M’, this Support Qualifier is set to ‘M’. When the IS notification parameter Support Qualifier is ‘C’, this Support Qualifier is set to ‘O’. When the IS notification parameter Support Qualifier is ‘O’, this Support Qualifier is set to ‘O’.

##### 12.3.X.2.2	Notification notifyFileReady

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.3.X.2.1.2-1 and to table 12.3.X.2.2-1.

Table 12.3.X.2.2-1: Mapping of IS notification parameters to SS equivalents in ‘perf3gppFields’

| 3GPP IS operation parameter name | SQ | SS parameter name in ‘perf3gppFields’ – clause A.3.X | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | M | dN | dN-Type | M |
| notificationType | M | notificationType (value = ‘notifyFileReady’) | notificationType-Type | M |
| fileInfoList | M | fileInfoList | array(fileInfo-Type) | M |
| additionalText | O | additionalText | additionalText-Type | O |

##### 12.3.X.2.3	Notification notifyFilePreparationError

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.3.X.2.1.2-1 and to table 12.3.X.2.3-1.

Table 12.3.X.2.3-1: Mapping of IS notification parameters to SS equivalents in ‘perf3gppFields’

| 3GPP IS operation parameter name | SQ | SS parameter name in ‘perf3gppFields’ – clause A.3.X | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | M | dN | dN-Type | M |
| notificationType | M | notificationType (value = ‘notifyFilePreparationError’) | notificationType-Type | M |
| fileInfoList | M | fileInfoList | array(fileInfo-Type) | M |
| reason | M | attributes | reason-Type | M |
| additionalText | O | additionalText | additionalText-Type | O |

#### 12.3.X.3	Resources

##### 12.3.X.3.1	Resource structure

Figure 12.3.X.3.1-1 shows the resource structure of the performance data file reporting MnS in the context of its integration with ONAP DCAE VES Collector R3 [22].

Figure 12.3.X.3.1-1: Resource URI structure of the performance data file reporting MnS for integration with ONAP DCAE VES R3 [22]

Table 12.3.X.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.3.X.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| eventListener | /eventListener | POST | Send notifications |

##### 12.3.X.3.2	Resource definitions

See [21].

#### 12.3.X.4	Data type definitions

See clause 12.3.1.1.4.

Next change

Annex A (normative): 
OpenAPI specification

## A.3	Generic performance assurance management service

### A.3.X	JSON schema of ‘perf3gppFields’ for integration with ONAP VES

{

"perf3gppFields": {

"description": "fields specific to 3GPP performance data reporting management service notifications for integration with ONAP VES",

"type": "object",

"properties": {

"perf3gppFieldsVersion": {

"description": "version of the perf3gppFields block",

"type": "string",

"enum": ["16.1"]

},

"provisioningMnSNotifications": {

"oneOf": [

{"$ref": "#/definitions/notifyFileReady-NotifType"},

{"$ref": "#/definitions/notifyFilePreparationError-NotifType"}

]

}

},

"definitions": {

"notifyFileReady-NotifType": {

"description": "fields specific to 3GPP notification NotifyFileReady",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"fileInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/fileInfo-Type"

}

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

},

"additionalProperties": false,

"required": [ "dN", "notificationType", "fileInfoList" ]

},

"notifyFilePreparationError-NotifType": {

"description": "fields specific to 3GPP notification NotifyFilePreparationError",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"fileInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/fileInfo-Type"

}

},

"reason": {

"$ref": "#/definitions/reason-Type"

}

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

},

"additionalProperties": false,

"required": [ "dN", "notificationType", "fileInfoList", "reason" ]

},

"dN-Type": {

"type": "string"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyFileReady",

"notifyFilePreparationError"

]

},

"dateTime-Type": {

"type": "string",

"format": "date-Time"

},

"uri-Type": {

"type": "string"

},

"long-Type": {

"type": "string",

"format": "long"

},

"fileInfo-Type": {

"type": "object",

"properties": {

"fileLocation": {

"$ref": "#/components/schemas/uri-Type"

},

"fileSize": {

"$ref": "#/components/schemas/long-Type"

},

"fileReadyTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"fileExpirationTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"fileCompression": {

"type": "string"

},

"fileFormat": {

"type": "string"

}

}

},

"reason-Type": {

"type": "string"

},

"additionalText-Type": {

"type": "string"

},

"notificationId-Type": {

"$ref": "#/components/schemas/long-Type"

}

}

}

End