3GPP TSG-SA5 Meeting #127	S5-196380

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0089 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of State Machine |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current Nchf relies on the state machine described by Diameter but since this is no longer relevant there is a need to have this described within the Nchf. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause for a description of the state machine implied by Nchf |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interpretation of when a message can or should be sent is left for interpretation and may cause interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.4.x	Session State Machine

For when there is a session it’s needed to for both the CTF and the CHF to agree on the state the other function is in to know when it is allowed to send what information, this is even more important when reporting for multiple Rating Groups during the same charging session.

The table below defines the different states which apply to a charging session for the CHF.

Table 5.4.x.1: Description of states in the CHF

| State: | Description: |
| --- | --- |
| Idle | No resource created and waiting for new requests |
| Created | Resource created and waiting for updates |

The table below defines the different states which apply to a charging session for the CTF.

Table 5.4.x.2: Description of states in the CTF

| State: | Description: |
| --- | --- |
| Idle | No trigger received and no session started |
| Pending | Session started and waiting for an answer to a request |
| Monitoring | Session started and monitored |

For the CHF the triggers are the reception of the requests from the CTF, but for the CTF

Table 5.4.x.3: Description of triggers in the CTF

| CTF Trigger: | Trigger Type | Description: |
| --- | --- | --- |
| Start of Multiple Service session | Immediate | There is new Multiple Service session detected in the CTF that needs charging. |
| Start of Rating Group | Immediate, Deferred | Within the Multiple Service session there is a service started that require a new Rating Group. |
| Update of Rating Group | Immediate, Deferred | The information for a monitored Rating Group is changed, including start and end of services that belongs to an ongoing Rating Group |
| Limit expiry for the Multiple Service session | Immediate | The limits set for reporting for the Multiple Service session |
| Limit expiry for a Rating Group | Immediate, Deferred | The limits set for reporting for the Rating Group |
| Quota management limit | Immediate | A limit for the granted quota has been reached, e.g. exhausted, validity time |
| Re-authorization | Immediate | The CHF has demanded a re-authorization of a Rating Group |
| End of Multiple Service session | Immediate | The Multiple Service session has ended |
| End of Rating Group | Immediate | There are no services using a specific Rating Group |
| Abort or termination request | Immediate | The CHF has requested that the Multiple Service session to be terminated |
| End of Multiple Service session | Immediate | The Multiple Service session has terminated |

#### 5.4.x.1	CTF State for Initial Convergent Charging

Table 5.4.x.3: Description of triggers in the CTF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Idle | Start of Multiple Service session | Send Charging Data Request [Initial] Start request timer | Pending |
| Pending | Received Charging Data Response, with service granted | Grant service Start monitoring of the service | Monitoring |
| Pending | Failure to send Charging Data Request, with service termination | Stop request timer Terminate the service | Idle |
| Pending | Failure to send Charging Data Request, with service continuation | Stop request timer Grant service Start monitoring of the service | Pending |
| Pending | Timeout of request timer, with service continuation | Stop request timer Grant service Start monitoring of the service | Pending |
| Pending | End of Multiple Service session | Queue Charging Data Request [Termination] | Pending |
| Pending | Start of Rating Group | Queue Charging Data Request [Update] | Pending |
| Pending | Update of Rating Group | Queue Charging Data Request [Update] | Pending |

End of changes