3GPP TSG-<SA5> Meeting # 127	S5-196426

# Sophia Antipolis, France,  14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0054 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add NRM fragment supporting the management of heartbeat notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add Stage 2 for the management of heartbeat notifications via the generic provisioning management service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new NRM fragment supporting the management of heartbeat notifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management of heartbeat notifications via the generic provisioning management service would not be possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.2, 4.3.x (New), 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.623 CR 0034 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document. See Annex A of a class diagram that combines this figure with Figure 1 of [2], the class diagram of UIM.

NOTE 1:	ManagedElement may be contained either

-	in a SubNetwork (since SubNetwork inherits from Domain_ and ManagedElement inherits from ManagedElement_ and Domain_ name-contained ManagedElement_ as observed in the figure of Annex A) or

-	in a MeContext instance as observed by the above figure or in the figure of Annex A.

This either-or relation cannot be shown by using an {xor} constraint in the above figure.

ManagedElement may also have no parent instance at all.

NOTE 2:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs.

NOTE 3:	If the configuration contains several instances of SubNetwork, exactly one SubNetwork instance shall directly or indirectly contain all the other SubNetwork instances.

NOTE 4:	The SubNetwork instance not contained in any other instance of SubNetwork is referred to as "the root SubNetwork instance".

NOTE 5:	ManagementNode shall be contained in the root SubNetwork instance.

NOTE 6:	If contained in a SubNetwork instance, IRPAgent shall be contained in the root SubNetwork instance.

NOTE 7:	For a clarification on the choice of containment of the IRPAgent (since it has three possible parents), see the def. of IRPAgent.

NOTE 8:    Cardinality * is identical to multiplicity 0..*.

# Figure 4.2.1-1: Generic NRM Containment/Naming and Association diagram

Each Managed Object is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of a ManagedElement instance could have a format like:

SubNetwork=Sweden,MeContext =MEC-Gbg-1, ManagedElement=RNC-Gbg-1.

NOTE 8:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs by virtue of inheritance from the GENERIC NRM.

Figure 4.2.1-2: VsDataContainer Containment/Naming and Association in GENERIC NRM diagram

The VsDataContainer is only used for the Bulk CM IRP.

Figure 4.2.1-3: Performance measurement control fragment

Figure 4.2.1-4: Measurement threshold monitoring fragment

Figure 4.2.1-x: Heartbeat notification management fragment

Next modification

### 4.2.2	Inheritance

# This clause depicts the inheritance relationships.

# The following figure shows the inheritance diagram.

# Figure 4.2.2-1: Generic Network Resource Model IRP Inheritance Hierarchy

# Figure 4.2.2-2: Performance measurement control fragment

# Figure 4.2.2-3: Measurement threshold monitoring fragment

# Figure 4.2.2-x: Heartbeat notification management fragment

Next modification

## 4.3	Class definitions

### 4.3.x	Heartbeat

#### 4.3.x.1	Definition

Hearbeat notifications are used to monitor the communication channel between a MnS Producer emitting notifications and the notification recipient. "Heartbeat" objects control the emission of heartbeat notifications. They are created by a MnS Consumer. In the "Heartbeat" creation request the MnS Consumer specifies the attributes as follows:

- The "heartbeatPeriod" attribute is set to the desired period between the emission of two heartbeat notifications.

- The "countDownTimer" attribute is set to the desired time until the emission of the first heartbeat notification after creation of the "Heartbeat" instance.

After creation the the "Heartbeat" object the MnS Producer counts down the "countDownTimer" attribute. When the timer reaches zero a heartbeat notification is emitted and the timer is set to the value of the "heartbeatPeriod" attribute, and counted down again.

During normal operation of heartbeat emission, the MnS Consumer can set to "countDownTimer" attribute to any value thereby extending or shortening the time until the next emission of a heartbeat notification as compared to the old value of the "countDownTimer" attribute. Setting the time to zero triggers an immediate emission of a heartbeat notification.

A MnS Consumer can suspend the emission of "notifyHeartbeat" notifications by setting the value of the "heartbeatPeriod" attribute to zero.

"notifyHeartbeat" notifications are sent to the recipient specified by the "notificationRecipientAddress" of the name containing "NotificationSubscription" oject.

A MnS Consumer needs to subscribe the notification recipient for receiving "notifyHeartbeat" notifications. The MnS Consumer managing the "Heartbeat" object and the notification recioient may or may not be the same entity.

When a MnS Consumer does not wish "notifyHeartbeat" notifications be emitted any more the MnS Consumer shall delete the "Heartbeat" object.

Support for sending heartbeat notifications and the capability of a MnS Producer to create and delete "Heatbeat" objects in response to MnS Consumer requests is advertised by the capability "3gpp-management-heartbeats". If the capability "3gpp-management-heartbeats" is not supported no "Heatbeat" objects can be created and no "notifyHeartbeat" notifications are emitted.

#### 4.3.x.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| heartbeatPeriod | M | T | T | F | F |
| countDownTimer | M | T | T | F | F |

#### 4.3.x.3	Attribute constraints

None.

#### 4.3.x.4	Notifications

This IOC emits the "notifyHeartbeat" notification.

The identifier of this notification is "3gpp-management-notifs-notifyHeartbeat".

The fields of this notification are defined in the following table.

| Parameter Name |  |  | SQ | Matching Information | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| objectClass |  |  | M | The string "Heartbeat" | Notification header |  |
| objectInstance |  |  | M | Heartbeat.objectInstance. | Notification header |  |
| eventTime |  |  | M | -- | Notification header |  |
| notificationId |  |  | O | -- | Notification header |  |
| systemDN | C | -- | Notification header |
| notificationType |  |  | M | The string "3gpp-management-notifs-notifyHeartbeat" | Notification header |  |
| heartbeatPeriod |  |  | M | Heartbeat.heartbeatPeriod |  |  |

Next modification

### 4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name |  | Documentation and Allowed Values |  |  | Properties |  |
| --- | --- | --- | --- | --- | --- | --- |
| farEndEntity |  | The value of this attribute shall be the Distinguished Name of the far end network entity to which the reference point is related. As an example, with EP_Iucs, if the instance of EP_Iucs is contained by one RncFunction instance, the farEndEntity is the Distinguished Name of the MscServerFunction instance to which this Iucs reference point is related. allowedValues: N/A |  |  | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| id |  | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| linkType |  | This attribute defines the type of the link. allowedValues: Signalling, Bearer, OAM&P, Other or multiple combinations of this type. |  |  | type: String multiplicity: 0..* isOrdered: F isUnique: T defaultValue: No default value isNullable: False |  |
| locationName |  | The physical location of this entity (e.g. an address). allowedValues: N/A |  |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| monitoringGP |  | This attribute defines the monitoring granularity period (in seconds). allowedValues: see NOTE 5 |  |  | type: Integer multiplicity: 1 isOrdered: F isUnique: T defaultValue: No default value isNullable: False |  |
| monitoredIOCName |  | It specifies one object class name for which the threshold monitor is created. When this attribute take effective, the threshold monitor is created for all of its instances in the name-containment tree whose top (tree) node is the IOC instance name-containing the subject <<IOC>>ThresholdMonitor instance containing this attribute. This attribute takes effective when the monitoredObjectDNs contained by the same <<IOC>>ThresholdMonitor instance is empty. allowedValues: The IOC names defined in the NRMs specifications. |  |  | type: String multiplicity: 1 isOrdered: F isUnique: T defaultValue: No default value isNullable: False |  |
| monitoringNotifTarget |  | This identifies the target of the notifications when the monitored measurement crosses or reaches the threshold set by the subject threshold monitor. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |
| monitoredObjectDNs |  | It specifies the object instance(s) for threshold monitoring. This attribute monitoredIOCName contained by the same <<IOC>>ThresholdMonitor instance does not take effective when this attribute is not empty. allowedValues: list of DN |  |  | type: dataType multiplicity: 1 isOrdered: F isUnique: T defaultValue: No default value isNullable: False |  |
| objectClass |  | An attribute which captures the name of the class from which the object instance is an occurrence of. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| objectInstance |  | An information which captures the Distinguished Name of any object. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| peeParametersList |  | This attribute contains the parameter list for the control and monitoring of power, energy and environmental parameters of ManagedFunction instance(s). This list contains the following parameters: - siteIdentification - siteLatitude (optional) - siteLongitude (optional) - siteDescription - equipmentType - environmentType - powerInterface siteIdentification: The identification of the site where the ManagedFunction resides. allowedValues: N/A siteLatitude: The latitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. This attribute is optional in case of BTSFunction and RNCFunction instance(s). allowedValues: -90.0000 to +90.0000 siteLongitude: The longitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. This attribute is optional in case of BTSFunction and RNCFunction instance(s). allowedValues: -180.0000 to +180.0000 siteDescription: An operator defined description of the site where the ManagedFunction instance resides. allowedValues: N/A equipmentType: The type of equipment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. environmentType: The type of environment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. powerInterface: The type of power. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. |  |  | type: String multiplicity: 0..* isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |  |
| priorityLabel |  | This is a label that consumer would assign a value on a concrete instance of the managed object. The management system takes the value of this attribute into account. The effect of this attribute value to the subject managed entity is not standardized | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| protocolVersion |  | Versions(s) and additional descriptive information for the protocol(s) used for the associated communication link. Syntax and semantic is not specified. allowedValues: N/A |  |  | type: String multiplicity: 0..* isOrdered: F isUnique: T defaultValue: no default value isNullable: False |  |
| setOfMcc |  | Set of Mobile Country Code (MCC). The MCC uniquely identifies the country of domicile of the mobile subscriber. MCC is part of the IMSI (TS 23.003 [5]) This list contains all the MCC values in subordinate object instances to this SubNetwork instance. allowedValues: See clause 2.3 of TS 23.003 [5] for MCC allocation principles. |  |  | type: Integer multiplicity: 1..* isOrdered: F isUnique: T defaultValue: No default value isNullable: False |  |
| swVersion |  | The software version of the ManagementNode or ManagedElement (this is used for determining which version of the vendor specific information is valid for the ManagementNode or ManagedElement). allowedValues: N/A |  |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| systemDN |  | The Distinguished Name (DN) of IRPAgent. Defined in 3GPP TS 32.300. allowedValues: N/A |  |  | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| supportedMonitoringGPs |  | The monitoring granularity periods (in seconds) supported by the producer for the monitored entities. AllowedValues: see NOTE 5. |  |  | type: integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| thresholdInfoList |  | It specifies the list of threshold information. It is list of < measurementType, direction, thresholdPack >, where thresholdPack is list of < thresholdLevel, thresholdValue, hysteresis (optional) > The measurementType shall be in one of the following form: - "family.measurementName.subcounter" for monitoring the measurement types with subcounters defined. - "family.measurementName" for monitoring the measurement types without subcounters defined. The direction can be 'Increasing' or ‘Decreasing' of the measurement values. - If it is "Increasing", the threshold crossing notification is triggered when the measurement value equals or exceeds a thresholdValue. - If it is "Decreasing", the threshold crossing notification is triggered when the measurement value equals or below a thresholdValue. The thresholdLevel indicates the level for the threshold. See NOTE 6 for the supported threshold levels. The hysteresis has a threshold-high and a threshold-low value that are different from the threshold value. A hysteresis, therefore, defines the threshold-high and threshold-low levels within which the measurementType value is allowed to oscillate without triggering the threshold crossing notification. AllowedValues: N/A |  |  | type: dataType multiplicity: 1s..* isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| userDefinedState |  | An operator defined state for operator specific usage. allowedValues: N/A |  |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| userLabel |  | A user-friendly (and user assignable) name of this object. allowedValues: N/A |  |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| vendorName |  | The name of the vendor. allowedValues: N/A |  |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| vnfParametersList |  | This attribute contains the parameter set of the VNF instance(s) corresponding to an NE. Each entry in the list contains: - vnfInstanceId - vnfdId (optional) - flavourId (optional) - autoScalable vnfInstanceId: VNF instance identifier (vnfInstanceId, see section 9.4.2 of [16] and section B2.4.2.1.2.3 of [17]). See Note 1. vnfdId: Identifier of the VNFD on which the VNF instance is based, see section 9.4.2 of [16]. This attribute is optional. Note: the value of this attribute is identical to that of the same attribute in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. flavourId: Identifier of the VNF Deployment Flavour applied to this VNF instance, see section 9.4.3 of [16]. This attribute is optional. Note: the value of this attribute is identical to that of the same attribute in clause 9.4.3 of ETSI GS NFV-IFA 008 [16]. autoScalable: Indicator of whether the auto-scaling of this VNF instance is enabled or disabled. The type is Boolean. See Note2. The presence of this attribute indicates that the ManagedFunction represented by the MOI is a virtualized function. See Note 3. allowedValues: N/A A string length of zero for vnfInstanceId means the VNF instance(s) corresponding to the MOI does not exist (e.g. has not been instantiated yet, has already been terminated). |  |  | type: String multiplicity: 0..* isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |  |
| vsData |  | Vendor specific attributes of the type vsDataType. The attribute definitions including constraints (value ranges, data types, etc.) are specified in a vendor specific data format file. allowedValues: -- |  |  | type: -- multiplicity: -- isOrdered: -- isUnique: -- defaultValue: -- isNullable: False |  |
| vsDataFormatVersion |  | Name of the data format file, including version. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| vsDataType |  | Type of vendor specific data contained by this instance, e.g. relation specific algorithm parameters, cell specific parameters for power control or re-selection or a timer. The type itself is also vendor specific. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| pMAdministrativeState |  | It indicates the administrative state of MeasurementControl. It describes the permission to use or prohibition against using the capability of MeasurementControl, imposed through the consumer of OAM services produced by MeasurementControl, The measurement report production would begin when pMadministrativeState is UNLOCKED and pMoperationalState is ENABLED. The meaning of these values is as defined in 3GPP TS 28.625 [c] and ITU-T X.731 [a]. allowedValues: "LOCKED", "SHUTTING DOWN", "UNLOCKED". |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “LOCKED” isNullable: False |  |
| pMOperationalState |  | It indicates the operational state of MeasurementControl. It describes if the resource is physically installed and working. allowedValues: “ENABLED”, “DISABLED”. The meaning of these values is as defined in 3GPP TS 28.625 [c] and ITU-T X.731 [a]. |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| fileLocation |  | It is the path to the file system where the produced measurement reports (containing PM data) are stored. There are two delivery methods (i.e. file-based and stream-based) to deliver Measurements to the consumer(s). This attribute is used for the file-based delivery method. The size of this fileLocation is decided by consumer and producer. The producer is expected to remove old files to make room for new files, when necessary. allowedValues: Not applicable. |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |
| managedObjectDNs |  | It identifies the managed functions whose Measurements are required to be produced. It identifies specific entities (say X, Y, Z) of the management containment tree. Consequently, it identifies all, including themselves (e.g. X, Y and Z), entities that are subordinates, in the sense of name-containment, of X, Y and Z. We called the identified entities a collection. The presence of this IOC would mean Measurement types specified in attribute MeasurementReader.measurementTypes, are required to be produced if the member (of the collection) is capable of supporting the Measurement types. allowedValues: N/A |  |  | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| managedObjectDNsBasic |  | It identifies the managed functions whose Measurements are required to be produced. It identifies specific entities (say X, Y, Z) of the management containment tree. The presence of this IOC would mean Measurements, of type specified in attribute MeasurementReader.measurementTypes, are required to be produced if X, Y, Z are capable of supporting the Measurement types. If managedObjectDNs of the same MeasurementReader instance has valid information, the information of this attribute is ignored. allowedValues: N/A |  |  | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| fileBasedGP |  | This defines the frequency of producing and delivering the measurement report. The measurement report would be produced immediately at the end of each GP. allowedValues: See Note 4. |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| streamBasedGP |  | It defines the frequency of producing and sending the Measurement to the consumer. allowedValues: See Note 4. |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| measurementTypes |  | It identifies one or more Measurement types. The Measurement type can be those specified in TS 28.552 [b], TS 32.404 [21} and can be those specified by other SDOs or can be vendor-specific. allowedValues: N/A |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| streamTarget |  | This identifies the target of the notification carrying the content of the measurement report. There are two delivery methods (i.e. file-based and stream-based) via which the consumer(s) can receive the Measurements. This attribute is used for the stream-based delivery method. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |
| measurementTypeAndGPs |  | It identifies the supported Measurement types and their supported GPs. This is a list of items. One item is composed of two parts: - One supported Measurement type; - One or more supported GPs (see Note 4). (A Measurement type cannot appear in more than one item.) For supported Measurement types, see “Documentation and Allowed Values” for measurementTypes. For supported GPs, see Note 4. allowedValues: N/A |  |  | type: TBD multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| nFServiceType |  | The parameter defines the type of the managed NF service instance allowedValues: See clause 7.2 of TS 23.501[22] |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: T defaultValue: No default value isNullable: False |  |
| operations |  | This parameter defines set of operations supported by the managed NF service instance. allowedValues: See TS 23.502[23] for supporting operations |  |  | type: Operation multiplicity: 1..* isOrdered: F isUnique: F defaultValue: No default value isNullable: False |  |
| Operation.name |  | This parameter defines the name of the operation of the managed NF service instance. allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: no default value isNullable: True |  |
| allowedNFTypes |  | This paramerter identifies the type of network functions allowed to access the operation of the managed NF service instance. allowedValues: See TS 23.501[22] for NF types |  |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| operationSemantics |  | This paramerter identifies the semantics type of the operation. See TS 23.502[23] allowedValues: “Request/Response”, “Subscribe/Notify”. |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| sAP |  | This parameter specifies the service access point of the managed NF service instance. allowedValues: N/A |  |  | type: SAP multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| host |  | This parameter specifies the host address of the managed NF service instance. It can be FQDN (See TS 23.003 [5]) or an IPv4 address (See RFC 791 [24]) or an IPv6 address (See RFC 2373 [25]). allowedValues: N/A |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |
| port |  | This parameter specifies the transport port of the managed NF service instance. allowedValues: 1 - 65535 |  |  | type: Integer multiplicity: 1 isOrdered: F isUnique: F defaultValue: No default value isNullable: False |  |
| ManagedNFService.operationalState |  | This parameter indicates the operational state of the managed NF service instance. It describes whether the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| ManagedNFService.administrativeState |  | This parameter indicates the administrative state of the manaed object instance. It describes the permission to use or prohibition against using the instance, imposed through the management services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| ManagedNFService.usageStae |  | This parameter indicates the usage state of the object instance. It describes whether the resource is actively in use at a specific instant, and if so, whether or not it has spare capacity for additional users at that instant. allowedValues: "IDLE", "ACTIVE", "BUSY". The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| registrationState |  | This parameter defines the registration status of the managed NF service instance. allowedValues: "Registered", "Deregistered". |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Deregistered isNullable: False |  |
| heartbeatPeriod |  | This attribute specifies the time between the emission of two "notifyHeartbeat" notifications. The time is specified in minutes. The value of zero has the special meaning of suspending the emission of heartbeat notifications. |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| countDownTimer |  | This attribute represents the value of a count down timer. Its value is counted down and when zero is reached a "notifyHeartbeat" notification is emitted and the value is reset to the value of the "heartbeatPeriod" attribute. |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| Note 1 : The value of this attribute is identical to that of the same attribute in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. Note 2: The value of this attribute is identical to that of the same attribute included in vnfConfigurableProperty in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. Note 3:The presence of the attribute vnfParametersList, whose vnfInstanceId with a string length of zero, in createMO operation can trigger the instantiation of the related VNF/VNFC instances. Note 4: The GP defines the measurement data production rate. The supported rates are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported GPs reflects the agreement between producer and the consumer involved. Note 5: The monitoring granularity period defines the measurements monitoring period. The supported monitoring periods are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported monitoring GPs reflect the agreement between producer and the consumer involved. Note 6: The supported threshold levels are dependent on the capacity of the producer involved (e.g. the processing power of the producer, number of measurements being measured by the producer at the time, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported levels can only reflect the negotiated agreement between producer and the consumer involved. |  |  |  |  |  |  |

End