3GPP TSG-SA5 Meeting #132e 	S5-204133

# e-meeting 17th 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0247 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the descriptions of UL packet delay between PSA UPF and UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UL packet delay transmitted to UPF from RAN is an important part of the UL packet delay between PSA UPF and UE. A liaison from SA2 to RAN3 confirms that the PDCP queuing delay should be considered in E2E UL packet delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the descriptions of UL packet delay between PSA UPF and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL packet delay between PSA UPF and UE is not correctly evaluated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.9.2.1, 5.4.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of changes

5.4.9.2.1	Average UL packet delay between PSA UPF and UE

a)	This measurement provides the average UL packet delay between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE:  The UPF may sample the GTP packets for QoS monitoring,  the specific sampling rate is up to implementation.

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4] and 38.415 [31]):

- 	T3 received in the GTP-U header of the monitoring response packet indicating the local time that the monitoring response packet was sent by the NG-RAN;

-	T4 that the monitoring response packet was received by the PSA UPF;

-	The UL Delay Result from UE to NG-RAN indicating the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU)  and the delay over Uu interface and PDCP queuing delay in UE (see 38.415 [31], and the UL Delay Result is denoted by DRul in the present document);

-	The S-NSSAI associated to the GTP PDU monitoring response packet.

The PSA UPF counts the number (N) of GTP PDU monitoring response packets for each S-NSSAI, and takes the following calculation for each S-NSSAI:

i=1N(T4i-T3i+DRuli)N

d)	Each measurement is a real representing the average delay in 0.1ms.

e)	GTP.DelayUlPsaUpfUeMean.SNSSAI, where SNSSAI identifies the S-NSSAI;

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

2nd of changes

5.4.9.2.2	Distribution of UL packet delay between PSA UPF and UE

a)	This measurement provides the distribution of UL packet delay between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE:  The UPF may sample the GTP packets for QoS monitoring,  the specific sampling rate is up to implementation.

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4] and 38.415 [31]):

- 	T3 received in the GTP-U header of the monitoring response packet indicating the local time that the monitoring response packet was sent by the NG-RAN;

-	T4 that the monitoring response packet was received by the PSA UPF;

-	The UL Delay Result from UE to NG-RAN indicating the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU)  and the delay over Uu interface and PDCP queuing delay in UE (see 38.415 [31], and the UL Delay Result is denoted by DRul in the present document);

-	The S-NSSAI associated to the GTP PDU monitoring response packet.

The PSA UPF 1) takes the following calculation for each GTP PDU monitoring response packet (packet i) for each S-NSSAI, and 2) increment the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounter per S-NSSAI.

T4i-T3i+ DRuli

d)	Each measurement is an integer representing the number of GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlPsaUpfUeDist.SNSSAI.bin, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

end of changes