3GPP TSG SA WG5 Meeting 134-e 	TDoc S5-206054rev1

electronic meeting, online, 16th - 25th November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.536 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify valid values for AssuranceTarget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | COSLA |  |  |  |  |  | Date: |  |  | 2020-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Draft CR S5-205398 does not define the valid values for AssuranceTarget. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add constraint for values of assuranceTargetName. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined values will lead to incompatible implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR proposes changes to the Draft CR in S5-205398 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.1.2.4	Attribute definitions

##### 4.1.2.4.1	Attribute properties

The following table defines the properties of attributes that are specified in the present document.

Table 4.1.2.4.1.1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| controlLoopLifeCyclePhase |  |  | It indicates the lifecycle phase of the AssuranceControlLoop instance. AllowedValues: Preparation, Commissioning, Operation and Decommissioning. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |
| assuranceTargetName |  |  | The name of the attribute which is part of a key-value-pair in the AssuranceGoal The assuranceTargetName shall be equal to the name of an attribute in the relevant ServiceProfile or SliceProfile. The relevant ServiceProfile or SliceProfile is identified by the attribute serviceProfileIdRef or sliceProfileIdRef in the AssuranceGoal. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |
| assuranceTargetValue | The value of the attribute which is part of a key-value-pair in the AssuranceGoal | type: Number multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| assuranceTargetList | This is an attribute containing a list of key-value-pairs that are part of an AssuranceGoal | type: Target multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| assuranceGoalId | A unique identifier of the assurance goal that should be supported by the AssuranceControlLoop. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| activeTime | It indicates the observation time expressed in number of timeUnits. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeUnit |  |  | It indicates the unit of time used to express the activeTime AllowedValues: second, minute, hour, day Editor’s note: the use of other values expressing units larger than days or smaller than seconds (i.e. ms) is FFS |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |
| sliceProfileIdRef |  |  | This holds a reference to a SliceProfile subject to assurance requirements relating to the NetworkSliceSubnet instance. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |
| serviceProfileIdRef |  |  |  | This holds a reference to a ServiceProfile subject to assurance requirements relating to the NetworkSliceSubnet instance. |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |
| assuranceGoalList | It is an attribute of an AssuranceControlLoop containing a list of AssuranceGoals. | type: AssuranceGoal multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the assurance control loop. It describes whether the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [x] and ITU-T X.731 [y]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the assurance control loop. It describes the permission to use or prohibition against using the instance, imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, The meaning of these values is as defined in 3GPP TS 28.625 [x] and ITU-T X.731 [y]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

End of changes