3GPP TSG SA WG5 Meeting 134-e		TDoc S5-205122rev6

# electronic meeting, online, 16th - 25th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0407 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add service based architecture for offline charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2020-11-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Offline only charging triggers for IMS service based charging is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Offline only charging triggers for IMS service based charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Offline only charging for IMS service based charging architecture options won’t be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.3	Applicable triggers for IMS charging

The IMS nodes for which SIP method a charging Data Request is sent shall be operator configurable. The tables below describe all possible Charging Data Requests that might be sent from IMS nodes.

The generation of a Charging Data Request message by the IMS nodes in response to a particular "triggering SIP method" shall be operator configurable (enable or disable).

Table 5.4.3.1: Default Trigger conditions in or MRFC

| Trigger Conditions |  | Trigger level | Converged Charging default category | Offline only charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SIP INVITE for initiating a multimedia ad hoc conferencing session |  | - | Immediate | Not Applicable | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] |
| SIP 2xx acknowledging an SIP INVITE for initiating a multimedia ad hoc conferencing session, and no charging session exists. |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP ACK acknowledging a SIP INVITE to connect an UE to the conferencing session |  | - | Immediate | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Update] |
| SIP RE-INVITE or SIP UPDATE[e.g. change in media components] |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP BYE(NOTE 1) |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Expiration of Interim |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Expiration of quota, Validity time expiry or other authorization triggers (quota threshold reached, …). |  | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP BYE message (both normal and abnormal session termination cases) (NOTE 2) |  | - | Immediate | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP CANCEL |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response with error codes 4xx, 5xx or 6xx indicating termination of an ongoing session |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | NOTE 1: This trigger only applies to a user leaving an ongoing conferencing session NOTE 2: This trigger only applies if this causes the ongoing conferencing session to terminate |  |  |  |  |  |  |

Table 5.4.3.2: Default Trigger conditions in or IMS-GWF or AS

| Trigger Conditions | Trigger level | Converged Charging default category | Offline only charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- | --- |
| SIP INVITE | - | Immediate | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] ECUR: Charging Data Request [Initial] |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or SIP UPDATE [e.g. change in media components] | - | Immediate | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Update] |
| RE-INVITE or SIP UPDATE [e.g. change in media components, terminating identity change] | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Expiration of quota, Validity time expiry or other authorization triggers (quota threshold reached, …). | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| Any SIP message (except those triggering a Debit / Reserve Units Request[Initial] or those not covered by the above triggers for Reserve Units Request[Update] conveying a SDP offer or its associated SDP answer before SIP session establishment | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP BYE message (both normal and abnormal session termination cases) | - | Immediate | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP 2xx acknowledging a SIP BYE message (only when last user location information of originating/ terminating party is required by operator for legal purpose). | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up procedure | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging non-session related SIP messages | - | Immediate | Not Applicable | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| Aborting a SIP session set-up procedure, using an internal trigger, or a SIP CANCEL | - | Immediate | Not Applicable | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] SCUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP Final/Redirection Response 3xx | - | Immediate | Not Applicable | Not Applicable | Not Applicable |  |
| SIP NOTIFY | - | Immediate | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] |
| SIP MESSAGE | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure | - | Immediate | Immediate | Not Applicable | Not Applicable |  |

End of changes