3GPP TSG-SA5 Meeting #134-e	S5-206195

# Online, , 16th Nov 2020 - 25th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0300 | rev | 1 | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of AMBR trigger usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2020-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When a trigger for session AMBR change is to be reported is not explicitly defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting that session AMBR change should be sent when the authorized session AMBR have been changed and is in force. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of session AMBR change is open for interpretation which may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-206195 |  |  |  |  |  |  |  |  |

First change

##### 6.2.5.3.5	Enumeration: TriggerType

Table 6.2.5.3.5-1: Enumeration TriggerType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FINAL | a service termination has happened |  |
| ABNORMAL_RELEASE | PDU session has abnormal released. |  |
| QOS_CHANGE | In request message, this value is used to indicate that QoS change has happened. Any of elements of QoSData may result in QoS change. |  |
| VOLUME_LIMIT | Volume limit has been reached. |  |
| TIME_LIMIT | Time limit has been reached |  |
| EVENT_LIMIT | Event limit has been reached |  |
| PLMN_CHANGE | PLMN has been changed. |  |
| USER_LOCATION_CHANGE | In request message, this value is used to indicate that User location has been changed. |  |
| RAT_CHANGE | In request message, this value is used to indicate that RAT type has been changed. |  |
| SESSION_AMBR_CHANGE | In request message, this value is used to indicate a change of the enforced authorized session AMBR. |  |
| UE_TIMEZONE_CHANGE | In request message, this value is used to indicate that UE timezone has been changed. |  |
| TARIFF_TIME_CHANGE | Tariff time change has happened. |  |
| MAX_NUMBER_OF_CHANGES_IN CHARGING_CONDITIONS | Max number of change has been reached |  |
| MANAGEMENT_INTERVENTION | Management intervention |  |
| CHANGE_OF_UE_PRESENCE_IN PRESENCE_REPORTING_AREA | In request message, this value is used to indicate that Change of UE presence in PRA has happened. In response message, this value is used to indicate a request of reporting the event that the user enters/leaves the area(s) as indicated in the presenceReportingArea Attribute |  |
| CHANGE_OF_3GPP_PS_DATA_OFF_STATUS | In request message, this value is used to indicate that Change of 3GPP PS Data off status has happened. |  |
| SERVING_NODE_CHANGE | A serving node (e.g., AMF) change in the NF Consumer |  |
| REMOVAL_OF_UPF | A used UPF is removed |  |
| ADDITION_OF_UPF | A new UPF is added. |  |
| INSERTION_OF_ISMF | A new I-SMF is inserted |  |
| REMOVAL_OF_ISMF | A used I-SMF is removed |  |
| CHANGE_OF_ISMF | A used I-SMF is removed, and a new I-SMF is inserted |  |
| HANDOVER_CANCEL | The handover is cancelled. |  |
| HANDOVER_START | The handover is started. |  |
| HANDOVER_COMPLETE | The handover is complete. |  |
| ADDITION_OF_ACCESS | Addition of access to the MA PDU session | ATSSS |
| REMOVAL_OF_ACCESS | Removal of access to the MA PDU session | ATSSS |
| START_OF_SDF_ADDITIONAL_ACCESS | Start of service data flow on additional access in a MA PDU session | ATSSS |

End of changes