3GPP TSG-SA5 Meeting #134e 	S5-206169

# e-meeting 16th – 25th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0016 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add lifecycle management of control loop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | COSLA |  |  |  |  |  | Date: |  |  | 2020-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Lifecycle management of control loop is missing in Rel-16 COSLA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added high level description for Lifecycle management of control loop. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management of control loop is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of changes

4.2.x	Lifecycle management of control loop

The lifecycle of a control loop can be described by the following four phases:

- 	Preparation

In the preparation phase the control loop does not exist.The preparation phase precedes the creation of a control loop. It includes the design of a control loop. E.g. the design of the SLS goals and the corresponding configurations for the control loop.

- 	Commissioning

In the commissioning phase the control loop designed in the preparation phase is instantiated. The instantiation includes the creation and registration of the control loop and its stages. It also includes activity of parameter configurations.

- 	Operation

After the commisioning phase, the control loop can be activated and run to assure its goals. During the runtime, the operation activities include monitor, evaluate, modify, deactive the control loop.

- 	Decommissioning

In the decommission phase the control loop is terminated. The termination activity includes the deregistration of the control loop and its stages. After termination the control loop does not exist anymore.

2nd of changes

6.2	Requirements

REQ-CSA-CON-01 The 3GPP management system shall have the capability to take actions for a set of communication services serving certain group of UEs based on the target SLS.

REQ-CSA-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSA-CON-03 The 3GPP management system shall have the capability to analyse the performance information related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-05	The 3GPP management system shall have the capability to collect NSI related data from one or more 5GC NF(s).

NOTE 1:	An example for NSI related data may be QoE data.

REQ-CSA-CON-06	The 3GPP management system shall have the capability to derive which communication service is associated to the QoE data from the collected NSI related QoE data.

REQ-CSA-CON-07	The 3GPP management system shall have the capability to ascertain SLS breach.

REQ-CSA-CON-08	The 3GPP management system shall have the capability to perform the root cause analysis (e.g., identifying the underlying reason) for an SLS breach.

REQ-CSA-CON-09	The 3GPP management system shall have the capability to take corrective actions against the root cause identified.

REQ-CSA-CON-10 The 3GPP management system shall have the capability to translate communicate service requirements to cross domain SLS goal and single domain SLS goal.

REQ-CSA-CON-11 The 3GPP management system shall have the capability to collect single domain SLS analysis as input to cross domain SLS analysis.

REQ-CSA-CON-12	The 3GPP management system shall have the capability to allow its authorized consumer to control the SLS assurance (e.g. specify the SLS to be assured, enable/disable, specify the assurance time and update the SLS assurance requirements).

REQ-CSA-CON-13	The 3GPP management system shall have the capability to allow its authorized consumer to obtain the SLS assurance progress information and fulfil information.

NOTE 2:	The management system refers to the producer of management service for SLS assurance.

REQ-CSA-CON-xx	The 3GPP management system shall have the capability of lifecycle management of a control loop.

end of changes