3GPP TSG-SA5 Meeting #134e 	S5-206185

# e-meeting 16th - 25th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0015 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update coslaNrm yml |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | COSLA |  |  |  |  |  | Date: |  |  | 2020-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Error $ref in AssuranceControlLoop-Single in coslaNrm.yml. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the $ref in AssuranceControlLoop-Single. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error $ref in AssuranceControlLoop-Single remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of changes

B.2.1	OpenAPI document "coslaNrm.yml"

openapi: 3.0.3

info:

title: coslaNrm

version: 16.4.0

description:

OAS 3.0.1 specification of the Cosla NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.536 V16.4.0; 5G NRM, Slice NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.536/

paths: {}

components:

schemas:

#------------ Type definitions ---------------------------------------------------

ControlLoopLifeCyclePhase:

anyOf:

- type: string

enum:

- PREPARATION

- COMMISSIONING

- OPERATION

- DECOMMISSIONING

- type: string

TimeUnit:

anyOf:

- type: string

enum:

- SECOND

- MINUTE

- HOUR

- DAY

- type: string

OperationalState:

anyOf:

- type: string

enum:

- ENABLED

- DISABLED

- type: string

AdministrativeState:

anyOf:

- type: string

enum:

- LOCKED

- SHUTTING_DOWN

- UNLOCKED

- type: string

ObservationTime:

type: integer

ObservationTimePeriod:

type: object

AssuranceControlLoopGoal:

type: object

AssuranceGoalStatus:

type: object

AssuranceGoalStatusObserved:

type: object

AssuranceGoalStatusPredicted:

type: object

#-------- Definition of concrete IOCs --------------------------------------------

AssuranceControlLoop-Single:

allOf:

- $ref: '#/components/schemas/Top-Attr'

- type: object

properties:

operationalState:

$ref: '#/components/schemas/OperationalState'

administrativeState:

$ref: '#/components/schemas/AdministrativeState'

controlLoopLifeCyclePhase:

$ref: '#/components/schemas/ControlLoopLifeCyclePhase'

observationTimePeriod:

allOf:

- $ref: '#/components/schemas/ObservationTimePeriod'

- type: object

properties:

observationTime:

$ref: '#/components/schemas/ObservationTime'

timeUnit:

$ref: '#/components/schemas/TimeUnit'

AssuranceGoalStatus:

allOf:

- $ref: '#/components/schemas/AssuranceGoalStatus'

- type: object

properties:

assuranceGoalStatusObserved:

$ref: '#/components/schemas/AssuranceGoalStatusObserved'

assuranceGoalStatusPredicted:

$ref: '#/components/schemas/AssuranceGoalStatusPredicted'

managedEntity-Multiple:

$ref: '#/components/schemas/ManagedEntity-Multiple'

assuranceControlLoopGoal:

$ref: '#/components/schemas/AssuranceControlLoopGoal'

ManagedEntity-Single:

oneOf:

- $ref: 'sliceNrm.yaml#/components/schemas/NetworkSlice'

- $ref: 'sliceNrm.yaml#/components/schemas/NetworkSliceSubnet'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

AssuranceControlLoop-Multiple:

type: array

items:

$ref: '#/components/schemas/AssuranceControlLoop-Single'

ManagedEntity-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedEntity-Single'

end of changes