3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211162

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0410 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add IMS offline only charging information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The charging data for converged charging have been included in TS 32.260. The charging information of offline only charging should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Offline charging information for IMS service based charging in TS 32.260. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Offline charging for IMS service based charging cannot use. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1, 6.4.1.2.1, 6.4.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.4 	Data description for IMS converged charging

6.4.1	Message contents

6.4.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [57] and include charging information. The Charging Data Request can be of type [Event, Initial, Update, Termination].

Table 6.4.1.1.1 describes the use of these messages for converged charging.

# Table 6.4.1.1.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | MRFC, IMS-GWF, SIP AS | CHF |
| Charging Data Response | CHF | MRFC, IMS-GWF, SIP AS |

Table 6.4.1.1.2 describes the use of these messages for offline only charging.

# Table 6.4.1.1.2: offline only charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | MRFC, SIP AS | CHF |
| Charging Data Response | CHF | MRFC, SIP AS |

Second change

6.4.1.2.1	Charging Data Request message

Table 6.4.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for IMS converged charging and IMS offline only charging.

# Table 6.4.1.2.1.1: Charging Data Request message contents

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | Described in TS 32.290 [45] |
| Subscriber Identifier | OM | M | Described in TS 32.290 [45] |
| NF Consumer Identification | M | M | Described in TS 32.290 [45] |
| NF Functionality | M | OC | Described in TS 32.290 [45] |
| NF Name | OC | OC | Described in TS 32.290 [45] |
| NF Address | OC | OC | Described in TS 32.290 [45] |
| NF PLMN ID | OC | OC | Described in TS 32.290 [45] |
| Invocation Timestamp | M | M | Described in TS 32.290 [45] |
| Invocation Sequence Number | M | M | Described in TS 32.290 [45] |
| Retransmission Indicator | OC | OC | Described in TS 32.290 [45] |
| One-time Event | OC | OC | Described in TS 32.290 [45] |
| One-time Event Type | OC | OC | Described in TS 32.290 [45] |
| Service Specification Information | OC | OC | Described in TS 32.290 [45] |
| Supported Features | OC | - | Described in TS 32.290 [45] |
| Notify URI | OC | - | Described in TS 32.290 [45] |
| Triggers | OC | OC | This field is described in TS 32.290 [45] and holds the IMS specific triggers described in clause 5.x |
| Multiple Unit Usage | OC | - | Described in TS 32.290 [45] |
| Rating Group | M | - | Described in TS 32.290 [45] |
| Requested Unit | OC | - | Described in TS 32.290 [45] |
| Used Unit Container | OC | - | Described in TS 32.290 [45] |
| Triggers | OC | - | This field is described in TS 32.290 [45] and holds the IMS specific triggers described in clause 5.x |
| IMS Charging Information | OM | OM | This field holds the IMS specific information described in clause 6.x |

Editor’s Note:	The full structure of the charging data request is FFS.

Third change

6.4.1.2.2	Charging Data Response message

Table 6.4.1.2.2.1 illustrates the basic structure of a Charging Data Response message as used for IMS converged charging and offline only charging.

# Table 6.4.1.2.2.1: Charging Data Response Message Contents

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | Described in TS 32.290 [45] |
| Invocation Timestamp | M | M | Described in TS 32.290 [45] |
| Invocation Result | OC | M | Described in TS 32.290 [45] |
| Result code | OC | M | Described in TS 32.290 [45] |
| Failed parameter | OC | OC | Described in TS 32.290 [45] |
| Failure Handling | OC | OC | Described in TS 32.290 [45] |
| Invocation Sequence Number | M | M | Described in TS 32.290 [45] |
| Session Failover | OC | OC | Described in TS 32.290 [45] |
| Triggers | OC | OC | Described in TS 32.290 [45] |
| Multiple Unit information | OC | - | Described in TS 32.290 [45] |
| Result Code | OC | - | Described in TS 32.290 [45] |
| Rating Group | OM | - | Described in TS 32.290 [45] |
| Granted Unit | OC | - | Described in TS 32.290 [45] |
| Validity Time | OC | - | Described in TS 32.290 [45] |
| Announcement Information | OC |  | Described in TS 32.281 [41] |
| IMS Charging Information | OM | OM | This field holds the IMS specific information described in clause 6.x |

Editor’s Note:	The full structure of the charging data response is FFS.

End of changes