3GPP TSG-SA5 Meeting #135e 	S5-211182

# electronic meeting, online, 25 January - 3 February 2021				Revision of S5-20xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0277 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the Usage Reporting for URLLC Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per TS 23.501 and TS 23.502, the 5GS to support Ultra Reliable Low Latency Communication (URLLC) is specified. The corresponding usage reporting mechanism should be added. As per message flows for URLLC specified in the TS 23.502, the UPF will perform or stop the packet duplication and elimination for the QoS Flow when receives the indication from SMF whether decide to perform the redundant transmission. SMF reports the usage with the indication to specify whether the packet duplication and elimination is performed for the redundant transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the editor’s note in clause 5.1.14.2 and 5.1.14.4. Clarify the usage reporting for URLLC in clause 5.1.14.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The highly reliable URLLC services charging is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.14.2, 5.1.14.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.14.2	Support redundant transmission for high reliability communication

The SMF reports the redundant transmission type to the CHF to indicate which redundant transmission type is used for the PDU session or service data flow.

The charging information is collected independently per each redundant PDU session. The quota is granted for each redundant PDU session independently. The SMF reports the usage per redundant PDU session.

Next change

5.1.14.4	Usage Reporting

For dual connectivity based end to end Redundant User Plane Paths, SMF shall collect and report the usage which is the amount of traffic delivered to and forwarded from core network for each redundant PDU session.

For redundant transmission at transport layer and N3/N9 interface, the SMF shall collect and report the usage which is the amount of traffic resulting from packet elimination process delivered to and forwarded from core network. The redundant transmission indication to specify whether the usage is reported for packet duplication or elimination is included.

The CHF may determine and indicate SMF to report the usage for packet duplication or elimination with the redundant transmission reporting indication which is high priority. If the SMF can not support the usage report mechanism dynamically indicated by CHF, the SMF ignores the redundant transmission reporting indication.

End of change