3GPP TSG-SA5 Meeting #137-e	S5-213148rev3

# Online, , 10th May 2021 - 19th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0301 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of GERAN/UTRAN access by SMF+PGW-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 、 |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_NIESGU |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Definition of charging information is missing when SMF+PGW-C is selected for GERAN or UTRAN access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an annex for supporting of GERAN/UTRAN access by SMF+PGW-C. Add SGSN as one of Serving Network Function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible for operators to simplify the charging architecture when 5GS is deployed and 2G or 3G still exists in their network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex X (normative):
Support of GERAN/UTRAN access

X.1	General

This Annex specifies Nchf from SMF+PGW-C enhanced to support UE accessing the network via GERAN/UTRAN.

X.2	5G data connectivity charging principles and scenarios

X.2.1	5G data connectivity charging principles

The 5G data connectivity charging principles as described in clause 5,1 shall apply for fields applicable when SMF+PGW-C supports GERAN/UTRAN access.

For GERAN/UTRAN access access scenario, the Charging Identifier for the PDP Context will be generated by SMF+PGW-C.

X.2.2	5G data connectivity converged online and offline charging scenarios

The 5G data connectivity converged online and offline charging scenarios as described in clause 5,2 shall apply for fields applicable when SMF+PGW-C supports GERAN/UTRAN access.

X.3	Data description for 5G data connectivity charging

X.3.1	Data description for supporting GERAN/UTRAN

X.3.1.1	Message contents

The Charging message as described in clause 6.1.1 shall apply to the SMF+PGW-C to support GERAN/UTRAN access.

When UE is connected to SMF+PGW-C via GERAN/UTRAN, the information included in clause 6.1.1 and TS 32.290[57] is supported.

X.3.1.2	Ga message contents

X.3.1.3	CDR description on the Bd interface

The CDR description defined in clause 6.1.3 shall apply for fields applicable when SMF+PGW-C supports GERAN/UTRAN access.

X.3.2	5G data connectivity charging specific parameters

X.3.2.1	Definition of 5G data connectivity charging information

The charging information defined in clause 6.2.1 is used for the SMF+PGW-C to support GERAN/UTRAN access.

The specific PDU session charging information when UE is connected to SMF+P-GW-C via GERAN/UTRAN is provided as defined in clause 6.2.1.2, with the following difference:

Table X.3.2.1-1: Structure of PDU Session Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Charging Id | OM | Refer table 6.2.1.2, but holds the Charging Id for PDP Context. |
| Home Provided Charging Id | - | This field is not applicable to GERAN/UTRAN access. |
| User Information | OM | Described in table 6.2.1.2. |
| User Identifier | OC | Refer table 6.2.1.2, but contains the identification of the user (i.e. MSISDN). |
| User Equipment Info | OC | Refer table 6.2.1.2, but holds the identification of the terminal (i.e. IMEI, MAC Address) . |
| unauthenticatedFlag | OC | Refer table 6.2.1.2, but indicates the served IMSI is not authenticated. |
| Roamer In Out | OC | Described in table 6.2.1.2. |
| User Location Info | OC | Described in table 6.2.1.2. |
| MA PDU Non 3GPP User Location info | - | This field is not applicable to GERAN/UTRAN access. |
| User Location Time | OC | Described in table 6.2.1.2. |
| MA PDU Non 3GPP User Location Time | - | This field is not applicable to GERAN/UTRAN access. |
| UE Time Zone | OC | Described in table 6.2.1.2. |
| Presence Reporting Area Information | OC | Described in table 6.2.1.2. |
| PDU Session Information | OC | Refer table 6.2.1.2,but of PDP Context information. |
| PDU Session ID | M | Refer table 6.2.1.2,but of PDP Context. |
| Network Slice Instance Identifier | - | This field is not applicable to GERAN/UTRAN access. |
| PDU Type | OM | Refer table 6.2.1.2, but of PDP Context. |
| PDU Address | OC | Described in table 6.2.1.2. |
| PDU Ipv4 Address | OC | Refer table 6.2.1.2, but of the served IMSI allocated for PDP Context. |
| PDU IPv6 Address with Prefix | OC | Refer table 6.2.1.2, but of the served IMSI allocated for PDP Context. |
| PDU Address prefix length | OC | Described in table 6.2.1.2. |
| IPv4 Dynamic Address Flag | OC | Described in table 6.2.1.2. |
| IPv6 Dynamic Address Flag | OC | Described in table 6.2.1.2. |
| SSC Mode | - | This field is not applicable to GERAN/UTRAN access. |
| MA PDU session information | - | This field is not applicable to GERAN/UTRAN access. |
| MA PDU session indicator | - | This field is not applicable to GERAN/UTRAN access. |
| ATSSS capability | - | This field is not applicable to GERAN/UTRAN access. |
| SUPI PLMN ID | OC- | Refer table 6.2.1.2 but of the IMSI. |
| Serving Network Function ID | OC | Described in table 6.2.1.2. |
| Serving Network Function Functionality | M | Refer table 6.2.1.2, but ….This field holds the functionality of the Serving Network Function: i.e. SGSN. This field holds “SGSN” when SMF+PGW-C serves GERAN/UTRAN access. |
| Serving Network Function Name | - | This field is not applicable to GERAN/UTRAN access. |
| Serving Network Function Addresses | OC | Described in table 6.2.1.2. |
| Serving Network Function FQDN | OC | Described in table 6.2.1.2. |
| Serving Network Function PLMN ID | OC | Described in table 6.2.1.2. |
| AMF Identifier | - | This field is not applicable to GERAN/UTRAN access. |
| Serving CN PLMN ID | OC | Described in table 6.2.1.2. |
| RAT Type | OC | Described in table 6.2.1.2. |
| MA PDU Non 3GPP RAT Type | - | This field is not applicable to GERAN/UTRAN access. |
| Data Network Name Identifier | M | Refer table 6.2.1.2,with the identifier of the APN the user is connected to. |
| DNN Selection Mode | - | This field is not applicable to GERAN/UTRAN access. |
| Authorized QoS Information | OC | Refer table 6.2.1.2, but to PDP Context. QoS information mapped according interaction with PCC as specified in clause 4.11.0a.2 of TS 23.502 [201] |
| Subscribed QoS Information | OC | Refer table 6.2.1.2, but for the PDP Context. QoS information mapped according interaction with PCC as specified in clause 4.11.0a.2 of TS 23.502 [201]. |
| Authorized Session-AMBR | OC | Refer table 6.2.1.2, but for the PDP Context. |
| Subscribed Session-AMBR | OC | Refer table 6.2.1.2, but for the PDP Context. |
| PDU session start Time | OC | Refer table 6.2.1.2, but when PDP Context starts. |
| PDU session stop Time | OC | Refer table 6.2.1.2, but when PDP Context terminates. |
| Diagnostics | OC | Described in table 6.2.1.2, but of the PDP Context. |
| Enhanced Diagnostics | OC | Described in table 6.2.1.2, but of the PDP Context. |
| Charging Characteristics | OC | Refer table 6.2.1.2, but for this PDP Context. |
| Charging Characteristics Selection Mode | OC | Described in table 6.2.1.2. |
| 3GPP PS Data Off Status | OC | Described in table 6.2.1.2. |
| Session Stop Indicator | - | This field is not applicable to GERAN/UTRAN access. |
| Redundant Transmission Type | - | This field is not applicable to GERAN/UTRAN access. |
| RAN Secondary RAT Usage Report | - | This field is not applicable to GERAN/UTRAN access. |
| NG RAN Secondary RAT Type | - | This field is not applicable to GERAN/UTRAN access. |
| Qos Flows Usage Reports | - | This field is not applicable to GERAN/UTRAN access. |
| QoS Flow Id | - | This field is not applicable to GERAN/UTRAN access. |
| Start Timestamp | - | This field is not applicable to GERAN/UTRAN access. |
| End Timestamp | - | This field is not applicable to GERAN/UTRAN access. |
| Downlink Volume | - | This field is not applicable to GERAN/UTRAN access. |
| Uplink Volume | - | This field is not applicable to GERAN/UTRAN access. |

The specific PDU Container Information when UE is connected to SMF+P-GW-C via GERAN/UTRAN is provided as defined in clause 6.2.1.3, with the following difference:

Table X.3.2.1-2: Structure of PDU Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Time of First Usage | OC | Described in table 6.2.1.3.1. |
| Time of Last Usage | OC | Described in table 6.2.1.3.1. |
| QoS Information | OC | Described in table 6.2.1.3.1. |
| QoS Characteristics | OC | Described in table 6.2.1.3.1. |
| AF Charging Identifier | - | This field is not applicable to GERAN/UTRAN access. |
| AF Charging Id String | - | This field is not applicable to GERAN/UTRAN access. |
| User Location Information | OC | Described in table 6.2.1.3.1. |
| UE Time Zone | OC | Described in table 6.2.1.3.1. |
| Presence Reporting Area Information | OC | Described in table 6.2.1.3.1. |
| Serving Network Function ID | OC | Described in table 6.2.1.3.1. |
| RAT Type | OC | Described in table 6.2.1.3.1. |
| Sponsor Identity | OC | Described in table 6.2.1.3.1. |
| Application Service Provider Identity | OC | Described in table 6.2.1.3.1. |
| Charging Rule Base Name | OC | Described in table 6.2.1.3.1. |
| 3GPP PS Data Off Status | OC | Described in table 6.2.1.3.1. |
| MA PDU Steering functionality | - | This field is not applicable to GERAN/UTRAN access. |
| MA PDU Steering mode | - | This field is not applicable to GERAN/UTRAN access. |

Roaming QBC information when UE is connected to SMF+P-GW via GERAN/UTRAN is defined in table X.3.2.1-3:

Table X.3.2.1-3:  Roaming QBC information

| Information Element | Category | Description |
| --- | --- | --- |
| MultipleQFIcontainer | OC | Refer table B.2.2.1.2-1, but for the PDP Context. |
| Triggers | OC | Described in table B.2.2.1.2-1. |
| Trigger Timestamp | OC | Described in table B.2.2.1.2-1. |
| Uplink Volume | OC | Described in table B.2.2.1.2-1. |
| Downlink Volume | OC | Described in table B.2.2.1.2-1. |
| Local Sequence Number | OC | Described in table B.2.2.1.2-1. |
| QFI Container information | OC | This field holds the data container information defined in clause X.3.2.1-4. |

Roaming QBC information when UE is connected to SMF+P-GW via GERAN/UTRAN is defined in table X.3.2.1-4:

Table X.3.2.1-4:  QFI Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| QoS Information | OC | Described in table B.2.2.1.3-1. |
| QoS Characteristics | OC | Described in table B.2.2.1.3-1. |
| User Location Information | OC | Described in table B.2.2.1.3-1. |
| Presence Reporting Area Information | OC | Described in table B.2.2.1.3-1. |
| RAT Type | OC | Described in table B.2.2.1.3-1. |
| Report Time | M | Described in table B.2.2.1.3-1. |
| 3GPP PS Data Off Status | OC | Described in table B.2.2.1.3-1. |
| EPS bearer Charging Id | OM | Refer table B.2.2.1.3-1, but for the PDP Context. |
| Diagnostics | OM | Refer table B.2.2.1.3-1, but for the PDP Context. |
| Enhanced Diagnostics | OC | Refer table B.2.2.1.3-1, but for the PDP Context. |

X.3.2.2	Detailed message format for converged charging

The basic structure of supported fields in Charging Data Request/Response message defined in clause 6.2.2 shall apply for fields applicable when SMF+PGW-C supports GERAN/UTRAN access.

X.3.2.3	Formal 5G data connectivity charging parameter description

TheCHF CDR parameters and resources attributes defined in clause 6.2.3 shall apply for fields applicable when SMF+PGW-C supports GERAN/UTRAN access.

End of change