3GPP TSG-SA5 Meeting #137e 	S5-213408

# e-meeting 10th - 19th May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CHO requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_HOO |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add requirements for the development of management of CHO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification level requrements for CHO management added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.1.1.X, 6.4.1.X, 6.4.1.X.1, 6.4.1.X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This is input to the Rel-17 28.313 draft CR for WI E_HOO. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-206212 |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

CHO	Conditional Handover

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

Next change

#### 6.1.1.X	CHO management

REQ-DCHO-FUN-1	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable CHO from one cell to another cell.

REQ-DCHO-FUN-2	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to configure parameters for CHO, e.g. threshold(s), timer(s).

REQ-DCHO-FUN-3	The producer of NF performance assurance MnS should have the capability to produce measurements related to CHO.

Next change

#### 6.4.1.X	CHO management

##### 6.4.1.X.1	Starting CHO for a cell

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure and start CHO (see TS 38.300 clause 9.2.3.4) from one cell to another cell. |  |
| Actors and Roles | D-SON management function to support the CHO function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - CHO is not in operation from the source cell to the target cell. |  |
| Begins when | The D-SON management function intends to enable CHO from the source cell to the target cell. |  |
| Step 1 (O) | The D-SON management function requests the producer of NF provisioning MnS to configure parameters for the management of CHO on the source cell. |  |
| Step 2 (M) | The D-SON management function requests the producer of NF provisioning MnS to enable the CHO function from the source cell to the target cell. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | CHO is in operation from the source cell to the target cell. |  |
| Traceability | REQ-DCHO-FUN-1, REQ-DCHO-FUN-2 |  |

##### 6.4.1.X.2	Start measuring CHO related measurements from a cell

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To receive CHO related measurements for a cell. |  |
| Actors and Roles | D-SON management function to support the CHO function. |  |
| Telecom resources | - gNB; - The producer of NF performance MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - The CHO function is in operation. |  |
| Begins when | The D-SON management function intends to receive CHO related measurements. |  |
| Step 1 (M) | The D-SON management function subscribes to CHO related measurement from the producer of NF performance MnS. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The D-SON management function receives measurements when they are published. |  |
| Traceability | REQ-DCHO-FUN-3 |  |

End of changes