3GPP TSG-SA5 Meeting #138-e 	S5-214388

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR |  | rev |  | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add YANG Solution Set for 5GDMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To define stage 3 YANG for 5G Discovery Management Service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add YANG solution set for 5GDMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be no stage 3 YANG for 5GDMS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.X (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge Link: https://forge.3gpp.org/rep/sa5/MnS/tree/S5-214388_Rel-17_CR_28.623_Add_YANG_Solution_Set_for_5GDMS |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Solution set is based on IOC agreed in S5-213538. |  |  |  |  |  |  |  |  |

1st Modified Section

## D.2.X	module _3gpp-common-mnsregistry.yang

<CODE BEGINS>

module _3gpp-common-mnsregistry {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-mnsregistry";

prefix "mnsregist3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the MNSRegistry Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-08-10 { reference "Initial revision"; }

feature MNSRegistry {

description "Class representing MsS Discovery Service Resgistry.";

}

grouping MNSRegistryGrp {

description "Represents the MNSRegistry IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

list mnsList {

description "List of MnS Data.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses MNSDataGrp;

}

}

}

grouping MNSDataGrp {

description " This datatype represents an available Management Service (MnS

and provides the data required to support its discovery.";

reference "3GPP TS 28.541";

leaf mnsLabel {

description "Human-readable name of management service.";

mandatory true;

type string;

}

leaf mnsType {

description "Type of management service.

allowedValues: PROVISIONING, FAULT_SUPERVISION, PERFORMANCE_ASSURANCE.";

mandatory true;

type enumeration {

enum PROVISIONING;

enum FAULT_SUPERVISION;

enum PERFORMANCE_ASSURANCE;

}

}

leaf mnsVersion {

description "Version of management service.";

mandatory true;

type string;

}

leaf componentTypeAAddress {

description "Addressing information for Management Service operations.";

mandatory true;

type string;

}

leaf componentTypeBAddress {

description "Addressing information for Management Service information

models.";

mandatory true;

type string;

}

}

augment "/subnet3gpp:SubNetwork" {

uses MNSRegistryGrp;

}

}

<CODE ENDS>