| 3GPP TR 28.826 V0.2.0 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on Nchf charging services phase 2 improvements and optimizations (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Overview	7

5	Charging scenarios and key issues	7

5.1	Charging information optimization	7

5.1.1	General	7

5.1.2	Use cases	7

5.1.2.1	Use Case #1a trigger without usage	7

5.1.2.2	Use Case #1b size of charging information	8

5.1.3	Potential charging requirements	8

5.1.4	Key issues	8

5.2	Charging request optimization	9

5.3	Rating input enhancement	9

5.3.1	Use Case #3a Enhancement of input to CHF Rating.	9

5.3.2	Potential charging requirements	9

5.3.3	The key issues	9

5.3.3.1 	Key issue #3a: Enhancement of input to CHF Rating	9

5.4	Chargeable events and sessions cancelling	9

5.5	Non-blocking mechanism enhancement	9

5.5.1	Use Case # 5a Failed to grant the quota	9

5.5.2	Potential charging requirements	10

5.5.3	The key issues	10

5.5.3.1 	Key issue #5a: the Non-Blocking Mode Disable/Enable	10

5.5.4	Solutions	10

6	Documentation improvements	10

7	Conclusions and recommendations	10

Annex A (informative): Change history	11

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This report is to study enhancement of the Nchf converged charging service.

# 1	Scope

The present document studies the potential use cases, requirements, and solutions for the enhancements of the Nchf converge charging service.

The study will cover and identify gaps for:

-	optimizations regarding amount of information in the request and number of requests,

-	enhancement of input to rating,

- 	cancelling of chargeable events for unsuccessful scenarios,

- 	enhancement of non-blocking mechanism,

-	enhancements of triggers having more consistent triggers and use of NF specific triggers

Improvements on how to maintain the source code presentations (yaml and ASN.1) the document structure for these will be studied,

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Overview

The Nchf converged charging services was first released in Rel-15. This first iteration was to a large extent, with some minor exceptions, copying and combining its predecessors, Ro and Rf, based on Diameter to a service based interface (SBI) supporting converged charging service. The depiction of the internal architecture of the Converged Charging System (CCS) was to a large extent based on the Online Charging System (OCS).

The combination of the Diameter Ro and Rf interfaces into one converged charging service together with the move of the integration point for CDRs have led to that all the information previously reported and transported over two or more interfaces now needs to be transported over a single interface. This makes it essential that the converged charging interface is optimized in regard to the amount of information and number of reports sent, or at least allows for such an optimization if needed.

Low latency requirements increased by new business cases may need enhancements to the non-blocking mechanism.

Charging of new service together with converged charging have increased the need for more information in the input to rating.

The use of event-based charging has led to a need for cancelling the charging for the chargeable event, e.g. refund, for unsuccessful scenarios or cases not possible to fulfil.

The document structure of the actual service, i.e. yaml, followed the handling of Ro and Rf, based on Diameter, which may need improvements.

# 5	Charging scenarios and key issues

## 5.1	Charging information optimization

### 5.1.1	General

When there is no 5G data connectivity usage, which may be quite frequent in the case of always connected, when a trigger is active and the event occurs, sending a lot of information might be unnecessary. For online the need of getting the trigger even if there is no usage depends on if it require rating and may impact the granted quota. For the offline it will depend on the use case if the trigger event is needs to be reported in the case of no usage.

The size of a request is only limited by the maximum size of the request in TS 29.500 [x] and controlled by the NF consumer. The CHF may reject the request if the request is over that limit. There is no other way for the CHF to control the size of the message other than controlling or limit the number of charging conditions changes before reporting, this won’t state the size or take compression into consideration.

### 5.1.2	Use cases

#### 5.1.2.1	Use Case #1a trigger without usage

The operator has an offer for 5G data connectivity, which is volume based and available for both online and offline (in converged charging this means quota and not quota managed) charged subscribers. The offer has different rates depending on the PLMN accessed i.e., if it’s roaming and where it’s roaming. The operator would also like to get statics on how much the users are using in different PLMNs, as well as for some IoT devices how many times they change PLMN.

A user has a UE that has a PDU session ongoing and is on the border between HPLMN and VPLMN and the UE keeps switching between the networks. The user chooses to only use the UE when it’s connected to the HPLMN.

An enterprise has several IoT devices that each have their PDU sessions ongoing.

The operator would like to minimize the number of interrogations and the information sent between the SMF and CHF, this means that this means that for:

- UEs that are online (quota managed) charged and have the offer, a PLMN change should only immediately trigger a CHF when there is usage

- UEs that are offline (not quota managed) charged and have the offer, a PLMN change should create a container only when there is usage

- some IoT devices that are offline (not quota managed) charged, a PLMN change should create a container independent of if there is usage or not

This could be relevant for the following triggers: QoS, GFBR guaranteed status, user location, serving node, Presence Reporting Area(s), 3GPP PS data off status, tariff time, UE time zone, PLMN, RAT type, Session-AMBR, UPF, I-SMF, handover status, access addition/removal and redundant transmission change

#### 5.1.2.2	Use Case #1b size of charging information

The operator has a number of CHFs wants to optimize their storage and would therefore like to control the size of the converged charging requests based on the available space in each CHF. It should be noted that there is a trade of between size of the requests and the number of requests.

There are two parts of to the storage in the CHF one is the CDR storage before sending it to the CGF and the allocation of the resource for the SBI. The resource may be stored uncompressed for fast access, whereas the charging information in the converged charging request may be compressed to limit network utilization. This means that the operator would like to control the size of the message both before and after the compression.

When a converged charging request is sent the amount of information included in the intimal is often limited since not much usage can have been done, so it is more important to be able to control the sizes of the update and termination than the initial.

The converged charging request can contain request for units, offline unit reports and online units report for the same rating group.

### 5.1.3	Potential charging requirements

REQ-CH_INFO-01: Optimization of charging information may be needed when zero units used.

REQ-CH_INFO-02: Support for control of charging request size.

### 5.1.4	Key issues

The following key issues are identified:

-	Key Issue #1a: Identify the charging scenarios and triggers which can be controlled when there is no usage.

-	Key Issue #1b: How to control if information is sent when there is no usage.

-	Key Issue #1c: How to control if containers are created when these is no usage.

-	Key Issue #1d: How to control the size of the information in the converged charging request, dependent or independent of compression.

-	Key Issue #1e: How to handle when unit request and report for one rating group are split over several requests

## 5.2	Charging request optimization

## 5.3	Rating input enhancement

### 5.3.1	Use Case #3a Enhancement of input to CHF Rating.

A End User (subscriber) has subscribed to a service with multiple QoSs. The PCF dynamically decides to authorize a QoS based on the network information. When granting a quota, the CHF may need additional authorized QoS information corresponding to the Rating Group to assist  CHF to grant quota per Rating Group.

As per the current specifications, the PDU charging information includes the default QoS information (Authorized QoS Information for PDU session and Subscribed QoS Information), which there is not the QoS information corresponding to the Rating Group.

The QoS information is an example as the input to the CHF rating.

Editor’s note: The other information for the enhancement of input to CHF rating will be introduced case by case and ffs.

### 5.3.2	Potential charging requirements

REQ-3GPPCH-ER-01	The 5G system should support the enhancement of input to CHF rating based on the QoS information.

### 5.3.3	The key issues

#### 5.3.3.1 	Key issue #3a: Enhancement of input to CHF Rating

This key issue is for investigating how to support the enhancement of input to CHF Rating considering REQ-3GPPCH-01.

This investigation covers the following:

-	identification and classification of information which can be used as the input to CHF rating.

-	identification the Network Funtions which can provide the input to support the CHF rating.

-	determine the interaction to support the enhancement of input to CHF rating.

## 5.4	Chargeable events and sessions cancelling

## 5.5	Non-blocking mechanism enhancement

### 5.5.1	Use Case # 5a Failed to grant the quota

When End user accesses the services and the non-blocking mode is adopted based on the PCC rule, the quota request is sent to the CHF not before the service delivery. The quota is fail to be granted by CHF as the following situations:

- the end-user service restrictions or limitations related to the end-user

- the end user's account could not cover the requested service

- CHF denies the service request in order to terminate the service for other reasons.

- Etc.

The service delivery will be started and then stopped because of the service start is beyond the control of the CHF.

In order to avoid the UE frequently initiates service requests which may cause the CHF attacking, or affect user’s experience, in that case, the non-blocking mode can be disabled temporarily based on the information of CHF and resumed later.

### 5.5.2	Potential charging requirements

REQ-3GPPCH-NB-01	The 5G system should support to disable temporarily the non-blocking mode.

REQ-3GPPCH-NB-02	The 5G system should support to resume the non-blocking mode.

### 5.5.3	The key issues

#### 5.5.3.1 	Key issue #5a: the Non-Blocking Mode Disable/Enable

This key issue is for investigating how to disable/enable non-blocking mode considering REQ-3GPPCH-NB-01 and REQ-3GPPCH-NB 02.

This investigation covers the following:

-	identification the non-blocking mode disable/enable is per serivce or per UE.

-	identification the Network Funtions which disable/enable non-blocking mode.

-	determination of the interactions required to disable/enable non-blocking mode for the special user/service.

### 5.5.4	Solutions

Editor’s note: using the Nchf_SpendingLimitControl service may be an alternative solution.

# 6	Documentation improvements

# 7	Conclusions and recommendations

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-09 | SA5#138e | S5-214430 |  |  |  | Initial skeleton (v0.0.0) of TR 28.826 | 0.0.0 |
| 2021-10 | SA5#139e | S5-215241 |  |  |  | Study document structure | 0.1.0 |
| 2021-10 | SA5#139e | S5-215469 |  |  |  | Addition of introduction and scope | 0.1.0 |
| 2021-10 | SA5#139e | S5-215470 |  |  |  | Addition of overview | 0.1.0 |
| 2021-10 | SA5#139e | S5-215590 |  |  |  | Add the topic for Non-blocking Mode Enhancement | 0.1.0 |
| 2021-10 | SA5#139e | S5-215591 |  |  |  | Add the topic for Enhancement of input to CHF Rating | 0.1.0 |
| 2021-11 | SA5#140e | S5-216523 |  |  |  | Use case trigger without usage | 0.2.0 |
| 2021-11 | SA5#140e | S5-216525 |  |  |  | Use case size of charging information | 0.2.0 |