3GPP TSG-SA5 Meeting #141-e 	S5-221139

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.130 | CR | 0019 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution description for the requirements for the management of the shared NG-RAN NE(s) in MOCN network sharing scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Orange,China Mobile, China Unicom,China Telecom, Deutsche Telekom,Ericsson, Telefonica |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS |  |  |  |  |  | Date: |  |  | 2022-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror correspo.nding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NR NRM to support NG-RAN MOCN network sharing is defined in TS 28.541, however, the description on how to use such NRM to support MOCN network sharing scenario/requirements is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add workflows for the management of the shared NG-RAN MOCN network sharing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[5]	3GPP TS 23.251: "Network sharing; Architecture and functional description".

[6]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements"

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage2".

[X]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[Y]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

2nd Change

X	Solution description for the requirements for the management of the shared NG-RAN NE(s) in MOCN network sharing scenario

X.1 Management of the shared NG-RAN NE(s) in MOCN network sharing scenario with the same cell Identity broadcast

The NG-RAN MOCN Network Sharing with same cell identity broadcast scenario is illustrated in Figure 4.1-2 and corresponding requirements is defined in clause 5.1.4. This clause describes the workflows for the management of the shared NG-RAN NE(s) in MOCN network sharing scenario with the same cell identity broadcast.

In this workflow, the radio access network (i.e. one or multiple shared NG-RAN NE(s)) is shared between two POPs (POP A identified by PLMN#1and POP B identified by PLMN#2). Both MnS consumer and MnS producer for the management of shared NG-RAN NE(s) belong to MOP.

For the Req-MOCN_SameCellId_Cfg-CON-1:

MnS consumer determines the individual EP_NgC MOI and EP_NgU MOI (see the attributes of NgC and NgU in TS 28.541[X]) for each POP (POP A and POP B), and requests MnS producer to create and configure EP_NgC MOI and EP_NgU MOI for each POP.

MnS producer creates and configures the EP_NgC MOI and EP_NgU MOI for each POP based on the requests from MnS consumer. The EP_NgC MOI and EP_NgU MOI are name containe by same GNBCUCPFunction MOI and GNBCUUPFunction MOI which is shared for different POPs.

For the Req- MOCN_SameCellId_Cfg-CON-2:

MnS consumer determines the attribute "PLMNInfoList" in NRCellDU MOI (see the attribute definition in TS 28.541[X]), which includes the PLMN#1 and PLMN#2, and requests MnS producer to configure NRCellDU MOI with attribute "PLMNInfoList".

MnS producer configures the NG-RAN NE(s) (i.e. subtree of ManagedElement MOI) based on the requests from MnS consumer, including configuring the NRCellDU MOI with attribute "PLMNInfoList" to include PLMN#1 and PLMN#2.

For the Req- MOCN_SameCellId_Per-CON-3:

MnS producer collects the individual measurements for POP A and POP B in PLMN granularity by utilizing PLMN granularity subcounter. For the concrete PLMN granularity measurements, see TS 28.552[Y].

MnS producer sends the individual measurements for POP A and POP B in PLMN granularity to MnS consumer.

X.2 Management of the shared NG-RAN NE(s) in MOCN network sharing scenario with the multiple cell Identity broadcast

The NG-RAN MOCN Network Sharing with multiple cell identity broadcast scenario is illustrated in Figure 4.1-3 and corresponding requirements is defined in clause 5.1.5. This clause describes the workflows for the management of the shared NG-RAN NE(s) in MOCN network sharing scenario with the muliple cell identity broadcast.

In this workflow, the radio access network (i.e. one or multiple shared NG-RAN NE(s)) is shared between two POPs (POP A identified by PLMN#1and POP B identified by PLMN#2). Both MnS consumer and MnS producer for the management of shared NG-RAN NE(s) belong to MOP.

For the Req-MOCN-MultiCellId-Cfg-CON-1:

MnS consumer determines the individual EP_NgC MOI and EP_NgU MOI (see the attributes of NgC and NgU in TS 28.541[X]) for each POP (POP A and POP B), and requests MnS producer to create and configure EP_NgC MOI and EP_NgU MOI for each POP.

MnS producer creates and configures the EP_NgC MOI and EP_NgU MOI for each POP based on the requests from MnS consumer. The EP_NgC MOI and EP_NgU MOI are name containe by corresponding POP's GNBCUCPFunction MOI and GNBCUUPFunction MOI.

For the Req-MOCN-MultiCellId-Cfg-CON-2

MnS consumer determines the individual OperatorDU MOI and NROperatorCellDU MOI (see the attributes of OperatorDU and NROperatorCellDU in TS 28.541[X]) for each POP (POP A and POP B), and requests MnS producer to create and configure OperatorDU MOI and NROperatorCellDU MOI for each POP.

MnS producer configures the NG-RAN NE(s) (i.e. subtree of ManagedElement) based on the requests from MnS consumer, including creates and configures OperatorDU and NROperatorCellDU MOI for each POP.

For the Req-MOCN-MultiCellId-Cfg-CON-3

MnS producer collects the individual measurements for POP A and POP B in PLMN granularity by utilizing PLMN granularity subcounter or associated with OperatorDU and NROperatorCellDU. For the concrete PLMN granularity measurements, see TS 28.552[Y].

MnS producer sends the individual measurements for POP A and POP B in PLMN granularity to MnS consumer.

For the Req-MOCN-MultiCellId-Cfg-CON-4 and Req-MOCN-MultiCellId-Cfg-CON-5

MnS consumer determines the individual EP_F1C MOI and EP_F1U MOI (see corresponding attributes in TS 28.541[X]) for each POP (POP A and POP B), and requests MnS producer to create and configure these MOIs for each POP. In case of common F1 intertface configuration, the values of the EP_F1C MOI and EP_F1U MOI attributes (including localAddress and remoteAddress) contained by different POP's OperatorDU MOI of the same GNBDUFunction MOI should be same.

MnS producer creates and configures the individual EP_F1C MOI and EP_F1U MOI for each POP based on the requests from MnS consumer. The EP_F1C MOI and EP_F1U MOI are name contained by corresponding POP's OperatorDU and associated to it's own GNBCUCPFunction MOI and GNBCUCPFunction MOI.

For the Req-MOCN-MultiCellId-Cfg-CON-6

MnS consumer determines the NRCellRelation MOI(s) (see corresponding attributes in TS 28.541[X]) for each POP (POP A and POP B), and requests MnS producer to create and configure NRCellRelation MOI(s) for each POP.

MnS producer configures the NG-RAN NE(s) (i.e. subtree of ManagedElement) based on the requests from MnS consumer, including creates and configures the individual NRCellRelation MOI for each POP.

End of Change