3GPP TSG-SA5 Meeting #143-e 	S5-223290rev2

- e-meeting, 9th – 17th April 2022

- Source:	Huawei

- Title:	Add the solution for CHF rating

- Document for:	Discussion

Agenda Item:	7.5.3

# 1	Decision/action requested

Discuss the solution for CHF rating.

# 2	References

[1]	3GPP TS 32.291 5G system, charging service;Stage 3

# 3	Rationale

Comprehensively consider the input information for CHF rating consideration.

# 4	Detailed proposal

1st change

### 5.3.1	General

In the current specifications for the CHF granting the quotas is based on the requested unit type (e.g. time or volume), and rating group as the input, and combined with the account of the subscriber from ABMF and the value of chargeable events in terms of monetary or non-monetary units from the RF.

Additional information that may be used to determine the reservation of quotas or rating consideration in the CCS is not defined, except for the rating group and requested unit.

Next change

### 5.3.5	Solutions

#### 5.3.5.1	Solutions #3.1 The Additional Information in RSU as the input for CHF rating

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

The possible solutions support the potential requirements REQ-3GPPCH-ER-01 and REQ-3GPPCH-ER-02 and Key Issue #3a, #3b and #3c to describe the enhancement of input to CHF rating.

##### 5.3.5.1.1	Solutions #3.1.1 The QoS in the Additional Information

The QoS information is measurement of the overall performance of a service.A solution could be to allow the QoS information in the Additional information as well as the rating group in the request for quota specified in the TS 32.291[x], as the input for CHF rating consideration, to be able to better allocate the right amount of quota needed at that moment for that rating group.

###### 6.1.6.2.1.9	Type RequestedUnit

Table 6.1.6.2.1.9-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| additionalInformation | AdditionalInformation | OC | 0..N | This field holds the additional information for the input of CHF rating consideration. | CHFRatingInput |
| NOTE 1: if none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

###### 6.1.6.2.2.X	Type AdditionalInformation

Table 6.1.6.2.2.x-1: Definition of type AdditionalInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qoSInformation | QosData | OC | 0..1 | This field describes the QoS applied for the requested used unit. |  |

##### 5.3.5.1.2	Solutions #3.1.2 The Service id in the Additional Information

The service identifier is the most detailed identification of a service data flow. A solution could be to allow the service identifier in the additional information as well as the rating group in the request for quota specified in the TS 32.291[x], as the input for CHF rating consideration, to be able to better allocate the right amount of quota needed at that moment for that rating group.

The additional information reuses the definition in the solution #3.1.1.

6.1.6.2.2.X	Type AdditionalInformation

Table 6.1.6.2.2.x-1: Definition of type AdditionalInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceId | ServiceId | OC | 0..N | This field identity of the used service linked to the rarting group. |  |

#### 5.3.5.2	Solutions #3.2 The QoS information in RSU as the input for CHF rating

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

The possible solution supports the potential requirements REQ-3GPPCH-ER-01 and partial supports Key Issue #3a, #3b and #3c to describe the enhancement of input to CHF rating.

The QoS information is the measurement of the overall performance of a service. A solution could be to allow the QoS information in the RequestedUnit as well as the rating group in the request for quota specified in the TS 32.291[x], as the input for CHF rating consideration, to be able to better allocate the right amount of quota needed at that moment for that rating group.

###### 6.1.6.2.1.9	Type RequestedUnit

Table 6.1.6.2.1.9-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| QosInformation | QosData | OC | 0..1 | This field describes the QoS applied for the RG. | CHFRatingInput |
| NOTE 1: if none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

End of change