3GPP TSG-SA5 Meeting #143e 	S5-223436

- Electronic meeting, online, 09 - 17 May 2022

- Source:	Huawei

- Title:	pCR TR 28.863 Key Issue # 1: Definition of KQI

- Document for:	Approval

Agenda Item:	6.5.20

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	 3GPP TS 28.863v0.1.0: "Study on Key Quality Indicators (KQIs) for 5G service experience"

[2]	 3GPP TS 32.862: "Study on Key Quality Indicators (KQIs) for service experience"

[3]	 3GPP TR21.905: "Vocabulary for 3GPP Specifications"

[4]	 3GPP TR26.944: "End-to-end multimedia services performance metrics"

[5]   ITU-T P.10/G.100 (11/2017): Vocabulary for performance and quality of service.

[6]   ETSI TS 102 250-1 V2.2.1 (2011-04): "Speech and multimedia Transmission Quality (STQ); QoS aspects for popular services in mobile networks; Part 1: Assessment of Quality of Service".

# 3	Rationale

The document gives a background survey of KQI related issues and the definition for KQI and 5G KQI.

# 4	Detailed proposal

It is proposed to apply the following changes as below in TR 28.863 [1].

Start of the first change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.862: "Study on Key Quality Indicators (KQIs) for service experience"

[3]	 3GPP TR26.944: "End-to-end multimedia services performance metrics"

[4]	 ITU-T P.10/G.100 (11/2017): Vocabulary for performance and quality of service.

[5]	 ETSI TS 102 250-1 V2.2.1 (2011-04): "Speech and multimedia Transmission Quality (STQ); QoS aspects for popular services in mobile networks; Part 1: Assessment of Quality of Service".

End of the first change

Start of the second change

## 4.1	Key Issue # 1: Definition of KQI

### 4.1.1	Description

In this clause, the background survey of the definition of KQI is provided.

#### 4.1.1.1	3GPP TR21.905

In TR21.905 [1],

no definition or abbreviation of KQI is provided.

#### 4.1.1.2	3GPP TR32.862

TR32.862 [2] gives a definition of KQI and some related survey on that:

The introduction of Key Quality Indicators (KQIs), which reflect the end-to-end service performance and quality, could be used to better represent real customer experience. NGMN NGCOR has defined principles for KQIs which could be used to evaluate the customer experience for fixed and mobile networks. It is proposed that SA5 takes the responsibility to study how to standardize the related service KQIs providing a standardised common base for end-to-end measurements and to fulfil the NGCOR requirements. KQIs have also been defined by ETSI (TS 102.250), 3GPP SA4 (TR 26.944), NGMN (P-SERQU) and QuEST Forum should also be considered.

The intention of KQI for the service experience is to try to reflect the customer experience with indicators reflecting them for example how many stalls occur while watching a movie, time delay before seeing the first screen of a movie. With this information, KQI could closely reflect the customer experience. The KPIs defined in the current specifications, e.g. 3GPP TS 32.450, are not differentiated by services, they only measure the bearer

The definition of KQI (Key Quality Indicators: Indicators which reflect the end-to-end service performance and quality) in TR32.862 should be reused and KQIs for the new scenarios need to be investigated and defined.

### 4.1.2	Potential solutions

#### 4.1.2.1	Potential solution #1: Definition of KQI

##### 4.1.2.1.1	Introduction

This clause describes the definition of KQI and 5G KQI.

##### 4.1.2.1.2	Description

Definition of KQI:

Key Quality Indicators (KQI): Indicators which reflect the end-to-end service performance and quality.

In 5G the vertical industries e.g. industry automation, intelligent vehicle, smart city, smart transportation etc., promote the services of e.g. video uploading service, AR/VR, remote controlling etc. The new services has the new requirements of lower latency, higher service priority, higher reliability, larger UL throughput,  huge connections for 5G networks or the combination of them. The diversity of the services requires the unique and different key quality indicators (KQI) models of 5G service experience. The 5G KQI are the KQIs of 5G service experience to represent real customer experience and reflect the end-to-end 5G service performance and quality.

Definition of 5G KQI:

5G Key Quality Indicators (5G KQI): Indicators which reflect the end-to-end 5G service performance and quality.

End of the second change