- 3GPP TSG-SA5 Meeting #143e	S5-223455rev1

- 9 May to 17 May 2022, E-meeting

- Source:	China Telecom

- Title:	Add potential solution for KI#1

- Document for:	Approval

- Agenda Item:	6.5.6.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TS 28.864-010: "Study on Enhancement of the management aspects related to NWDAF ".

[2]	3GPP TS 28.288: Architecture enhancements for 5G System (5GS) to support network data analytics services".

# 3	Rationale

In the last meeting, the key issue #1 for NWDAFFunction IOC enhancement to support Multiple NWDAF Deployment (see TR 28.864 [1]) was proposed and approved.

As described in TS 23.288 [2], the NWDAF architecture support the deployment of multiple NWDAF instances. If multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF (i.e.  aggregator NWDAF), as a collection of distributed NFs, or as a combination of both. The deployment of multiple NWDAF introduces the capability of "analytics aggregation capability" and "analytics metadata provisioning capability ".

This pCR is to provide the pontential solution for KI # 1. The solution is to enhance the NWDAFFunction IOC to support reflecting the differences on the NWDAF capability aspect in multiple NWDAF deployment.

# 4	Detailed proposal

Start of 1st Change

# 2	References

[x]	3GPP TS 28.541: "5G Network Resource Model (NRM) ".

## 4.1	Key Issue #1:  NWDAFFunction IOC enhancement to support Multiple NWDAF Deployment

### 4.1.2	Potential solutions

#### 4.1.2.1	Potential solution #1: the attribute for NWDAF capability

##### 4.1.2.1.1	Introduction

This solution is provided on how to enhance the NWDAFFunction IOC to support reflecting the differences on the NWDAF capability aspect in multiple NWDAF deployment. The proposed attribute indicating the different NWDAF capability is defined in the NWDAFFunction IOC.

The proposed solution can be a potential solution for the NWDAFFunction IOC enhancement to support multiple NWDAF deployment.

##### 4.3.2.1.2	Description

This proposed attribute indicating the "analytics aggregation capability" and "analytics metadata provisioning capability" of NWDAF is defined in NWDAFFunction IOC as follows:

Attribute name: nWDAFCapabilityIdentifier;

Documentation: It represents the different NWDAF capability identifier, which indicates whether the NWDAF has the "analytics aggregation capability" or "analytics metadata provisioning capability".

Allowed Values: The allowed value can be “0”, “1”, “2” and “3”, where “1” indicates the NWDAF has the "analytics aggregation capability", “2” indicates the NWDAF has the "analytics metadata provisioning capability", “3” indicates the NWDAF has both "analytics aggregation capability" and "analytics metadata provisioning capability" and “0” indicates the NWDAF does not have these two capabilities.

End of Modified Sections