3GPP TSG-SA5 Meeting #143-e 	S5-223417

# e-meeting, 9- 17May 2022

# Source:	CMCC, Huawei

# Title:	pCR TR 28.830 Add solution of key issue performance degradation

# Document for:	Approval

Agenda Item:	6.5.7.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	 SP-220153: "New SID on Fault Supervision Evolution"

[2]	S5-222733: "draft TR 28.830 Fault supervision evolution"; v0.1.0

3	Rationale

This document describes how to use incident management to automatically process and associate multi-dimensional data, quickly identify and generate quality deterioration incidents, and eliminate manual information check.

It is proposed to add solution of key issue performance degradation in draft TR 28.830.

4	Detailed proposal

This document proposes the following changes in TR 28.830.

1st Change

5	Key Issues and potential solutions

5.X	Key Issue #2: Performance degradation

5.X.1	Description

Editor’s note: This clause provides a description of the key issue.

5.X.2	Potential solutions

5.X.2.a	Potential solution #<1>: <Performance degradation>

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

This section describes how to identify and resolve performance degradation incidents.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

(1) Awareness: collects data such as alarms, resource utilization, performance, configuration or incident information from other incident MnS produce, etc.

(2) Analytics:

-  Incident identification: performs multi-data source correlation, including top cell identification, performance indicator trend correlation, and alarm correlation, to generate performance degradation incidents.

-  Incident diagnosis, demarcation, and solution recommendation: performs demarcation and root cause diagnosis, analyzes the performance degradation causes of each top N cell, and recommends potential recovery solutions.

(3) Decision: provides performance degradation incident evaluation, and resolution decision.

(4) Execution: executes corresponding recovery measures according to the diagnosis result and selected solution.

5.X.3	Conclusion - Impact on normative work

End of change