3GPP TSG-SA5 Meeting #144-e 	S5-224212

# e-meeting, 27 June - 1 July 2022

# Source:	Ericsson

# Title:	Adding solutions in clause 7.4 for CHF to CHF communication

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

The key issues #4d and #4c are missing solutions, reusing and adapting the solutions from clause 7.2.

# 4	Detailed proposal

First change

#### 7.2.4.x	Solution #4.x: Reusing Nchf_ConvergedCharging service API between CHFs

##### 7.2.4.x.1	General

A possible solution for key issue #4d covering requirements REQ-CH_CVTOA-01 and REQ-CH_CVTOA-02, service based interface to use between home CHF and additional actor CHF, would be to reuse the Nchf_ConvergedCharging service API. This would mean that the H-CHF would proxy the request from the AMF, SMF or SMSF to the A-CHF, and the same with the response. The H-CHF could do some changes to message like: filter (e.g. trigger), enrich, or convert (e.g., rating groups).

##### 7.2.4.x.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

#### 7.2.4.y	Solution #4.y: New Nchf service API between CHFs

##### 7.2.4.y.1	General

A possible solution for key issue #4d covering requirements REQ-CH_CVTOA-01 and REQ-CH_CVTOA-02, service based interface to use between home CHF and additional actor CHF, would be to create a new service API. This would mean that the H-CHF would translate the request from the AMF, SMF or SMSF to the new service API towards the H-CHF. This would mean that the message could look completely different and even the triggers in the H-CHF for sending the message towards the A-CHF wouldn’t have to be related to the request from AMF, SMF or SMSF.

##### 7.2.4.y.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

#### 7.2.4.z	Solution #4.z: Using NRF to find A-CHF

##### 7.2.4.z.1	General

A possible solution for key issue #4c, finding the correct CHF for solution #4.3  where H-CHF communicating with A-CHF.

Editor’s Note: The solution #4.1 where H-CHF communicate with A-CHF is FFS.

##### 7.2.4.z.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

##### 7.2.4.z.3	Message flows

The H-CHF would in this case use the NRF provided service to find the A-CHF, the selection can be based on any available attribute such as SUPI, CHF group, etc., where the A-CHF would be registered in the NRF.

#### 7.2.4.w	Solution #4.w: Using local configuration to find A-CHF

##### 7.2.4.w.1	General

A possible solution key issue #4c, finding the correct CHF for solution #4.3  where H-CHF communicating with A-CHF.

Editor’s Note: The solution #4.1 where H-CHF communicate with A-CHF is FFS.

##### 7.2.4.w.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

##### 7.2.4.w.3	Message flows

The H-CHF would in this case use the SUPI e.g., SUPI lists, IMSI number series, to find the A-CHF. In this case the A-CHF address would be pre-provisioned in the H-CHF and allocated for example to a SUPI list or an IMSI series.

End of changes