3GPP TSG-SA5 Meeting #144-e 	S5-224079

# e-meeting, 27 June - 1 July 2022

# Source:	Huawei

# Title:	Potential solution for SLA monitoring and evaluation

# Document for:	Approval

Agenda Item:	6.9.1.3

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TR 28.907 Study on enhancement of management of non-public networks v0.2.0

3	Rationale

It is proposed to add a potential solution for key issue #4 in draft TR 28.907 [1].

4	Detailed proposal

This document proposes the following changes in TR 28.907 [1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.557: "Management and orchestration; Management of Non-Public Networks (NPN); Stage 1 and stage 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

[4]	3GPP TS 22.867 "Study on 5G Smart Energy and Infrastructure".

[5]	5G-ACIA: Exposure of 5G Capabilities for Connected Industries and Automation Applications, https://5g-acia.org/whitepapers/exposure-of-5g-capabilities-for-connected-industries-and-automation-applications-2/

[6]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[y]	3GPP TS 28.552: "Management and orchestration;5G performance measurements".

[z]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

2nd Change

5.4.x	Potential solutions

5.4.x.1	Introduction

This clause describes briefly the potential solution for SLA monitoring and evaluation.

5.4.x.2	Description

The high-level process for NPN SLA management in figure 5.4.x-1.

Figure 5.4.x-1: high level process for NPN SLA management

In figure 5.4.x-1, step 1-3 describes the workflows of NPN creation or update which is specified as NPN provisioning procedures in [2] in detail. The step 4, 5 and 3 are the NPN network monitoring and SLA assurance loop.

The NPN management system (e.g. NPN-SP/NPN-OP/NEP) collects performance data (e.g. packet delay in [y] and reliability in [z]) to monitor the NPN status through performance assurance MnS. Based on the network KPIs, the NPN management system evaluates the network availability and reliability to assurance the SLA fulfilment. If the SLA is not fulfilled, the NPN management system adopts some optimization methods to promote the network performance, such as SLA parameter coordination among RAN NEs, RAN resource and priority scheduling adjustment.

3rd Change

A.x	Procedure for SLA monitoring and evaluation

@startuml

"NPN-SC" --> [ 1. SLA Requirement mapping] "NPN-SP"

--> [ 2. SLS Requirement decomposition] "NPN-OP/NEP"

--> [ 3. Network creation\n/update]" NPN Network"

--> [ 4. Network monitoring\nand assurance]"NPN-OP/NEP"

--> [ 5. Network\nmonitoring]"NPN-SP"

@enduml

End of change