| 3GPP TR 28.827 V1.6.0 (2023-01) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G charging for additional roaming scenarios and actors (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Business roles	11

5	Concepts and overview	11

5.1	General	11

5.2	Home Routed	11

5.3	Local Breakout	11

5.4	Billing and Charging Evolution (BCE)	12

6	Architecture considerations	12

7	Charging scenarios and key issues	12

7.1	Charging in visited MNO for wholesale charging towards home MNO	12

7.1.1	Use cases	12

7.1.1.1	Use case #1a: Visited MNO charging home MNO for 5G data connectivity provided to the home MNO’s subscribers	12

7.1.1.2	Use case #1b: Visited MNO charging home MNO for 5G connection and mobility provided to the home MNO’s CSCs	13

7.1.1.3	Use case #1c: Visited MNO charging home MNO for SMS provided to the home MNO’s subscribers	13

7.1.2	Potential charging requirements	13

7.1.3	Key issues	13

7.1.4	Potential solutions	14

7.1.4.1	Solution #1.1: CDR in VPLMN for wholesale of 5G data connectivity	14

7.1.4.1.1	General	14

7.1.4.1.2	Reference architecture	14

7.1.4.1.3	Message flows	15

7.1.4.2	Solution #1.2: CDR in VPLMN for wholesale of 5G connection and mobility	15

7.1.4.2.1	General	15

7.1.4.2.2	Reference architecture	15

7.1.4.2.3	Message flows	16

7.1.4.3	Solution #1.3: Nchf to VPLMN for wholesale of 5G data connectivity	17

7.1.4.3.1	General	17

7.1.4.3.2	Reference architecture	17

7.1.4.3.3	Message flows	17

7.1.4.4	Solution #1.4: Roaming Charging Profile exchanged between HPLMN and VPLMN for 5G data connectivity charging	19

7.1.4.4.1	General	19

7.1.4.4.2	Reference architecture	19

7.1.4.4.3	Message flows	19

7.1.4.5	Solution #1.5: CDR in VPLMN for wholesale of SMS	20

7.1.4.5.1	General	20

7.1.4.5.2	Reference architecture	20

7.1.4.5.3	Message flows	21

7.1.4.6	Solution #1.6: Nchf to VPLMN for wholesale of 5G connection and mobility	22

7.1.4.6.1	General	22

7.1.4.6.2	Reference architecture	23

7.1.4.6.3	Message flows	23

7.1.4.7	Solution #1.7: Roaming charging trigger preconfigured in VPLMN based on negotiation with HPLMN	23

7.1.4.7.1	General	23

7.1.4.7.2	Reference architecture	23

7.1.4.7.3	Message flows	23

7.1.4.8	Solution #1.8: Only QBC trigger negotiation towards the V-CHF	24

7.1.4.8.1	General	24

7.1.4.8.2	Reference architecture	24

7.1.4.8.3	Message flows	24

7.1.4.9	Solution #1.9: V-SMF connects to V-CHF with two charging sessions	24

7.1.4.9.1	General	24

7.1.4.9.2	Reference architecture	24

7.1.4.9.3	Message flows	24

7.1.4.10	Solution #1.10: Separate QBC and FBC PDU session level triggers	26

7.1.4.10.1	General	26

7.1.4.10.2	Reference architecture	26

7.1.4.10.3	Message flows	26

7.2	Convey charging information from visited MNO to home MNO	27

7.2.1	Use cases	27

7.2.1.1	Use case #2a: Home MNO charging subscriber for 5G data connectivity at roaming using local breakout	27

7.2.1.2	Use case #2b: Home MNO charging subscriber for 5G data connectivity at roaming using home routed	27

7.2.1.3	Use case #2c: Home MNO charging subscriber for SMS provided at roaming	27

7.2.1.4	Use case #2d: Home MNO charging subscriber for 5G connection and mobility provided to home MNO’s subscribers in roaming scenario	28

7.2.2	Potential charging requirements	28

7.2.3	Key issues	28

7.2.4	Potential solutions	29

7.2.4.1 	Solution #2.1: V-CHF communicating with H-CHF for retail charging of 5G, single charging session to V-CHF	29

7.2.4.1.1	General	29

7.2.4.1.2	Reference architecture	29

7.2.4.1.3	Message flows	31

7.2.4.2 	Solution #2.2: Visited SMF (CTF) communicating with both H-CHF and V-CHF	36

7.2.4.2.1	General	36

7.2.4.2.2	Reference architecture	36

7.2.4.2.3	Message flows	37

7.2.4.3 	Solution #2.3: V-CHF communicating with H-CHF for SMS charging reconciliation between MNOs	37

7.2.4.4 	Solution #2.4: Visited SMSF (CTF) communicating with both H-CHF and V-CHF	37

7.2.4.4.1	General	37

7.2.4.4.2	Reference architecture	37

7.2.4.4.3	Message flows	38

7.2.4.5 	Solution #2.5: Visited AMF (CTF) communicating with both H-CHF and V-CHF	39

7.2.4.5.1	General	39

7.2.4.5.2	Reference architecture	39

7.2.4.5.3	Message flows	40

7.2.4.6	Solution #2.6: Reusing Nchf_ConvergedCharging service API between CHFs	40

7.2.4.6.1	General	40

7.2.4.6.2	Reference architecture	40

7.2.4.7	Solution #2.7: New Nchf service API between CHFs	40

7.2.4.7.1	General	40

7.2.4.7.2	Reference architecture	41

7.2.4.8	Solution #2.8: Using NRF to find H-CHF	41

7.2.4.8.1	General	41

7.2.4.8.3	Message flows	41

7.2.4.9	Solution #2.9: Using SUPI to find H-CHF	41

7.2.4.9.1	General	41

7.2.4.9.2	Reference architecture	41

7.2.4.9.3	Message flows	41

7.2.4.10	Solution #2.10: UE triggered V-CHF and H-CHF communication	41

7.2.4.10.1	General	41

7.2.4.10.2	Reference architecture	41

7.2.4.10.3	Message flows	41

7.2.4.11	Solution #2.11: Charging data request triggered V-CHF and H-CHF communication	42

7.2.4.11.1	General	42

7.2.4.11.2	Reference architecture	42

7.2.4.11.3	Message flows	42

7.2.4.12	Solution #2.12: Roaming charging profile update	42

7.2.4.12.1	General	42

7.2.4.12.2	Reference architecture	42

7.2.4.12.3	Message flows	42

7.2.4.13	Solution #2.13: V-CHF selected charging information for reporting to H-CHF	42

7.2.4.13.1	General	42

7.2.4.13.2	Reference architecture	42

7.2.4.13.3	Message flows	43

7.2.4.14	Solution #2.14: V-SMF selected charging information for reporting to H-CHF	43

7.2.4.14.1	General	43

7.2.4.14.2	Reference architecture	43

7.2.4.14.3	Message flows	43

7.2.4.15	Solution #2.15: V-SMF setting charging information indication by information element group	43

7.2.4.15.1	General	43

7.2.4.15.2	Reference architecture	43

7.2.4.15.3	Message flows	44

7.2.4.16	Solution #2.16: CDF and OCF communicating with SMS-SC for retail charging of SMS	44

7.2.4.16.1	General	44

7.2.4.16.2	Reference architecture	44

7.1.4.16.3	Message flows	45

7.2.4.17	Solution #2.17: configure the roaming charging profile update	45

7.2.4.17.1	General	45

7.2.4.17.2	Reference architecture	45

7.2.4.17.3	Message flows	45

7.2.5	Evaluation	46

7.3	Charging in an MNO for the purpose of wholesale charging towards an additional actor	46

7.3.1	Use cases	46

7.3.1.1	Use case #3a: MNO does wholesale charging for 5G data connectivity towards additional actors	46

7.3.1.2	Use case #3b: MNO does wholesale charging for 5G connection and mobility towards additional actors	47

7.3.1.3	Use case #3c: MNO does wholesale charging for 5G SMS towards additional actors	47

7.3.2	Potential charging requirements	47

7.3.3	Key issues	47

7.3.4	Potential solutions	48

7.3.4.1	Solution #3.1: CDR in home MNO for wholesale of non-roaming 5G of additional actor	48

7.3.4.1.1	General	48

7.3.4.1.2	Reference architecture	48

7.3.4.1.3	Message flows	49

7.3.5	Evaluation	49

7.4	Convey charging information from an MNO to an additional actor	49

7.4.1	Use cases	49

7.4.1.1	Use case #4a: Additional actor does retail charging for 5G data connectivity	49

7.4.1.2	Use case #4b: Additional actor does retail billing for 5G data connectivity	49

7.4.1.3	Use case #4c: Use case for additional actor does wholesale charging for 5G data connectivity towards another additional actor	49

7.4.2	Potential charging requirements	50

7.4.3	Key issues	50

7.4.4	Potential solutions	50

7.4.4.1	Solution #4.1: Home SMF (CTF) communicating with both H-CHF and A-CHF for retail charging of 5G data connectivity	50

7.4.4.1.1	General	50

7.4.4.1.2	Reference architecture	51

7.4.4.1.3	Potential charging flows	51

7.4.4.2	Solution #4.2: Additional actor only does retail billing of subscribers	51

7.4.4.2.1	General	51

7.4.4.2.2	Reference architecture	52

7.4.4.2.3	Potential charging flows	52

7.4.4.3 	Solution #4.3: H-CHF communicating with A-CHF for retail charging of 5G	52

7.4.4.3.1	General	52

7.4.4.3.2	Reference architecture	53

7.4.4.3.3	Message flows	53

7.4.4.4	Solution #4.4: Reusing Nchf_ConvergedCharging service API between CHFs	53

7.4.4.4.1	General	53

7.4.4.4.2	Reference architecture	54

7.4.4.5	Solution #4.5: New Nchf service API between CHFs	54

7.4.4.5.1	General	54

7.4.4.5.2	Reference architecture	54

7.4.4.6	Solution #4.6: Using NRF to find A-CHF	54

7.4.4.6.1	General	54

7.4.4.6.2	Reference architecture	54

7.4.4.6.3	Message flows	54

7.4.4.7	Solution #4.7: Using local configuration to find A-CHF	54

7.4.4.7.1	General	54

7.4.4.7.2	Reference architecture	54

7.4.4.7.3	Message flows	54

7.4.5	Evaluation	54

7.5	Convey charging information from visited MNO to home MNO, and home MNO to an additional actor	55

7.5.1	Use cases	55

7.5.1.1	Use case #5a: Use case for additional actor does retail changing for 5G data connectivity while roaming	55

7.5.2	Potential charging requirements	55

7.5.3	Key issues	55

7.5.4	Potential solutions	56

7.5.4.1	Solution #5.1: CHF to CHF communication	56

7.5.4.1.1	General	56

7.5.4.1.2	Reference architecture	56

7.5.4.1.3	Message flows	56

7.5.4.2	Solution #5.2: SMF to multiple CHF communication	57

7.5.4.2.1	General	57

7.5.4.2.2	Reference architecture	57

7.5.4.2.3	Message flows	57

7.5.5	Evaluation	57

7.6	Reconciliation of wholesale charging from the visited MNO with home MNO information	58

7.6.1	Use cases	58

7.6.1.1	Use case #6a: support for validation of usage information received from visited MNO at the home MNO	58

7.6.2	Potential charging requirements	58

7.6.3	Key issues	58

7.6.4	Potential solutions	58

7.6.4.1	Solution #6.1: Use home routing	58

7.6.4.1.1	General	58

8	Conclusions and recommendations	58

Annex A (informative): Change history	60

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies the potential use cases, requirements, and solutions for the 5G charging for additional roaming scenarios and actors, in regards to roaming local break out and home routed and taking the requirements brought by adoption of GSMA BCE (Billing Charging Evolution), as a replacement for TAP.

The study will cover and identify gaps for:

-	charging in the visited MNO network for the purpose of wholesale charging towards the home MNO

-	capabilities for reconciliation of wholesale charging reports from the visited MNO network with MNO home network information

-	charging in the visited MNO network and convey it to the home MNO network for the purpose of retail charging

-	charging in the MNO’s network for the purpose of wholesale charging towards an additional charging model actor e.g. MVNO

-	charging in the visited MNO network and convey it to the home MNO’s network for the purpose of home MNO wholesale charging towards an additional charging model actor e.g. MVNO

-	charging in the visited MNO network and convey it to an MVNO hosted by a home MNO for the purpose of retail charging

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501:"System Architecture for the 5G System".

[3]	GSMA TD.201: "Common Billing and Charging Processes ".

[4]	3GPP TS 32.255:" Charging management;5G Data connectivity domain charging; stage 2".

[5]	3GPP TS 32.256:" Charging management; 5G connection and mobility domain charging; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[7]	GSMA WA.51: "5G SA Implementation Guidelines".

[8]	3GPP TS 32.274: " Charging management; Short Message Service (SMS) charging".

[9]	3GPP TS 32.256: "Charging management; 5G connection and mobility domain charging; Stage 2".

[10]		3GPP TS 23.540: "5G System: Technical realization of Service Based Short Message Service "

[11]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[12]	3GPP TS 32.240: "Charging management; Charging architecture and principles”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCE	Billing and Charging Evolution

CHF	Charging Function

HR	Home Routed

LBO	Local Breakout

SDF	Service Data Flow

# 4	Business roles

5G roaming scenarios involves the services or capabilities provided by multiple actors and entities in the form of following business roles:

-	Home Mobile Network Operator (home MNO): This is an operator of PLMN where the MCC and MNC of the PLMN identity is same as the MCC and MNC of the UE’s SUPI, , also referred to as HPLMN.

-	Visited Mobile Network Operator (visited MNO): This is an operator of PLMN where the MCC and MNC of the PLMN identity is different from the MCC and MNC of the UE’s SUPI, also referred to as VPLMN

-	Additional actor: This is a retailer or a wholesaler of mobile services but does not own licensed radio spectrum, an additional actor that is a retailer is often referred to as Mobile Virtual Network Operator (MVNO).

In deployments, there could be scenarios where one or more of the business roles are supported by a single enterprise, i.e., one enterprise may play a single business role or multiple business roles. The present document does not impose any restrictions to the possible deployment scenarios.

There could be various business models in terms of charging for the network services provided and between the entities being served, for example (but not limited to):

-	Home MNO charges the subscribers for usage of network services in visited MNO.

-	Home MNO charges the MVNO for its subscribers’ network service usage in home MNO, or network service usage in visited MNO

-	MVNO charges the subscribers for usage of network services in visited MNO and/or home MNO.

-	Visited MNO charges home MNO for its subscribers’ usage of network services in visited MNO.

The solutions described in the present TR aim to flexibly support the charging scenarios without restriction to any specific business model.

# 5	Concepts and overview

## 5.1	General

The 5G charging for additional roaming scenarios and actors includes the following aspects:

-	HR Roaming (Home Routed), refer to TS 23.501 [2];

-	LBO Roaming (Local Breakout), refer to TS 23.501 [2];

-	BCE (Billing and Charging Evolution), refer to GSMA TD 201 [3].

## 5.2	Home Routed

The 5G System roaming architecture in the case of home routed scenario with service-based interfaces within the Control Plane, as defined by TS 23.501[2] Figure 4.2.4-3.

The 5G System roaming architecture in the case of home routed scenario using the reference point representation, as defined by TS 23.501[2] Figure 4.2.4-6.

## 5.3	Local Breakout

The 5G System roaming architecture with local breakout with service-based interfaces within the Control Plane as defined by TS 23.501[2] Figure 4.2.4-1.

The 5G System roaming architecture in the case of local break out scenario using the reference point representation, as specified in TS 23.501 [2] Figure 4.2.4-4.

## 5.4	Billing and Charging Evolution (BCE)

The BCE process is a simplified and flexible optional settlement method, tailor made for future wholesale roaming settlement needs of operators. The process is driven by new formats like Usage Data Report, Billing Statement Report and Detailed Data Record which together build a robust framework for seamless revenue assurance.

The BCE process is specified in GSMA TD.201 [3] clause 2.2.

The BCE and TAP (Transferred Account Procedures) process will continue to work in parallel for the foreseeable future.

According to GSMA WA.51 [7]. clause 12, there is no support for 5G SA Wholesale charging using the legacy TAP process. The operators must support BCE in order to launch 5G SA roaming and inter-PLMN charging. This document only considers the requirements from BCE.

# 6	Architecture considerations

Editor’s Note: the charging architecture for additional roaming scenarios and actors is FFS along with the related key issues.

# 7	Charging scenarios and key issues

## 7.1	Charging in visited MNO for wholesale charging towards home MNO

### 7.1.1	Use cases

#### 7.1.1.1	Use case #1a: Visited MNO charging home MNO for 5G data connectivity provided to the home MNO’s subscribers

This use case focuses on visited MNO and home MNO business roles.

A CSC (UE) has a subscription with the home MNO which allows usage of 5G data connectivity.

The visited MNO to home MNO charging could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	Rating Group

-	Service ID

-	Location

-	QoS information.

Editor’s Note: the list of the above aspects is ffs and should be check whether in the BCE.

This use case is applicable for the Home Routed Roaming and Local Breakout Roaming.

For the Home Routed Roaming using QBC, the visited MNO to home MNO charging could be based on "Roaming Charging Profile", which may be negotiated between the VPLMN and the HPLMN.

#### 7.1.1.2	Use case #1b: Visited MNO charging home MNO for 5G connection and mobility provided to the home MNO’s CSCs

This use case focuses on visited MNO and home MNO business roles.

A CSC (UE) has a subscription with the home MNO which allows usage of 5G data connectivity connection and mobility.

The visited MNO to home MNO charging is based on number of home MNO’s UEs (i.e. unique SUPI) connected to the visited MNO’s network over a specific period of time.

#### 7.1.1.3	Use case #1c: Visited MNO charging home MNO for SMS provided to the home MNO’s subscribers

This use case focuses on visited MNO and home MNO business roles.

A CCS (UE) has a subscription with the home MNO which allows usage of SMS over NAS in 5GS.

The visited MNO to home MNO charging could be based on number of SMS sent or received by the home MNO UEs served by the visited MNO’s network considering of the following aspects:

-	RAT type;

-	Location

### 7.1.2	Potential charging requirements

REQ-CH_VMNO-01: The charging mechanism in visited MNO should support charging based on the 5G data connectivity usage for each home MNO.

REQ-CH_VMNO-02: The charging mechanism in visited MNO should support collecting charging information related to 5G data connectivity usage for each home MNO.

REQ-CH_VMNO-03: The charging mechanism in visited MNO should support charging based on the 5G connection and mobility usage for each home MNO.

REQ-CH_VMNO-04: The charging mechanism in visited MNO should support collecting charging information related to 5G connection and mobility usage for each home MNO.

REQ-CH_VMNO-05: The charging mechanism in visited MNO may support roaming charging profile negotiation related to 5G data connectivity charging with each home MNO.

REQ-CH_VMNO-06: The charging mechanism in visited MNO should support charging based on SMS usage for each home MNO.

REQ-CH_VMNO-07: The charging mechanism in visited MNO should support collecting charging information related to SMS usage for each home MNO.

REQ-CH_VMNO-08: The charging triggers to be used in visited MNO may be negotiation with each home MNO.

### 7.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: Aggregation of charging information in visited MNO for 5G data connectivity usage per home MNO.

-	Key Issue #1b: Aggregation of charging information in visited MNO for 5G connection and mobility usage per home MNO.

-	Key Issue #1c: Aggregation of charging information in visited MNO for SMS usage per home MNO.

-	Key Issue #1d: Whether the FBC should be negotiated between visited MNO and home MNO. If needed, how to provide the negotiation mechanism for FBC triggers synchronization, including the charging information and triggers.

-	Key Issue #1e: How to handle QoS flow level triggers in relationship to QFIs.

-	Key Issue #1f: How to handle PDU session level triggers when both FBC and QBC are applicable.

-	Key Issue #1g: How to handle charging trigger negotiation between visited and home MNO.

### 7.1.4	Potential solutions

#### 7.1.4.1	Solution #1.1: CDR in VPLMN for wholesale of 5G data connectivity

##### 7.1.4.1.1	General

A possible solution for key issue 1a and #1g covering requirements REQ-CH_VMNO-01 and REQ-CH_VMNO-02, wholesale charging for 5G data connectivity provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

##### 7.1.4.1.2	Reference architecture

Figure 7.1.4.1.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.1.2-2: Roaming 5G data connectivity scenario in reference point representation

The SMF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may send this for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the amount of data per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.1.3	Message flows

For both home routed roaming and LBO, the flows for 5G data connectivity charging would be the same as in clause 5.2.2.12 of TS 32.255 [4] with the difference that for the LBO case would be that there would be no interaction with the home PLMN’s UPF, SMF and CHF.

#### 7.1.4.2	Solution #1.2: CDR in VPLMN for wholesale of 5G connection and mobility

##### 7.1.4.2.1	General

A possible solution for key issues 1b covering requirements REQ-CH_VMNO-03 and REQ-CH_VMNO-04, wholesale charging for 5G connection and mobility provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

##### 7.1.4.2.2	Reference architecture

Figure 7.1.4.2.2-1: Roaming 5G connection and mobility scenario in service-based interface representation

Figure 7.1.4.2.2-2: Roaming 5G connection and mobility scenario in reference point representation

The AMF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may be send for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the number of connected UEs per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.2.3	Message flows

The flows for 5G connection and mobility charging would be the same as in clause 5.2.2.5 of TS 32.256 [5], with the exception that the there is no need to interact with the home PLMN’s CHF and that only PEC is required. The CDRs generated in the CHF in VPLMN would in this case be used as input to the wholesale charging of the HPLMN. The following figure 7.1.4.2.2-1 describes a Registration charging message flow in PEC in VPLMN scenario for roaming, based on figure 4.2.2.2.2-1 of TS 23.502 [6] description

Figure 7.1.4.2.3-1: Roaming Registration – PEC in VPLMN

1-20a.	Registration procedure initiated by UE per steps 1 to 20a of figure 4.2.2.2.2-1 of TS 23.502 [6], with PCF, AUSF and UDM in HPLMN.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a. The AMF in VPLMN sends Charging Data Request [Event] to the CHF in VPLMN for the UE successful registration, indicating "roamer in".

21ch-b. The CHF in VPLMN creates the CDR for this registration.

21ch-c. The CHF in VPLMN acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21b to 24. Same steps as in figure 4.2.2.2.2-1 of TS 23.502 [6].

#### 7.1.4.3	Solution #1.3: Nchf to VPLMN for wholesale of 5G data connectivity

##### 7.1.4.3.1	General

A possible solution for key issue #1a covering requirements REQ-CH_VMNO-01 and REQ-CH_VMNO-02, wholesale charging for 5G data connectivity provided to the home MNO by the visited MNO, covering local breakout case. In this case there is also charging information generated in the home MNO for retail purposes.

##### 7.1.4.3.2	Reference architecture

Figure 7.1.4.3.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.3.2-2: Roaming 5G data connectivity scenario in reference point representation

##### 7.1.4.3.3	Message flows

For local breakout roaming, the flows for 5G data connectivity charging are present as the following figure 7.1.4.3.3-1.

Figure 7.1.4.3.3-1: Local breakout roaming for 5G data connectivity charging

1-3a.	UE initiates a new PDU session. V-SMF selection by the AMF.

4ch-a. The UE is identified as a roamer (PLMN ID of the received SUPI is different from VPLMN PLMN ID), the V-CHF is selected accordingly. A Charging Data Request [Initial, Roaming Charging profile] is sent to V-CHF, indicating "in-bound roamer".

4ch-b. The V-CHF opens a CDR (indicating "in-bound roamer")

4ch-c. The V-CHF acknowledges by sending Charging Data Response [Initial, Roaming Charging profile] to the V-SMF.

7ch-a. A Charging Data Request [Update] is sent to V-CHF with charging information received from V-SMF.

7ch-b. The V-CHF updates the CDR.

7ch-c. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

10ch-a. A Charging Data Request [Termination] is sent to V-CHF.

10ch-b. The V-CHF closes the CDR

10ch-c. The V-CHF acknowledges by sending Charging Data Response [Termination] to the V-SMF.

Step 5, Step 8 and Step 11 are detailed in Figure 7.2.4.2.1-2 clause 7.2.

#### 7.1.4.4	Solution #1.4: Roaming Charging Profile exchanged between HPLMN and VPLMN for 5G data connectivity charging

##### 7.1.4.4.1	General

A possible solution for key issue #1a covering requirement REQ-CH_VMNO-05, Roaming Charging Profile negotiation enhancement for 5G data connectivity charging provided to the home MNO by the visited MNO.

##### 7.1.4.4.2	Reference architecture

The reference architecture would be the same as in clause 4.2.2 of TS 32.255 [4].

##### 7.1.4.4.3	Message flows

The message flows about roaming charging profile negotiation for 5G data connectivity charging are present in the figure 5.2.2.12.2.1, PDU session establishment, of TS 32.255 [4].

In the step 13,

13ch-a. Based on "received PDU session conditions" trigger, a Charging Data Request [Update] is sent to CHF with charging information received from H-SMF, which includes the HPLMN selected "Roaming Charging Profile" and counts per QFI are started.

13ch-b. The CHF updates the CDR.

13ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

During PDU session establishment, for Home Routed Roaming using QBC, the roaming charging profile is sent from the V-CHF to the V-SMF which is then forwarded to the H-CHF (using the H-SMF). The H-CHF may respond with a new roaming charging profile. The H-CHF provided roaming charging profile is then used by the V-SMF (as well as the H-SMF) for the QBC, the new charging roaming profile is also sent to the V-CHF from V-SMF. The corresponding trigger "received PDU session conditions" in the step 13ch-a should be clarified.

The message flows about roaming charging profile negotiation for Roaming Home routed PDU session - inter-PLMN V-SMF change, specified in the figure 5.2.2.14.9.1, of TS 32.255[4],

In the step 8ach-a,

8ach-a. A Charging Data Request [Update] is sent to H-CHF in HPLMN, indicating the V-SMF change and "Roaming Charging Profile" received from the VPLMN.

8ach-b. The H-CHF in HPLMN updates the CDR.

8ach-c. The H-CHF in HPLMN acknowledges by sending Charging Data Response [Update] to the H-SMF and supplies the HPLMN selected "Roaming Charging Profile" for this VPLMN.

In the step 8cch-a,

8cch-a. A Charging Data Request [Update] is sent to V-CHF with charging information received from H-SMF, which includes the HPLMN selected "Roaming Charging Profile".

8cch-b. The V-CHF updates the CDR.

8cch-c. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

How to report the updated "Roaming Charging Profile" to V-CHF should be clarified without the addition of a special trigger. The updated "Roaming Charging Profile" will be reported in the next Charging Data Request [Update] which is armed by subsequent triggers.

The message flows about roaming charging profile negotiation for roaming Home routed PDU session - intra-PLMN V-SMF change are present in the clause 5.2.2.14.10 of TS 32.255[4].

The Roaming Charging Profile is transferred from old V-SMF to the new V-SMF.

3- 4b.	SM Context retrieval by new V-SMF from the old V-SMF. The Charging Id containing Charging Id1 value, the "Home Roaming Charging Profile", and the V-CHF address, are conveyed to the new V-SMF.

No enhancement is required. The Roaming Charging Profile is NOT negotiated again.

#### 7.1.4.5	Solution #1.5: CDR in VPLMN for wholesale of SMS

##### 7.1.4.5.1	General

A possible solution for key issue 1c covering requirements REQ-CH_VMNO-06 and REQ-CH_VMNO-07, wholesale charging for SMS provided to the home MNO by the visited MNO.

##### 7.1.4.5.2	Reference architecture

Figure 7.1.4.5.2-1: Roaming SMS in service-based interface representation

Figure 7.1.4.5.2-2: Roaming SMS in reference point representation

The SMSF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may be sent for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the amount of SMSs per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.5.3	Message flows

Figure 7.1.4.5.3-1 shows a scenario for Home MNO UE SMS submission in visited MNO SMSF for PEC mode, based on clause 5.4.2.5 of TS 32.274 [8]

The CDRs generated in the CHF in VPLMN are used as input for wholesale charging of the HPLMN.

Figure 7.1.4.5.3-1: Roaming SMS submission to SMSF – PEC in VPLMN

1- 3.	Initial procedures as per clause 5.4.2.5 of TS 32.274 [8].

4. The SMSF in VPLMN sends Charging Data Request [Event] to CHF in VPLMN for the SMS indicating "roamer in".

5. The CHF in VPLMN creates a CDR for this SMS.

6.  The CHF in VPLMN acknowledges by sending Charging Data Response [Event] to the SMSF in VPLMN.

7.	Forward SMS as per clause 5.4.2.5 of TS 32.274 [8].

#### 7.1.4.6	Solution #1.6: Nchf to VPLMN for wholesale of 5G connection and mobility

##### 7.1.4.6.1	General

A possible solution for key issue #1b covering requirements REQ-CH_VMNO-03 and REQ-CH_VMNO-04, wholesale charging for 5G connection and mobility provided to the home MNO by the visited MNO. In this case there is also charging information generated in the home MNO for retail purposes, studied in clause 7.2.

##### 7.1.4.6.2	Reference architecture

Figure 7.1.4.6.2-1: Roaming 5G connection and mobility scenario in service-based interface representation

Figure 7.1.4.6.2-2: Roaming 5G connection and mobility scenario in reference point representation

##### 7.1.4.6.3	Message flows

The flows for 5G connection and mobility charging would be the same as in clause 5.2.2.5 of TS 32.256 [5],

#### 7.1.4.7	Solution #1.7: Roaming charging trigger preconfigured in VPLMN based on negotiation with HPLMN

##### 7.1.4.7.1	General

A possible solution for key issue #1g covering requirements REQ-CH_VMNO-05 and REQ-CH_VMNO-08, using preconfigured roaming charging triggers.

##### 7.1.4.7.2	Reference architecture

The reference architecture would be the same as in clauses7.1.4.1.2, 7.1.4.2.2, and 7.1.4.5.2.

##### 7.1.4.7.3	Message flows

The negotiation of how the charging triggers should be configured is done as part of the roaming agreement between the VPLMN and HPLMN outside the scope of 3GPP SA5.

The negotiated roaming charging triggers are configured in the VPLMN’s V-CHF, V-SMF, AMF, and SMSF. These preconfigured charging triggers could be specific to the HPLMN of the UE. In SMF case the roaming charging triggers would only be exchanged between the V-SMF and V-CHF.

#### 7.1.4.8	Solution #1.8: Only QBC trigger negotiation towards the V-CHF

##### 7.1.4.8.1	General

A possible solution for key issue #1d covering requirements REQ-CH_VMNO-01, and REQ-CH_VMNO -02, about the FBC negotiation between visited MNO and home MNO.

The FBC (Flow Based Charging) is not used for wholesale and settlement between visited MNO and home MNO. The triggers of FBC are not used for negotiation between visited MNO and home MNO. The V-CHF sets the FBC triggers independent of HPLMN.

##### 7.1.4.8.2	Reference architecture

The reference architecture would be the same as in solution #1.3 clause 7.1.4.3.2.

##### 7.1.4.8.3	Message flows

The detailed message flows would be the same as in clause 5.2.2.18.2 of TS 32.255 [4]. V-CHF receives charging information based on FBC triggers and generates records. There is no negotiation of FBC triggers between visited MNO and Home MNO.

#### 7.1.4.9	Solution #1.9: V-SMF connects to V-CHF with two charging sessions

##### 7.1.4.9.1	General

A possible solution for key issue #1a, covering requirements REQ-CH_VMNO-01, and REQ-CH_VMNO-02, the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF), From the standpoint of the V-SMF, there are two charging sessions between V-SMF and V-CHF, covering local breakout case.

##### 7.1.4.9.2	Reference architecture

The reference architecture would be the same as Figure 7.2.4.1.2-2.

##### 7.1.4.9.3	Message flows

For the V-SMF communicating with the H-CHF, the V-CHF is responsible for sending the charging messages between V-SMF and H-CHF.

Figure 7.1.4.9.3-1: Message flow for two charging sessions

During the PDU session establishment, V-SMF establishes the two charging sessions in the separate charging resource with V-CHF.

In the interaction between the V-SMF and V-CHF, V-SMF reports the charging information to V-CHF.

In the interaction between the V-SMF and H-CHF, V-SMF reports the charging information to the V-CHF in the charging session between the V-SMF and V-CHF, V-CHF sends the charging information to H-CHF in the charging session between the H-CHF and V-CHF. For QBC, the triggers and charging information are same for both charging sessions.

For FBC, trigger does not have synchronization mechanism, i.e. for V-CHF and H-CHF, it’s possible that the FBC charging information and trigger are not same for the two charging sessions.

#### 7.1.4.10	Solution #1.10: Separate QBC and FBC PDU session level triggers

##### 7.1.4.10.1	General

A possible solution for key issue #1f covering requirements REQ-CH_VMNO-01, and REQ-CH_VMNO -02, about the PDU Session level setting for QBC triggers.

The QBC triggers and FBC triggers have separate PDU session level triggers i.e., there will be QBC specific PDU session triggers and FBC specific PDU session triggers.

##### 7.1.4.10.2	Reference architecture

The reference architecture would be the same as the local breakout architecture in solution #1.3 clause 7.1.4.3.2, or home routed architecture in Figure 4.2.4 of TS 32.255.

##### 7.1.4.10.3	Message flows

For home routed scenario, the message flow is same with the PDU session establishment message flow in clause 5.2.2.12.2 of TS 32.255 [4]. In step 11ch-d of Figure 5.2.2.12.2.1 in TS 32.255 [4], the H-CHF acknowledges by sending Charging Data Response to the H-SMF and optionally includes a "Roaming Charging Profile". The "Roaming Charging Profile" may include both QoS flow and PDU session level triggers (e.g. , both QoS change and user location change) for QBC, for FBC both rating group and PDU session level triggers may be included in the trigger elements of Charging Data Response.

For local breakout scenario, the message flow is same with the PDU session establishment message flow in clause 5.2.2.18.2 of TS 32.255 [4]. In step 9ch-d2 of Figure 5.2.2.18.2-1 in TS 32.255 [4], the H-CHF acknowledges by sending Charging Data Response to the V-SMF and optionally includes a "Roaming Charging Profile". The "Roaming Charging Profile" may include both QoS flow and PDU session level triggers (e.g. , both QoS change and user location change) for QBC, for FBC both rating group and PDU session level triggers may be included in the trigger element of Charging Data Response.

This means that the values and status of PDU session level triggers may be different for QBC and FBC. For example, a PDU Session level "Expiry of data time limit per PDU session" for FBC is set to 600 seconds while it for QBC is set to 900 seconds, this means that after 600 seconds all the rating groups shall be reported but no QoS flows, then after another 300 seconds all QoS flows shall be reported but no rating groups. While updates can in some cases coincide, it would be difficult to sort out the reason for and order of the trigger and would therefore preferably always be separate for FBC and QBC.

#### 7.1.4.11	Solution #1.11: QoS flow level trigger Setting during PDU session

##### 7.1.4.11.1	General

A possible solution for key issue #1e covering requirements REQ-CH_VMNO-01, and REQ-CH_VMNO -02, about the QoS flow level trigger setting during the PDU session.

The QBC triggers mechanism is specified in the clause 5.2.1.6 TS 32.255 [4] and Roaming Charging Profile is specified in clause 5.2.1.7 of TS 32.255 [4]. The QBC triggers include PDU session level triggers and QoS Flow level triggers.

##### 7.1.4.11.2	Reference architecture

The reference architecture would be the same as the local breakout architecture in solution #1.3 clause 7.1.4.3.2 or Home Routed architecture in Figure 4.2.4 of TS 32.255.

##### 7.1.4.11.3	Message flows

The message flow is same with the PDU session establishment message flow in clause 5.2.2.12.2 or 5.2.2.18.2 of TS 32.255 [4]. In a Roaming Charging Profile negotiation, the QBC triggers are common for all QFIs.

## 7.2	Convey charging information from visited MNO to home MNO

### 7.2.1	Use cases

#### 7.2.1.1	Use case #2a: Home MNO charging subscriber for 5G data connectivity at roaming using local breakout

This use case focuses on home MNO and subscriber business roles.

A subscriber has a UE and a subscription with the home MNO which allows usage of 5G data connectivity while roaming. The subscriber is roaming in a visited network and is allowed to use local breakout.

The home MNO charging of the subscriber could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	Visited network

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

#### 7.2.1.2	Use case #2b: Home MNO charging subscriber for 5G data connectivity at roaming using home routed

This use case focuses on home MNO and subscriber business roles.

A subscriber has a UE and a subscription with the home MNO which allows usage of 5G data connectivity while roaming. The subscriber is roaming in a visited network and the connection is home routed.

The home MNO charging of the subscriber could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	Visited network

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

#### 7.2.1.3	Use case #2c: Home MNO charging subscriber for SMS provided at roaming

This use case focuses on visited MNO and home MNO business roles.

The visited MNO to home MNO charging is based on number of SMS sent or received by the home MNO UEs served by the visited MNO’s network considering of the following aspects:

-	RAT type;

-	Location

Number of SMS sent or received under the above aspects needs to be reconciliated between home MNO and visited MNO.

#### 7.2.1.4	Use case #2d: Home MNO charging subscriber for 5G connection and mobility provided to home MNO’s subscribers in roaming scenario

This use case focuses on visited MNO and home MNO business roles.

The visited MNO to home MNO charging is based on connection and mobility sent or received by the home MNO UEs served by the visited MNO’s network considering of the following aspects:

-	RAT type;

-	Location

### 7.2.2	Potential charging requirements

REQ-CH_CVTOH-01: The charging mechanism in visited MNO should support conveying charging information for the 5G data connectivity usage to the home MNO for each UE.

REQ-CH_ CVTOH-02: The charging mechanism in visited MNO should support collecting charging information related to 5G data connectivity usage for each UE.

REQ-CH_ CVTOH-03: The charging mechanism in visited MNO should support conveying charging information for SMS usage to the home MNO for each UE.

REQ-CH_ CVTOH-04: The charging mechanism in visited MNO should support collecting charging information related to SMS usage for each UE.

REQ-CH_ CVTOH-05: The charging mechanism in visited MNO should support conveying charging information for 5G connection and mobility to the home MNO for each UE.

REQ-CH_ CVTOH-06: The charging mechanism in visited MNO should support collecting charging information related to 5G connection and mobility for each UE.

### 7.2.3	Key issues

The following key issues are identified:

-	Key Issue #2a: Collection of charging mechanism in visited MNO about the 5G data connectivity usage.

-	Key Issue #2b: Conveying the charging information to the home MNO for 5G data connectivity usage.

- 	Key Issue #2c: Finding the correct CHF

-	Key Issue #2d: Service based interface to use between visited CHF and home CHF

-	Key Issue #2e: Trigger handling between visited CHF and home CHF for 5G data connectivity usage.

-	Key Issue #2f: Collection of charging mechanism in visited MNO for SMS usage.

-	Key Issue #2g: Conveying the charging information to the home MNO for SMS usage

-	Key Issue #2h: Collection of charging mechanism in visited MNO for 5G connection and mobility.

-	Key Issue #2i: Conveying the charging information to the home MNO for 5G connection and mobility.

### 7.2.4	Potential solutions

#### 7.2.4.1 	Solution #2.1: V-CHF communicating with H-CHF for retail charging of 5G, single charging session to V-CHF

##### 7.2.4.1.1	General

A possible solution for key issues #2a, #2b, #2f, #2g, #2h and #2i covering requirements REQ-CH_CVTOH-01, REQ-CH_CVTOH-02, REQ-CH_CVTOH-03, REQ-CH_CVTOH-04, REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06, retail charging for 5G data connectivity provided, 5G connection and mobility, and SMS usage to the home MNO’s users by the visited MNO, in the case of local breakout.

There is a single charging session between NF (e.g. V-SMF) and V-CHF. For the NF communicating with the H-CHF, the V-CHF is responsible for generating the charging information to H-CHF and handling its own charging information.

There is a one-to-one relationship between the PDU session and the charging session towards the V-CHF.

##### 7.2.4.1.2	Reference architecture

Figure 7.2.4.1.2-1: Roaming 5G scenario in service-based interface representation

The visited CHF does converged charging for interconnect (wholesale), while the home CHF does converged charging for the subscriber (retail).

Figure 7.2.4.1.2-2: Roaming 5G scenario in reference point representation

##### 7.2.4.1.3	Message flows

Figure 7.2.4.1.3-1 shows a scenario for Session Based Charging (SBC) with CHF-to-CHF communication. Applicable for SMF, without update also applicable for SMSF and AMF. During the PDU session establishment, NF (e.g. V-SMF) establishes the single charging session for reporting charging information to V-CHF and H-CHF.

Figure 7.2.4.1.3-1: SBC - Session based charging with CHF-to-CHF communication

1)	Request for service delivery: A request for session establishment is received in NF (CTF). The service is configured to be authorized by V-CHF before start.

2)	Units determination: NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial, Quota requested]: NF (CTF) sends an initial charging data request to V-CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Determine H-CHF: V-CHF creates a charging resource for the NF (CTF), determines that it need to interact with H-CHF and which H-CHF it should contact and generates an initial charging home request to H-CHF.

5)	Charging home request [Initial, Quota requested]: V-CHF sends the request to H-CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

6)	Account, Rating, Reservation control: H-CHF rates the charging home requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, H-CHF performs the corresponding reservations.

7)	Open CDR: based on policies, H-CHF opens a CDR related to the service.

8)	Charging home response [Initial, Quota granted]: H-CHF grants authorization to V-CHF for the service to start, with the reserved number of units.

9)	Open CDR: based on policies, V-CHF opens a CDR related to the service.

10)	Charging data response [Initial, Quota granted]: V-CHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

11)	Granted units supervision: NF (CTF) monitors the consumption of the granted units.

12)	Start of service delivery: NF (CTF) starts to deliver the content/service based on the reserved number of units.

13)	Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

14)	Charging data request [Update, Unit used, Quota requested]: NF (CTF) sends an intermediate charging data request to V-CHF, for more units to be granted for the service to continue and reporting the used units.

15)	Charging home request [Update, Unit used, Quota requested]: V-CHF sends an intermediate charging home request to H-CHF, for more units to be granted for the service to continue and reporting the used units.

16)	Account, Rating, Reservation control: H-CHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

17)	 Update CDR: based on policies, H-CHF updates the CDR with charging data related to the service.

18)	Charging home response [Update, Quota granted]: H-CHF grants quota to V-CHF for the service to continue, with the reserved number of units.

19)	 Update CDR: based on policies, V-CHF updates the CDR with charging data related to the service.

20)	Charging data response [Update, Quota granted]: V-CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

21)	Granted units supervision: NF (CTF) monitors the consumption of the granted units.

22)	Service delivery ongoing: NF (CTF) continues the service delivery based on the granted quota.

23)	Service released: the service is terminated or released.

24)	Charging data request [Termination, Unit used]: NF (CTF) sends a terminate charging data request to the V-CHF, for charging data related to the service termination with the final consumed units.

25)	Charging home request [Termination, Unit used]: V-CHF sends a terminate charging home request to the H-CHF, for charging data related to the service termination with the final consumed units.

26)	Account, Rating control: H-CHF performs the service termination process involving rating entity and user's account balance.

27)	 Close CDR: based on policies, H-CHF closes the CDR with charging data related to the service termination and the last reported units.

28)	Charging home response [Termination]: H-CHF informs the V-CHF on the result of the terminate charging home request.

29)	 Close CDR: based on policies, V-CHF closes the CDR with charging data related to the service termination and the last reported units.

30)	Charging data response [Termination]: V-CHF informs the NF (CTF) on the result of the terminate charging data request.

Figure 7.2.4.1.3-2 shows a scenario for Immediate Event Charging (IEC) with CHF-to-CHF communication. Applicable for SMSF and AMF. During the PDU session establishment, NF (e.g. V-SMF) establishes the single charging session for reporting charging information to V-CHF and H-CHF.

Figure 7.2.4.1.3-2: IEC - Immediate event charging with CHF-to-CHF communication

1)	Request for content/service delivery: A request for session establishment is received in NF (CTF). The service is configured to be authorized by V-CHF before start.

2)	Units determination: NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Event, Units]: NF (CTF) sends the request to V-CHF for the service to be granted authorization to start, and to allow the number of units if determined in item 2.

4)	Determine H-CHF: V-CHF creates a charging resource for NF (CTF), determines that it need to interact with H-CHF and which H-CHF it should contact, and generates a charging home request to H-CHF.

5)	Charging home request [Event, Units]: V-CHF sends the request to H-CHF for the service to be granted authorization to start, and to allow the number of units if determined in step 2.

6)	Account, Rating control: H-CHF rates the charging home requests and checks if corresponding funds are available on the user's account balance. If the account has sufficient funds, H-CHF performs the corresponding reservations.

7)	Create CDR: based on policies, H-CHF creates a CDR related to the service.

8)	Charging home response [Event, Units]: H-CHF grants authorization to V-CHF for the service to start, with the allowed number of units.

9)	Create CDR: based on policies, V-CHF creates a CDR related to the service.

10)	Charging data response [Event, Units]: V-CHF grants authorization to NF (CTF) for the service to start, with the allowed number of units.

11)	Granted units supervision: NF (CTF) monitors the consumption of the allowed units.

12)	Content/service delivery: NF (CTF) starts to deliver the content/service based on the allowed number of units.

Figure 7.2.4.1.3-3 shows a scenario for Post Event Charging (PEC) with CHF-to-CHF communication. Applicable for SMSF and AMF. During the PDU session establishment, NF (e.g., V-SMF) establishes the single charging session for reporting charging information to V-CHF and H-CHF.

Figure 7.2.4.1.3-3: PEC - Post event charging with CHF-to-CHF communication

1)	Request for content/service delivery: A request for session establishment is received in NF (CTF). The service is configured not to be authorized by CHF before start.

2)	Content/service delivery: NF (CTF) starts to deliver the content/service.

3)	Charging data request [Event, Units]: NF (CTF) sends a charging data request to V-CHF with the units used for recording, and possibly accounting and rating control.

4)	Determine H-CHF: V-CHF creates a charging resource for NF (CTF), determines that it need to interact with H-CHF and which H-CHF it should contact, and generates a charging home request to H-CHF.

5)	Charging home request [Event, Units]: V-CHF sends the request to H-CHF with the units used for recording, and possibly accounting and rating control.

6)	Account, Rating control: H-CHF may rate the charging home request based on the number of units used and update the user's account balance.

7)	Create CDR: based on policies, H-CHF creates a CDR related to the service.

8)	Charging home response [Event]: H-CHF informs V-CHF on the result of the charging home request.

9)	Create CDR: based on policies, V-CHF creates a CDR related to the service.

10)	Charging data response [Event]: V-CHF informs NF (CTF) on the result of the charging data request.

#### 7.2.4.2 	Solution #2.2: Visited SMF (CTF) communicating with both H-CHF and V-CHF

##### 7.2.4.2.1	General

A possible solution for key issues #2a and #2b covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02, retail charging for 5G data connectivity provided to the home MNO’s user by the visited MNO. Roaming 5G data connectivity scenario in service-based interface representation is the same with Figure 7.2.4.1.1-1.

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

##### 7.2.4.2.2	Reference architecture

Figure 7.2.4.2.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.2.4.2.2-2: Roaming 5G data connectivity scenario in reference point representation

##### 7.2.4.2.3	Message flows

This would be same as for solution #1.3 in clause 7.1.4.3.

#### 7.2.4.3 	Solution #2.3: V-CHF communicating with H-CHF for SMS charging reconciliation between MNOs

Covered by clause 7.2.4.1.

#### 7.2.4.4 	Solution #2.4: Visited SMSF (CTF) communicating with both H-CHF and V-CHF

##### 7.2.4.4.1	General

A possible solution for key issues #2f and #2g covering requirements REQ-CH_CVTOH-03, and REQ-CH_CVTOH-04, charging for SMS provided to the home MNO’s users by the visited MNO, in roaming scenario.

The visited CHF performs converged charging for interconnect, while the home CHF performs converged charging for reconciliation.

##### 7.2.4.4.2	Reference architecture

The Roaming SMS architecture in service-based interface representation is the same as in Figure 7.2.4.4.2-1

Figure 7.2.4.4.2-1: Roaming SMS in reference point representation

##### 7.2.4.4.3	Message flows

Figure 7.2.4.4.3-1 shows a scenario for SMS PEC with CTF to both CHFs communication.

Figure 7.2.4.4.3-1: Roaming PEC SMS - two CHFs communication

1- 3.	Initial procedures as per clause 5.4.2.5 of TS 32.274 [8].

4. The SMSF in VPLMN sends Charging Data Request [Event] to H-CHF for the SMS.

5. The H-CHF creates a CDR for this SMS.

6.  The H-CHF acknowledges by sending Charging Data Response[Event] to the SMSF in VPLMN.

7. The SMSF in VPLMN sends Charging Data Request [Event] to V-CHF for the SMS.

8. The V-CHF creates a CDR for this SMS.

9.  The V-CHF acknowledges by sending Charging Data Response[Event] to the SMSF in VPLMN.

10.	Forward SMS as per clause 5.4.2.5 of TS 32.274 [8].

#### 7.2.4.5 	Solution #2.5: Visited AMF (CTF) communicating with both H-CHF and V-CHF

##### 7.2.4.5.1	General

A possible solution for key issues #2h and #2i covering requirements REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06, retail charging for 5G connection and mobility provided to the home MNO’s user by the visited MNO.

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

##### 7.2.4.5.2	Reference architecture

Figure 7.2.4.5.2-1: Roaming 5G connection and mobility scenario in service-based interface representation

Figure 7.2.4.5.2-2: Roaming 5G connection and mobility scenario in reference point representation

##### 7.2.4.5.3	Message flows

This would be same as TS 32.256 [9] clause 5.2.2.5.

#### 7.2.4.6	Solution #2.6: Reusing Nchf_ConvergedCharging service API between CHFs

##### 7.2.4.6.1	General

A possible solution for key issue #2d covering requirements REQ-CH_CVTOH-01, REQ-CH_CVTOH-02, REQ-CH_CVTOH-03, REQ-CH_CVTOH-04, REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06, service based interface to use between visited CHF and home CHF, would be to reuse the Nchf_ConvergedCharging service API. This would mean that the V-CHF would proxy the request from the AMF, SMF or SMSF to the H-CHF, and the same with the response. The V-CHF could do some changes to message like: filter (e.g. trigger), enrich, or convert (e.g., rating groups).

##### 7.2.4.6.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

#### 7.2.4.7	Solution #2.7: New Nchf service API between CHFs

##### 7.2.4.7.1	General

A possible solution for key issue #2d covering requirements REQ-CH_CVTOH-01, REQ-CH_CVTOH-02, REQ-CH_CVTOH-03, REQ-CH_CVTOH-04, REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06, service based interface to use between visited CHF and home CHF, would be to create a new service API. This would mean that the V-CHF would translate the request from the AMF, SMF or SMSF to the new service API towards the H-CHF. This would mean that the message could look completely different and even the triggers in the V-CHF for sending the message towards the H-CHF wouldn’t have to be related to the request from AMF, SMF or SMSF.

##### 7.2.4.7.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

#### 7.2.4.8	Solution #2.8: Using NRF to find H-CHF

##### 7.2.4.8.1	General

A possible solution for key issue #2c, finding the correct CHF for solutions #2.1 and #2.3 where V-CHF communicating with H-CHF.

Editor’s Note: The solution #2.1 where V-CHF communicate with H-CHF is FFS.

7.2.4.8.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1 and #2.3 clause 7.2.4.3.

##### 7.2.4.8.3	Message flows

The V-CHF would in this case use the vNRF provided service to find the H-CHF, where the vNRF and hNRF may communicate according to TS 23.501 clause 6.2.6.

#### 7.2.4.9	Solution #2.9: Using SUPI to find H-CHF

##### 7.2.4.9.1	General

A possible solution for key issue #2c, finding the correct CHF for solutions #2.1 and #2.3 where V-CHF communicating with H-CHF.

##### 7.2.4.9.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1 and #2.3 clause 7.2.4.3.

##### 7.2.4.9.3	Message flows

The V-CHF would in this case use the SUPI i.e., the IMSI number series, to find the V-CHF. In this case the H-CHF address would be pre-provisioned in the V-CHF and allocated to a IMSI series.

#### 7.2.4.10	Solution #2.10: UE triggered V-CHF and H-CHF communication

##### 7.2.4.10.1	General

A possible solution for key issue #2e, would be to have the trigger for when the V-CHF should trigger a charging session with the H-CHF based on that the V-CHF gets a charging data request containing a SUPI that the V-CHF don’t have a session with a H-CHF setup.

##### 7.2.4.10.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.10.3	Message flows

In this case, the SMF, AMF, and SMSF establish the charging session and events with the V-CHF per as specified in the TS 32.255 [4], TS 32.256 [5], and TS 32.274 [8]. The V-CHF would establish the charging session with the H-CHF based on the SUPI i.e., on a per UE basis.

Editor’s note: the message flows for the UE based solution are FFS

#### 7.2.4.11	Solution #2.11: Charging data request triggered V-CHF and H-CHF communication

##### 7.2.4.11.1	General

A possible solution for key issue #2e, the trigger for interaction for V-CHF communicating with H-CHF is based on the reception of a charging data request by the V-CHF.

##### 7.2.4.11.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.11.3	Message flows

In this case V-CHF as the NF consumer towards the H-CHF, triggers a charging data request based on the reception of a charging data request from another NF consumer. The V-CHF to may filter or convert the trigger from the V-CHF’s NF consumer i.e., for 5G data connectivity charging the V-CHF may use either FBC or QBC triggers to trigger a charging data request towards the H-CHF.

#### 7.2.4.12	Solution #2.12: Roaming charging profile update

##### 7.2.4.12.1	General

A possible solution for key issue #2e covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02, trigger handling between VPLMN and HPLMN, is to use the roaming charging profile and allow it to be updated.

##### 7.2.4.12.2	Reference architecture

The reference architecture would be the same as in solution #2.2 clause 7.2.4.2.

##### 7.2.4.12.3	Message flows

The Roaming charging profile may be requested at any time by the V-SMF. This means that the V-SMF could include it in every charging data request, allowing the V-SMF, V-CHF or H-CHF to change it:

1.	The V-SMF includes Roaming charging profile in a Charging data request [Update] to V-CHF

2.	Changed by V-CHF and transferred to V-SMF

3.	Transferred from V-SMF to H-CHF

4.	Changed by H-CHF and transferred to V-SMF

5.	Applied in V-SMF

#### 7.2.4.13	Solution #2.13: V-CHF selected charging information for reporting to H-CHF

##### 7.2.4.13.1	General

A possible solution for key issues #2a, #2b, #2f, #2g, #2h, and #2i, the charging information provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session (From the standpoint of V-SMF, all the charging information for V-CHF and H-CHF are exchanged in one charging session between V-SMF and V-CHF), V-CHF is responsible for the charging reporting for H-CHF.

##### 7.2.4.13.2	Reference architecture

The reference architecture would be the same as figure 7.2.4.1.2-2.

##### 7.2.4.13.3	Message flows

During the PDU session establishment, V-SMF establishes the charging session with V-CHF, same with the figure 7.2.4.1.3-1 in solution #2.1.

V-CHF is responsible for distinguishing the charging information for H-CHF and V-CHF.

1.	The V-SMF sends Charging data request to V-CHF with the charging information;

2.	V-CHF determines the charging information for V-CHF and handled it.

3.	Based on the triggers on V-CHF, V-CHF constructs and reports the charging information to H-CHF;

4.	H-CHF sends the charging data response to V-CHF;

5.	V-CHF sends charging data response from H-CHF and/or sends the charging response from V-CHF.

#### 7.2.4.14	Solution #2.14: V-SMF selected charging information for reporting to H-CHF

##### 7.2.4.14.1	General

A possible solution for key issue #2a and #2b, the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session, V-SMF is responsible for the charging reporting for the V-CHF and H-CHF.

##### 7.2.4.14.2	Reference architecture

The reference architecture would be the same as figure 7.2.4.1.2-2.

##### 7.2.4.14.3	Message flows

During the PDU session establishment, V-SMF establishes the single charging session with V-CHF, same with the figure 7.2.4.1.3-1 in solution #2.1.

V-SMF is responsible for distinguishing the charging information for H-CHF and V-CHF. The V-CHF sends the charging information between V-SMF and H-CHF.

1.	The V-SMF sends charging data request to V-CHF with both the charging information for V-CHF and charging information for H-CHF with the indication;

2.	Based on the indication, V-CHF handles the charging information for V-CHF;

3.	Based on the indication, V-CHF sends charging information to H-CHF;

4.	H-CHF sends the charging data response to V-CHF;

5.	V-CHF sends charging data response from H-CHF and/or sends the charging data response from V-CHF its own.

#### 7.2.4.15	Solution #2.15: V-SMF setting charging information indication by information element group

##### 7.2.4.15.1	General

A possible solution for key issue #2a and #2b, the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session, V-SMF is responsible for the charging reporting for the V-CHF and H-CHF, V-SMF sets an indication for a group of information elements for V-CHF, and sets another indication for the other group of information elements for H-CHF.

##### 7.2.4.15.2	Reference architecture

The reference architecture would be the same as figure 7.2.4.1.2-2.

##### 7.2.4.15.3	Message flows

In step 1 of message flow figure 7.2.4.1.3-1, take the following charging information of FBC for example, the V-SMF set two indications for two groups of information elements, one indication is for V-CHF, the other indication is for H-CHF.

Table 7.2.4.15.3-1: Structure of Roaming QBC Information

Multiple Unit Usage (Group2 for V-CHF)

Rating Group

Used Unit Containerr

Target Indication: V-CHF

Multiple Unit Usage (Group2 for H-CHF)

Rating Group

Requested Unit

Used Unit Container

Target Indication: H-CHF

#### 7.2.4.16	Solution #2.16: CDF and OCF communicating with SMS-SC for retail charging of SMS

##### 7.2.4.16.1	General

A possible solution for key issues #2f and #2g covering requirements REQ-CH_CVTOH-03, and REQ-CH_CVTOH-04, wholesale charging for SMS provided to the home MNO by the visited MNO. In this case the home MNO uses charging architecture defined in TS 32.240 [12] annex E, to generate charging information.

##### 7.2.4.16.2	Reference architecture

In the below architectures the CHF in home MNO is only added for completes and the SMS-SC will only interact with the CDF and OCF.

Figure 7.2.4.16.2-1: Roaming SMS scenario in service-based interface representation

Figure 7.2.4.16.2-2: Roaming SMS scenario in reference point representation

##### 7.1.4.16.3	Message flows

The flows for SMS charging would be the same as in TS 32.274 [8], that are based on the interactions specified in TS 23.540 [10] and TS23.040 [11].

#### 7.2.4.17	Solution #2.17: configure the roaming charging profile update

##### 7.2.4.17.1	General

A possible solution for key issue #2e covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02.

In some 5G use case, PDU session may keep for a long time, it’s possible that triggers in HPLMN change during PDU session for some reason like user subscription change, if so, the triggers in VPLMN needs to keep alignment with HPLMN.

##### 7.2.4.17.2	Reference architecture

The reference architecture would be the same as the local breakout architecture in solution #2.2 clause 7.2.4.2 or Home Routed architecture in Figure 4.2.4 of TS 32.255.

##### 7.2.4.17.3	Message flows

The message flow is same with the PDU session establishment message flow in clause 5.2.2.12.2 or 5.2.2.18.2 of TS 32.255 [4]. If the roaming charging profile is requested at the PDU session establishment by the V-SMF, the H-CHF may provide multiple Roaming Charging Profiles in the charging data response at the PDU session establishment, some Roaming Charging Profiles are with Profile change time, some Roaming Charging Profiles are without Profile change time, the Roaming Charging Profiles without Profile change time are applied immediately, the Roaming Charging Profiles with Profile change time are applied at the Profile change time.

Table 7.2.4.x.3-1 is an example of two Roaming Charing Profiles. One Roaming Charging Profile is with Profile change time, the other Roaming Charging Profile is without Profile change time, the Roaming Charging Profile without Profile change time is applied immediately, the Roaming Charging Profile with Profile change time is applied at the Profile change time.

Table 7.2.4.17.3-1: Multiple Roaming Charing Profiles

Roaming Charging Profile

Trigger

Trigger type

Trigger category

Time Limit

Volume Limit

Max Number of charging condition changes

Partial record method

Roaming Charging Profile

Trigger

Trigger type

Trigger category

Time Limit

Volume Limit

Max Number of charging condition changes

Partial record method

Profile change time

### 7.2.5	Evaluation

Solutions #2.2 and #2.1 both solves key issues #2a and #2b and covers requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02. Solution #2.1 solves more key issues and covers more requirements, and #2.2 is already supported in the scope of 5G connectivity charging.

Solutions #2.4, #2.1, #2.16 all solves key issues #2f and #2g and covers requirements REQ-CH_CVTOH-03, and REQ-CH_CVTOH-04. Solution #2.1 solves more key issues and covers more requirements, and #2.16 is already supported in the scope of SMS charging as is.

Solutions #2.5 and #2.1 both solves key issues #2h and #2i and covers requirements REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06. Solution #2.1 solves more key issues and covers more requirements and #2.5 is already supported in the scope of 5G connection and mobility charging as is.

Solutions #2.8 and 2.9 both solves key issues #2c, both require solution #2.1 or #2.3, and can be supported at the same time depending on the need and capability of the V-CHF.

Solutions #2.6 and #2.7 both solves key issues #2d.

Solutions #2.10, #2.11 and #2.12 all solves key issues #2e.

Editor’s note:	Further evaluations are FFS.

## 7.3	Charging in an MNO for the purpose of wholesale charging towards an additional actor

#### 7.3.1	Use cases

##### 7.3.1.1	Use case #3a: MNO does wholesale charging for 5G data connectivity towards additional actors

This use case focuses on MNO and MVNO business roles.

A subscriber has a UE and a subscription with the MVNO which allows usage of 5G data connectivity while in the home MNO.

The MNO to MVNO charging could be based on data volume that the MVNO UE has used in the MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

##### 7.3.1.2	Use case #3b: MNO does wholesale charging for 5G connection and mobility towards additional actors

This use case focuses on MNO and MVNO (additional actor) business roles.

A CSC (UE) has a subscription with the additional actor which allows usage of 5G data connectivity connection and mobility.

The home MNO to additional actor charging is based number of home MNO’s UEs (i.e. unique SUPI) connected to the visited MNO’s network over a specific period of time.

##### 7.3.1.3	Use case #3c: MNO does wholesale charging for 5G SMS towards additional actors

This use case focuses on MNO and MVNO (additional actor) business roles.

A CCS (UE) has a subscription with the MVNO which allows usage of SMS over NAS in 5GS.

The home MNO to additional actor charging could be based on number of SMS sent or received by the additional actor’s UEs served by the home MNO’s network considering of the following aspects:

-	RAT type;

-	Location

### 7.3.2	Potential charging requirements

REQ-CH_MNOMVNO-01: The charging mechanism in MNO should support collecting charging information related to 5G data connectivity usage for each MVNO.

REQ-CH_MNOMVNO-02: The charging mechanism in MNO should support collecting charging information related to 5G connection and mobility usage for each MVNO.

REQ-CH_MNOMVNO-03: The charging mechanism in MNO should support collecting charging information related to SMS usage for each MVNO.

### 7.3.3	Key issues

The following key issues are identified:

-	Key Issue #3a: Collection of charging information in home MNO about the 5G data connectivity usage per MVNO.

-	Key Issue #3b: Collection of charging information in home MNO about the 5G connection and mobility usage per MVNO.

-	Key Issue #3c: Collection of charging information in home MNO about the SMS usage per MVNO.

### 7.3.4	Potential solutions

#### 7.3.4.1	Solution #3.1: CDR in home MNO for wholesale of non-roaming 5G of additional actor

##### 7.3.4.1.1	General

A possible solution for key issue #3a, #3b, and #3c covering requirements REQ-CH_MNOMVNO-01, REQ-CH_MNOMVNO-02, and REQ-CH_MNOMVNO-03, charging for 5G data connectivity, connection and mobility, SMS, provided to the additional actor by the home MNO, covering non-roaming case.

##### 7.3.4.1.2	Reference architecture

Figure 7.3.4.2-1: Additional actor 5G scenario in service-based interface representation

Figure 7.3.4.1.2-2: Additional actor 5G scenario in reference point representation

The AMF, SMF, and SMSF with CTF interact with CHF using Nchf interface, without any requirement of quota management. The CCS produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of data per additional actor, it could be done based on SUPI, IMSI series, SNSSAI(s), DNN(s) etc., the wholesale part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.3.4.1.3	Message flows

The flows would be the same as the non-roaming cases in clause 5.2.2 of TS 32.255 [4], clause 5.2.2 of TS 32.256 [5] and clause 5.4.2 of TS 32.274 [8].

### 7.3.5	Evaluation

There is only one solution (#3.1), it solves all key issues (#3a, #3b, and #3c), and covers all requirements (REQ-CH_MNOMVNO-01, REQ-CH_MNOMVNO-02, and REQ-CH_MNOMVNO-03).

Editor’s Note:	Further evaluations are FFS.

## 7.4	Convey charging information from an MNO to an additional actor

### 7.4.1	Use cases

#### 7.4.1.1	Use case #4a: Additional actor does retail charging for 5G data connectivity

This use case focuses on additional actor (e.g., MVNO) and subscriber business roles.

A subscriber has a UE and a subscription with the additional actor which allows usage of 5G data connectivity. The additional actor has their own billing system and charging function (CHF), but no other NFs.

The additional actor charging of the subscriber could be based on data volume that the UE has used in the MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

#### 7.4.1.2	Use case #4b: Additional actor does retail billing for 5G data connectivity

This use case focuses on additional actor (e.g., MVNO) and subscriber business roles.

A subscriber has a UE and a subscription with the additional actor which allows usage of 5G data connectivity. The additional actor has their own billing system, but no CHFs.

The additional actor charging of the subscriber could be based on data volume that the UE has used in the home MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

#### 7.4.1.3	Use case #4c: Use case for additional actor does wholesale charging for 5G data connectivity towards another additional actor

This use case focuses on home MNO and additional actor (wholesaler] roles.

A subscriber has a UE and a subscription with an additional actor (retailer) which allows usage of 5G data connectivity. The additional actor (retailer) has an agreement with another additional actor (wholesaler) for usage of 5G data connectivity, the other additional actor (wholesaler) in turn has agreement with the home MNO for 5G data connectivity, this allows the additional actor’s (retailer) subscribers (UEs) to use the home MNO’s network. The additional actors have their own billing system and charging function (CHFs), but no other NFs.

The additional actor (wholesaler) charging of the additional actor (retailer) could be based on data volume that additional actor’s (retailer) subscribers (UEs) has used in the home MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.4.2	Potential charging requirements

REQ-CH_CHTOA-01: The charging mechanism in MNO should support conveying charging information for the 5G data connectivity usage to the MVNO for each UE.

REQ-CH_ CHTOA-02: The charging mechanism in MNO should support collecting charging information related to 5G data connectivity usage for each UE.

### 7.4.3	Key issues

The following key issues are identified:

-	Key Issue #4a: Collection of charging mechanism in MNO about the 5G data connectivity usage.

-	Key Issue #4b: Conveying the charging information to the MVNO

- 	Key Issue #4c: Finding the additional actor’s CHF

-	Key Issue #4d: Service based interface to use towards the additional actor’s CHF

-	Key Issue #4e: Trigger handling towards the additional actor’s CHF

### 7.4.4	Potential solutions

#### 7.4.4.1	Solution #4.1: Home SMF (CTF) communicating with both H-CHF and A-CHF for retail charging of 5G data connectivity

##### 7.4.4.1.1	General

The possible solutions for key issues #4a and #4b covering requirements REQ-CH_CHTOA-01, and REQ-CH_CHTOA-02, retail charging for 5G data connectivity provided by the additional actor, to the MVNO by the home MNO.

##### 7.4.4.1.2	Reference architecture

Figure 7.4.4.1.2-1: Additional actor with only CHF  5G data connectivity scenario in service-based interface representation

Figure 7.4.4.1.2-2: Additional actor with only CHF  5G data connectivity scenario in service-based interface representation

The MNO CHF interacts with the additional actor CHF does converged charging for the subscriber.

##### 7.4.4.1.3	Potential charging flows

The flows would be the same as in clause 7.2.4.2 where the V-CHF corresponds to the home MNO’s CHF and the H-CHF corresponds to the additional actor’s CHF.

#### 7.4.4.2	Solution #4.2: Additional actor only does retail billing of subscribers

##### 7.4.4.2.1	General

The possible solutions for key issues #4a and #4b covering requirements REQ-CH_CHTOA-01, and REQ-CH_CHTOA-02, retail billing for 5G data connectivity by the additional actor, the 5G data connectivity is provided by the home MNO.

##### 7.4.4.2.2	Reference architecture

Figure 7.4.4.2.2-1: Additional actor with only billing 5G data connectivity scenario in service-based interface representation

Figure 7.4.4.2.2-2: Additional actor with only billing 5G data connectivity scenario in service-based interface representation

The home MNO CCS produces CDRs that the additional actor uses for billing of the subscriber.

##### 7.4.4.2.3	Potential charging flows

The flows would be the same as in clause 7.1.4.1.2 where the V-CHF corresponds to the home MNO’s CHF and the interconnect would be replaced by sending information for retail charging.

#### 7.4.4.3 	Solution #4.3: H-CHF communicating with A-CHF for retail charging of 5G

##### 7.4.4.3.1	General

A possible solution for key issues #4a and #4b covering requirements REQ-CH_CHTOA-01, and REQ-CH_CHTOA-02, retail charging for 5G data connectivity, 5G connection and mobility, and SMS usage provided to the additional actor’s users by the home MNO.

The home CHF does converged charging for interconnect, while the additional actor CHF does converged charging for the subscriber.

##### 7.4.4.3.2	Reference architecture

Figure 7.2.4.3.2-1: MNO – Additional actor5G scenario in service-based interface representation

Figure 7.2.4.3.2-2: MNO – Additional actor5G scenario in reference point representation

##### 7.4.4.3.3	Message flows

This would be same as for solution #2.1 in clause 7.2.4.1 where the H-CHF is changed to A-CHF, and the V-CHF is changed to CHF.

#### 7.4.4.4	Solution #4.4: Reusing Nchf_ConvergedCharging service API between CHFs

##### 7.4.4.4.1	General

A possible solution for key issue #4d covering requirements REQ-CH_CVTOA-01 and REQ-CH_CVTOA-02, service based interface to use between home CHF and additional actor CHF, would be to reuse the Nchf_ConvergedCharging service API. This would mean that the H-CHF would proxy the request from the AMF, SMF or SMSF to the A-CHF, and the same with the response. The H-CHF could do some changes to message like: filter (e.g. trigger), enrich, or convert (e.g., rating groups).

##### 7.4.4.4.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

#### 7.4.4.5	Solution #4.5: New Nchf service API between CHFs

##### 7.4.4.5.1	General

A possible solution for key issue #4d covering requirements REQ-CH_CVTOA-01 and REQ-CH_CVTOA-02, service based interface to use between home CHF and additional actor CHF, would be to create a new service API. This would mean that the H-CHF would translate the request from the AMF, SMF or SMSF to the new service API towards the H-CHF. This would mean that the message could look completely different and even the triggers in the H-CHF for sending the message towards the A-CHF wouldn’t have to be related to the request from AMF, SMF or SMSF.

##### 7.4.4.5.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

#### 7.4.4.6	Solution #4.6: Using NRF to find A-CHF

##### 7.4.4.6.1	General

A possible solution for key issue #4c, finding the correct CHF for solution #4.3 where H-CHF communicating with A-CHF.

Editor’s Note: The solution #4.1 where H-CHF communicate with A-CHF is FFS.

##### 7.4.4.6.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

##### 7.4.4.6.3	Message flows

The H-CHF would in this case use the NRF provided service to find the A-CHF, the selection can be based on any available attribute such as SUPI, CHF group, etc., where the A-CHF would be registered in the NRF.

#### 7.4.4.7	Solution #4.7: Using local configuration to find A-CHF

##### 7.4.4.7.1	General

A possible solution key issue #4c, finding the correct CHF for solution #4.3 where H-CHF communicating with A-CHF.

Editor’s Note: The solution #4.1 where H-CHF communicate with A-CHF is FFS.

##### 7.4.4.7.2	Reference architecture

The reference architecture would be the same as in solution #4.3 clause 7.4.4.3.

##### 7.4.4.7.3	Message flows

The H-CHF would in this case use the SUPI e.g., SUPI lists, IMSI number series, to find the A-CHF. In this case the A-CHF address would be pre-provisioned in the H-CHF and allocated for example to a SUPI list or an IMSI series.

### 7.4.5	Evaluation

Solutions #4.1, #4.2, and #4.3 all solves key issues #4a and #4b and covers requirements REQ-CH_CVTOA-01, and REQ-CH_CVTOA-02. Solution #4.3 solves more key issues and covers more requirements, #4.1 is partly supported in the scope of 5G connectivity charging, and #4.2 is outside the current scope of 3GPP SA5.

Solutions #4.4 and #4.5 both solves key issues #4d and require solution #4.3.

Editor’s Note:	Further evaluations are FFS.

Editor’s Note:	Scope of the additional actors that are applicable (e.g., only those having a CHF) are FFS.

## 7.5	Convey charging information from visited MNO to home MNO, and home MNO to an additional actor

### 7.5.1	Use cases

#### 7.5.1.1	Use case #5a: Use case for additional actor does retail changing for 5G data connectivity while roaming

This use case focuses on visited MNO, home MNO, MVNO and subscriber business roles.

A subscriber has a UE and a subscription with the MVNO which allows usage of 5G data connectivity. The MVNO has their own billing system and charging function (CHF), but no other NFs. The MVNO has an agreement with the home MNO that allows its subscribers roaming and the home MNO as a roaming agreement with the visited MNO.

The MVNO charging of the subscriber could be based on data volume that the UE has used in the visited MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.5.2	Potential charging requirements

REQ-CH_CVTOHTOA-01: The charging mechanism in visited MNO should support conveying charging information for the 5G data connectivity usage to home MNO, allowing further conveying it to the MVNO for each UE.

REQ-CH_ CVTOHTOA-02: The charging mechanism in home MNO should support conveying charging information for the 5G data connectivity usage to MVNO for each UE.

REQ-CH_ CVTOHTOA-03: The charging mechanism in MVNO should support receiving charging information related to 5G data connectivity usage for each UE.

### 7.5.3	Key issues

The following key issues are identified:

-	Key Issue #5a: Collection of charging mechanism in visited MNO about the 5G data connectivity usage.

-	Key Issue #5b: Conveying the charging information from visited to the home MNO

-	Key Issue #5c: Conveying the charging information to the home MNO and MVNO

- 	Key Issue #5d: Finding the correct CHF in home MNO and MVNO

-	Key Issue #5e: Service based interface to use between visited CHF, home CHF and MVNO

-	Key Issue #5f: Trigger handling between visited CHF, home CHF and MVNO

### 7.5.4	Potential solutions

#### 7.5.4.1	Solution #5.1: CHF to CHF communication

##### 7.5.4.1.1	General

A possible solution for key issue #5a, #5b, and #5c covering requirements REQ-CH_CVTOHTOA-01, REQ-CH_CVTOHTOA-02, and REQ-CH_CVTOHTOA-03, charging for 5G roaming provided to the additional actor by the visited MNO through the home MNO.

##### 7.5.4.1.2	Reference architecture

Figure 7.5.4.1.2-1: Additional actor with only CHF 5G roaming scenario in service-based interface representation

Figure 7.5.4.1.2-2: Additional actor with only CHF 5G roaming scenario in reference point representation

The visited and home MNO CHF does converged charging for interconnect, while the additional actor CHF does converged charging for the subscriber.

##### 7.5.4.1.3	Message flows

This would be a combination of the flows in clauses 7.2.4.1 and 7.4.4.3.

#### 7.5.4.2	Solution #5.2: SMF to multiple CHF communication

##### 7.5.4.2.1	General

A possible solution for key issue #5a, #5b, and #5c covering requirements REQ-CH_CVTOHTOA-01, REQ-CH_CVTOHTOA-02, and REQ-CH_CVTOHTOA-03, charging for 5G data connectivity provided to the additional actor by the visited MNO through the home MNO.

##### 7.5.4.2.2	Reference architecture

Figure 7.5.4.2.2-1: Additional actor with only CHF 5G data connectivity roaming scenario in service-based interface representation

Figure 7.5.4.2.2-2: Additional actor with only CHF 5G data connectivity roaming scenario in reference point representation

The visited and home MNO CHF does converged charging for interconnect, home MNO CHF does converged charging for wholesale, and the additional actor CHF does converged charging for the subscriber.

##### 7.5.4.2.3	Message flows

This would be a combination of the flows in clauses 7.2.4.2 and 7.4.4.1.

### 7.5.5	Evaluation

Solutions #5.1 and #5.2 both solves key issues #5a, #5b, and #5c and covers requirements REQ-CH_CVTOHTOA-01, REQ-CH_CVTOHTOA-02, and REQ-CH_CVTOHTOA-03.

Editor’s Note:	There are no solutions for key issues #5d, #5e, and #5f.

Editor’s Note:	Further evaluations are FFS.

Editor’s Note:	A solution where the V-SMF connects to H-CHF and V-CHF, while H-CHF connects to A-CHF is FFS.

## 7.6	Reconciliation of wholesale charging from the visited MNO with home MNO information

### 7.6.1	Use cases

#### 7.6.1.1	Use case #6a: support for validation of usage information received from visited MNO at the home MNO

This use case focuses on visited MNO and home MNO business roles.

When the home MNO receives the interconnect charges it would like to have the possibility to validate the received information with information controlled by itself.

### 7.6.2	Potential charging requirements

REQ-CH_RECON-01: The charging mechanism should allow the home MNO to monitor the usage without any involvement of the visited MNO.

### 7.6.3	Key issues

The following key issues are identified:

-	Key Issue #6a: Collection of usage of home MNO independent and without control of the visited MNO.

### 7.6.4	Potential solutions

#### 7.6.4.1	Solution #6.1: Use home routing

##### 7.6.4.1.1	General

A possible solution for key issue #6a covering requirements REQ-CH_RECON-01, would be to use home routed, where the connection is sent to the home MNO, see TS 32.255 [4].

# 8	Conclusions and recommendations

Local Breakout has been present in the standards for a long time but a solution for charging has been missing until now. The deployment of Network Slices, Edge Computing and different use cases that need local traffic routing have created the necessity of addressing all the gaps for deployments of local breakout. In this TR some use cases have been covered and some solutions provided.

Considering the different solutions to the use cases, the solutions where the SMF communicates with both the V-CHF and H-CHF at the same point in time i.e., solutions #2.2 (Visited NF (CTF) communicating with both H-CHF and V-CHF) and #4.1 (Additional actor has CHF and does retail charging), are preferred. This since these are the only solution that can be included in the time frame of Rel-17 and that covers the needed requirements. The focus for the specified solution will be to limit and minimize the impact on and to the CHF and SMF.

Editor’s note: Other solutions may be added depending on further study.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-09 | SA5#138e | S5-214431 |  |  |  | Initial skeleton (v0.0.0) of TR 28.827 | 0.0.0 |
| 2021-09 | SA5#138e | S5-214433 |  |  |  | Study document structure | 0.1.0 |
| 2021-09 | SA5#138e | S5-214434 |  |  |  | Addition of business roles and concepts | 0.1.0 |
| 2021-09 | SA5#138e | S5-214436 |  |  |  | Use case for visited MNO charging home MNO for 5G connection and mobility | 0.1.0 |
| 2021-09 | SA5#138e | S5-214437 |  |  |  | Use case for visited MNO charging home MNO for 5G data connectivity | 0.1.0 |
| 2021-09 | SA5#138e | S5-214703 |  |  |  | Addition of introduction and scope | 0.1.0 |
| 2021-09 | SA5#138e | S5-214716 |  |  |  | Use case for home MNO charging subscriber for 5G data connectivity | 0.1.0 |
| 2021-10 | SA5#139e | S5-215244 |  |  |  | New possible solution for use case #2a | 0.2.0 |
| 2021-10 | SA5#139e | S5-215471 |  |  |  | Use case for additional actor does retail charging for home UE | 0.2.0 |
| 2021-10 | SA5#139e | S5-215472 |  |  |  | Use case for additional actor does retail charging for roaming UE | 0.2.0 |
| 2021-10 | SA5#139e | S5-215447 |  |  |  | Use case for home MNO does wholesale charging towards additional actor | 0.2.0 |
| 2021-10 | SA5#139e | S5-215588 |  |  |  | Clarify the Business Role | 0.2.0 |
| 2021-10 | SA5#139e | S5-215589 |  |  |  | Addition of the LBO,HR and BCE in Concept and Overview | 0.2.0 |
| 2021-10 | SA5#139e | S5-215596 |  |  |  | Clarification on the use cases and key issues for Topic 7.1 | 0.2.0 |
| 2021-10 | SA5#139e | S5-215597 |  |  |  | Addition of the LBO message flows for Topic 7.1 | 0.2.0 |
| 2021-10 | SA5#139e | S5-215598 |  |  |  | Addition the new solution in the Topic 7.2 | 0.2.0 |
| 2021-11 | SA5#140e | S5-216521 |  |  |  | Correction on the potential charging architectures | 0.3.0 |
| 2021-11 | SA5#140e | S5-216526 |  |  |  | Adding reference point architecture to clause 7.1.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216527 |  |  |  | Adding SBI architecture to clause 7.2.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216528 |  |  |  | Adding reference point architecture to clause 7.3.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216529 |  |  |  | Adding reference point architecture to clause 7.4.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216530 |  |  |  | New use case MVNO without CHF | 0.3.0 |
| 2021-11 | SA5#140e | S5-216531 |  |  |  | New potential solution MVNO without CHF | 0.3.0 |
| 2021-11 | SA5#140e | S5-216532 |  |  |  | Adding reference point architecture to clause 7.5.4 | 0.3.0 |
| 2022-02 | SA5#141e | S5-221122 |  |  |  | Editorial Correction on the Figures | 0.4.0 |
| 2022-02 | SA5#141e | S5-221322 |  |  |  | Corrections to clause 7.2 | 0.4.0 |
| 2022-02 | SA5#141e | S5-221323 |  |  |  | Corrections to clause 7.3 | 0.4.0 |
| 2022-02 | SA5#141e | S5-221324 |  |  |  | Corrections to clause 7.4 | 0.4.0 |
| 2022-02 | SA5#141e | S5-221656 |  |  |  | Adding Conclusions | 0.4.0 |
| 2022-02 | SA5#141e | S5-221679 |  |  |  | Adding use case and solution for clause 7.6 | 0.4.0 |
| 2022-02 | SA5#141e | S5-221681 |  |  |  | Corrections to clause 7.1 | 0.4.0 |
| 2022-02 | SA5#141e | S5-221732 |  |  |  | Clarification on the Roaming Charging Profile | 0.4.0 |
| 2022-03 | SA#95e | SP-220130 |  |  |  | Presented for information | 1.0.0 |
| 2022-04 | SA5#142e | S5-222240 |  |  |  | Addition of the solution for the Roaming Charging Profile | 1.1.0 |
| 2022-04 | SA5#142e | S5-222241 |  |  |  | Correction on the solution 1.3 | 1.1.0 |
| 2022-04 | SA5#142e | S5-222797 |  |  |  | Add a SMS use case and solution for 7.1 | 1.1.0 |
| 2022-04 | SA5#142e | S5-222798 |  |  |  | Add a SMS use case Key issues and requirements for 7.2 | 1.1.0 |
| 2022-04 | SA5#142e | S5-222799 |  |  |  | Add CHF to CHF solution for SMS in 7.2 | 1.1.0 |
| 2022-04 | SA5#142e | S5-222800 |  |  |  | Add CTF to two CHFs solution for SMS in 7.2 | 1.1.0 |
| 2022-04 | SA5#142e | S5-222810 |  |  |  | Additional BCE and TAP relationship | 1.1.0 |
| 2022-05 | SA5#143e | S5-223112 |  |  |  | Use case for 5G connection and mobility in clause 7.2 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223115 |  |  |  | Updating solution 2.2 with SMF | 1.2.0 |
| 2022-05 | SA5#143e | S5-223116 |  |  |  | Adding CHF to CHF solution to clause 7.4 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223117 |  |  |  | Updating solution 5.1 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223121 |  |  |  | Solution for issue 2d servce to use between CHFs | 1.2.0 |
| 2022-05 | SA5#143e | S5-223712 |  |  |  | Adding AMF solution to clause 7.2 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223713 |  |  |  | Updating solution 2.1 with SMSF and AMF | 1.2.0 |
| 2022-05 | SA5#143e | S5-223715 |  |  |  | Correcting solution 1.3 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223719 |  |  |  | Use case for 5G connection and mobility in clause 7.3 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223720 |  |  |  | Adding SMF to multiple CHF solution to clause 7.5 | 1.2.0 |
| 2022-05 | SA5#143e | S5-223721 |  |  |  | Solution for issue 2c CHF selection | 1.2.0 |
| 2022-07 | SA5#144e | S5-224199 |  |  |  | Mapping requirement and issue in clause 7.1 | 1.3.0 |
| 2022-07 | SA5#144e | S5-224201 |  |  |  | Mapping requirement and issue in clause 7.3 | 1.3.0 |
| 2022-07 | SA5#144e | S5-224202 |  |  |  | Mapping requirement and issue in clause 7.4 | 1.3.0 |
| 2022-07 | SA5#144e | S5-224203 |  |  |  | Mapping requirement and issue in clause 7.5 | 1.3.0 |
| 2022-07 | SA5#144e | S5-224204 |  |  |  | Mapping requirement and issue in clause 7.6 | 1.3.0 |
| 2022-07 | SA5#144e | S5‑224469 |  |  |  | Clarification on the Roaming Charging Profile for HR | 1.3.0 |
| 2022-07 | SA5#144e | S5‑224470 |  |  |  | Add the solution for the charging session granularity | 1.3.0 |
| 2022-07 | SA5#144e | S5‑224471 |  |  |  | Add the solution for the charging information granularity | 1.3.0 |
| 2022-07 | SA5#144e | S5‑224472 |  |  |  | Add the key issues for the FBC negotiation | 1.3.0 |
| 2022-07 | SA5#144e | S5‑224473 |  |  |  | Add the key issue for QoS flow level of QBC triggers setting | 1.3.0 |
| 2022-07 | SA5#144e | S5-224474 |  |  |  | Correcting message flows in clause 7.2.4 | 1.3.0 |
| 2022-07 | SA5#144e | S5-224475 |  |  |  | Adding solutions in clause 7.1 for VPLMN wholesale charging of HPLMN | 1.3.0 |
| 2022-07 | SA5#144e | S5-224476 |  |  |  | Adding solution in clause 7.1 for roaming charging profile | 1.3.0 |
| 2022-07 | SA5#144e | S5-224477 |  |  |  | Adding solution in clause 7.2 using roaming charging profile for triggers | 1.3.0 |
| 2022-07 | SA5#144e | S5-224478 |  |  |  | Adding solutions in clause 7.4 for CHF to CHF communication | 1.3.0 |
| 2022-08 | SA5#145e | S5-225298 |  |  |  | Mapping requirement and issue in clause 7.2 | 1.4.0 |
| 2022-08 | SA5#145e | S5-225436 |  |  |  | Correction on the Unified Term for NF in the HPLMN and VPLMN | 1.4.0 |
| 2022-08 | SA5#145e | S5-225649 |  |  |  | Add the solution of V-SMF connects to V-CHF with single charging session | 1.4.0 |
| 2022-08 | SA5#145e | S5-225730 |  |  |  | Adding use case in clause 7.4 where there are multiple additional actors | 1.4.0 |
| 2022-08 | SA5#145e | S5-225734 |  |  |  | Update of business roles | 1.4.0 |
| 2022-08 | SA5#145e | S5-225748 |  |  |  | Add new solution for FBC negotiation | 1.4.0 |
| 2022-08 | SA5#145e | S5-225751 |  |  |  | Add the solution of V-SMF connect to V-CHF with two charging sessions | 1.4.0 |
| 2022-08 | SA5#145e | S5-225752 |  |  |  | Add the solution V-CHF controls reporting in single charging session | 1.4.0 |
| 2022-08 | SA5#145e | S5-225753 |  |  |  | Add the solution V-SMF controls reporting in single charging session | 1.4.0 |
| 2022-11 | SA5#146 | S5-226437 |  |  |  | Restructure clause 7.2.1 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226694 |  |  |  | Correcting solution for charging trigger configuration | 1.5.0 |
| 2022-11 | SA5#146 | S5-226695 |  |  |  | Adding solution for SMS in clause 7.2 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226696 |  |  |  | Updating solutions 2.10 and 2.11 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226697 |  |  |  | Correcting solutions 2.13 to 2.15 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226698 |  |  |  | New key issue for selection of charging information in clause 7.2.3 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226699 |  |  |  | Evaluation of solutions for clause 7.2 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226700 |  |  |  | Evaluation of solutions for clause 7.3 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226701 |  |  |  | Evaluation of solutions for clause 7.4 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226702 |  |  |  | Evaluation of solutions for clause 7.5 | 1.5.0 |
| 2022-11 | SA5#146 | S5-226703 |  |  |  | Add new solution for PDU session level negotiation | 1.5.0 |
| 2022-11 | SA5#146 | S5-226704 |  |  |  | Add new solution for QBC trigger setting | 1.5.0 |
| 2022-11 | SA5#146 | S5-226705 |  |  |  | Add new solution for Roaming Charging Profile Update | 1.5.0 |
| 2023-01 | SA5#146Bis-e | S5-231119 |  |  |  | Correcting covered requirements for solution #1.9 | 1.6.0 |
| 2023-01 | SA5#146Bis-e | S5-231260 |  |  |  | Corrections | 1.6.0 |
| 2023-01 | SA5#146Bis-e | S5-231271 |  |  |  | Correcting solution #1.10 in clause 7.1 | 1.6.0 |
| 2023-01 | SA5#146Bis-e | S5-231279 |  |  |  | Correcting solution #2.1 in clause 7.2 | 1.6.0 |