3GPP TSG-SA5 Meeting #147 	S5-232296

Athens, Greece, 27th February - 3rd March 2023

# Source:	Huawei

# Title:	Improve description of prediction and statistics of management data

# Document for:	Approval

Agenda Item:	6.4.2.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	S5-227066 Draft CR 28.104 Use case and Requirements for Prediction and Statistics for Management data

# 3	Rationale

The existing text is very inconsistent with using the terms “statistics and predictions” versus “predictions and statistics” which makes the text very difficult to read.

The existing text does not make it clear if this capability applies at cross-domain level, at domain level, or both.

# 4	Detailed proposal

The following changes are proposed to [1]

1st change

### 7.2.x	Prediction and statistics of Management data

##### 7.2.x.1	Description

This use case is for the analytics (predictions and statistics) of the management data including 5G PMs defined in TS 28.552 [4] and 5G KPIs defined in TS 28.554 [5].

This MDA capability is applicable for prediction and statistics of cross-domain management data and domain-specific management data.

##### 7.2.x.2	Use case

Certain scenarios might need a prediction or statistics on existing management data, e.g. prediction or statistics of single or multiple PMs and KPIs. The MnS consumer of this use case (who may also be a MnS producer) might be interested in the analytics (statistics or prediction) on existing management data (PMs and KPIs) which can be then consumed to produce services on other use cases such as mobility optimization or load balancing etc. The MnS consumer gets a specific type of prediction or statistics (e.g., mean, standard deviation, correlation, etc.)  of specific management data (PMs/KPIs) on network objects.

Statistics of the management data include the statistical calculation on the data themselves over a period in the past and/or statistics of management data following a sliding window procedure to produce statistics. Predictions of management data may include predictions of the data themselves over a future period, and/or predictions of the data following a sliding window procedure.

The main objective of this use case is to provide analytics (statistics and/or predictions) on the existing management data.

##### 7.2.x.3	Requirements

Table 7.2.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SPM_MDA-01 | MDA capability for management data analytics shall be able to produce predictions for the requested management data. | Prediction and statistics of Management data |
| REQ-SPM_MDA-02 | MDA capability for management data analytics shall be able to produce statistics for the requested management data. | Prediction and statistics of Management data |

Editor's note: Requirements will be enhanced to mention the specific set of PMs and KPIs on which the prediction and the statistics are performed.

End of change