3GPP TSG-SA5 Meeting #147 	S5-232295

Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0039 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add coodination requirement for user experience case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA5 finalized the Release 17 standardization of MDA in TS 28.104. The content of TS 28.104 provides the framework of MDA (different MDAFs and MDASs) with a number of MDA capabilities as described in clause 7 and data definitions for MDA capabilities in clause 8. However, the MDA capabilities in current TS 28.104 are not required to provide the specific MDA capabilities for single domain MDAF or cross domain MDAF. For example, the service experience analysis MDA in clause 8.4.2 seems provide the cause analysis to indicate the user experience issues based on multiple enabling data from RAN, core network and QoE data. However, the quality of user experience may be measured based on a number of network performance KPIs (e.g., a composited KPIs) provided by core network domain. Thus, the coordination requirement is proposed to be part of user experience analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add coordination requirements for user experience analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Analytics output cannot cover all possible scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1.1, 7.2.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 7.2.2.1	Service experience analysis

##### 7.2.2.1.1	Description

This MDA capability is for the service experience analysis.

This MDA capability is applicable for cross-domain service experience analysis and domain-specific service experience analysis.

##### 7.2.2.1.2	Use case

Service experience of end user is key indicator that directly reflects the user satisfaction degree. In 5G system, the diversity of network services is expanding all the time and the requirements of different services especially from vertical users are being standardized. Considering these diverse requirements and expectation from end user perspective (e.g. priorities of SLA related attributes such as latency, throughput, maximum number of users or different required values of these attributes), the service experience as a comprehensive indicator need to be extensively analysed.

##### 7.2.2.1.3	Requirements

Table 7.2.2.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SER_EXP_MDA-01 | Cross-domain MDA capability for service experience analysis shall be able to identify the source of service experience issue, e.g. RAN issue, CN issue, TN issue, UE issue, service provider issue. | Service experience analysis |
| REQ-SER_EXP_MDA-02 | MDA capability for service experience analysis shall be able to provide the analytics output with following information describing the current service experience aspects and potentially future prediction: - The predicted future service experience and/or observed service experience statistics. - Service experience degradation root cause analysis. | Service experience analysis |
| REQ-SER_EXP_MDA-03 | MDA capability for service experience analysis shall be able to provide the level of service experience. | Service experience analysis |
| REQ-SER_EXP_MDA-04 | MDA capability for service experience analysis shall be able to provide the recommendation for improving service experience. | Service experience analysis |
| REQ-SER_EXP_MDA-0x | Cross-domain MDA capability for service experience analysis may be able to provide coordination function which is based on the analytics outputs from single domains. | Service experience analysis |
| REQ-SER_EXP_MDA-0y | Cross-domain MDA capability for service experience analysis shall be able to collection analytics outputs and provide the analytics including: - Service experience degradation demarcation analysis. | Service experience analysis |

End of change