3GPP TSG-SA5 Meeting #149 S5-234429 # Berlin, Germany, 22 – 26 May, 2023 CR-Form-v12.2. CHANGE REQUEST. 28.552; CR; 0440; rev; -; Current version:; 18.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New measurement “Number of RRC release” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 28.552, currently there is no performance measure which characterizes the number of RRC release messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure showing the count of RRC release messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, RRC Retainability KPI (TS 28.554 CR0124) cannot be derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.15.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.554 CR0124 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 5.1.1.15.X	Number of RRC release messages

a)	This measurement provides the number of RRC release messages, The measurement is split into subcounters per suspendConfig availability.

b)	CC.

c)	Transmission of a "RRCRelease" message to UE.

d)	A single integer value.

e)	RRC.Release.WithSuspendConfig and RRC.Release.WithoutSuspendConfig.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	This measurement is to support the calculation of RRC retainability KPI.

# >>>>END OF CHANGES<<<<