3GPP TSG-SA5 Meeting #150	S5-235557

# Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0200 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MB-SMF as consumer of ConvergedCharging service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 、 |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | China Mobile Com. Corporation, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_CH |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the support of 5MBS charging, the MB-SMF should be added as service consumer of Nchf_ConvergedCharging service and Nchf_OfflineOnlyCharging service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MB-SMF as consumer of Nchf_ConvergedCharging service and Nchf_OfflineOnlyCharging service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MB-SMF cannot consume Nchf_ ConvergedCharging service and Nchf_OfflineOnlyCharging service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.1, 6.2.2, 6.2.3, 6.2.4, 6.5.1, 6.5.2, 6.5.3, 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

5GC	5G Core Network

5GS	5G System

AMF	Access and Mobility Management Function

CCS	Converged Charging System

CEF	Charging Enablement Function

CHF	Charging Function

IEC	Immediate Event Charging

MB-SMF	Multicast/Broadcast Session Management Function

MMS	Multimedia Messaging Service

MnS	Management Service

NF	Network Function

PCF	Policy Control Function

SBI	Service based Interface

SMSF	Short Message Service Function

SMF	Session Management Function

Next change

6.2.1	General

Service description: The ConvergedCharging service provides charging for session and event based NF services. This ConvergedCharging service offers charging:

-	With quota management (online; this includes support for both blocking mode and non-blocking mode)

-	Without quota management (offline)

-	Charging information record generation

The following table shows the CHF Services and CHF Service Operations.

Table 6.2.1-1: NF services provided by the CHF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nchf_ConvergedCharging | Create | Request/Response | SMF, SMSF, AMF, SMF+PGW-C, NEF, IMS-Node, MMS-Node, CEF, MnS Producer, EES, MB-SMF |
|  | Update | Request/Response | SMF, SMF+PGW-C, IMS-Node, MB-SMF |
|  | Release | Request/Response | SMF, SMSF, AMF, NEF, SMF+PGW-C, IMS-Node, MMS-Node, MB-SMF |
|  | Notify | Notify | SMF, SMF+PGW-C, IMS-Node |

The applicability of ConvergedCharging service to:

-	SMF, MB-SMF and SMF+PGW-C as consumer is specified in TS 32.255 [30] for 5G data connectivity domain charging;

-	IMS-Node as consumer is specified in TS 32.260 [31] for IMS charging;

-	NEF as consumer is specified in TS 32.254 [32] for exposure function Northbound Application Program Interfaces charging;

-	AMF as consumer is specified in the TS 32.256 [33] for 5G connection and mobility domain charging;

-	SMSF as consumer is specified in TS 32.274 [34] for short message service charging;

-	CEF as consumer is specified in the TS 28.201 [35] and TS 28.202[36] for Network slice charging and edge computing charging;

-	MnS Producer as consumer is specified in the TS 28.201 [35] and TS 28.202 [36] for Network slice charging.

-	MMS-Node as consumer is specified in TS 32.270 [37] for multimedia messaging service charging.

-	EES as consumer is specified in the TS 32.257 [38] for edge computing charging.

The input and output parameters described in the clauses below are common to all NF Consumers. The usage of these common parameters and additional NF Consumer specific parameters are specified in dedicated charging specifications.

Next change

6.2.2	Nchf_ConvergedCharging_Create service operation

Service operation name: Nchf_ConvergedCharging_Create

Description: Provides charging capabilities before service delivery, offers charging with and without quota management, as well as charging information record generation. It is used for both session and event based charging. Provides means for the NF Consumer to create the resource of the charging session. If it is used for session based charging the operation also makes an implicit subscribe to notification of events in CHF that requires re-authorization or abort.

The service operation may be used to request quota authorisation for service delivery and may open a CDR in the CHF, based on the information provided by the NF Consumer.

Known NF Consumers: SMF, SMSF, AMF, IMS-Node, MMS-Node, SMF+PGW-C, NEF, CEF, MnS Producer, EES, MB-SMF.

Inputs, Required: Subscriber identifier, either service identification or rating group.

Inputs, Optional: Requested service units, one-time event, destination address, provider, location information, time and date.

Outputs, Required: Result indication.

Outputs, Optional: Granted service units, validity time, triggers.

Next change

6.2.3	Nchf_ConvergedCharging_Update service operation

Service operation name: Nchf_ConvergedCharging_Update

Description: Provides charging capabilities during service delivery, offers usage reporting and quota management, as well as charging information record generation.

The service operation is used to report usage and may request further quota authorisation, if the trigger conditions occurs, this operation may cause update of the CDR or production of an interim CDR in the CHF.

Known NF Consumers: SMF, IMS-Node, SMF+PGW-C, MB-SMF.

Inputs, Required: Subscriber identifier (Optional for emergency session), session identifier, reporting reason.

Inputs, Optional: Requested service units, used service units.

Outputs, Required: Result indication.

Outputs, Optional: Granted service units, validity time, triggers.

Next change

6.2.4	Nchf_ConvergedCharging_Release service operation

Service operation name: Nchf_ConvergedCharging_Release

Description: Provides charging capabilities after service delivery, offers usage reporting and charging information record generation. Provides means for the NF Consumer to release the resource of charging session information.

The charging delete request is used to report usage and close the CDR in the CHF if it has been opened.

Known NF Consumers: SMF, AMF, IMS-Node, MMS-Node, SMF+PGW-C, SMSF, NEF, MB-SMF.

Inputs, Required: Subscriber identifier, session identifier, release reason.

Inputs, Optional: Used service units.

Outputs, Required: Result indication.

Outputs, Optional: None.

Next change

6.5.1	General

Service description: The OfflineOnlyCharging service provides charging for session based NF services. This OfflineCharging service offers charging information record generation.

The following table shows the CHF Services and CHF Service Operations.

Table 6.5.1-1: NF services provided by the CHF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nchf_OfflineOnlyCharging | Create | Request/Response | SMF, MB-SMF, IMS-Node |
|  | Update | Request/Response | SMF, MB-SMF, IMS-Node |
|  | Release | Request/Response | SMF, MB-SMF, IMS-Node |

The applicability of OfflineOnlyCharging service to SMF and MB-SMF as NF consumer is specified in TS 32.255 [30] for 5G data connectivity domain charging. The applicability of OfflineOnlyCharging service to IMS-Node as NF consumer is specified in TS 32.260 [31] for IMS charging.

The input and output parameters described in the clauses below are common to all NF Consumers. The usage of these common parameters and additional NF Consumer specific parameters are specified in dedicated charging specifications.

Next change

6.5.2	Nchf_OfflineOnlyCharging_Create service operation

Service operation name: Nchf_OfflineOnlyCharging_Create

Description: Provides charging capabilities before service delivery, offers charging information record generation. Provides means for the NF Consumer to create the resource of the charging session.

The service operation shall open a CDR in the CHF, based on the information provided by the NF Consumer.

Known NF Consumers: SMF, MB-SMF, IMS-Node.

Inputs, Required: Subscriber identifier, either service identification or rating group.

Inputs, Optional: destination address, provider, location information, time and date.

Outputs, Required: Result indication.

Outputs, Optional: triggers.

Next change

6.5.3	Nchf_OfflineOnlyCharging_Update service operation

Service operation name: Nchf_OfflineOnlyCharging_Update

Description: Provides charging capabilities during service delivery, charging information record generation.

If the trigger conditions occurs, this operation may cause update of the CDR or production of an interim CDR in the CHF.

Known NF Consumers: SMF, MB-SMF.

Inputs, Required: Subscriber identifier (exception for emergency session), session identifier, reporting reason.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: triggers.

Next change

6.5.4	Nchf_OfflineOnlyCharging_Release service operation

Service operation name: Nchf_OfflineOnlyCharging_Release

Description: Provides charging capabilities after service delivery, charging information record generation. Provides means for the NF Consumer to release the resource of charging session information.

The charging delete request is used to close the CDR in the CHF if it has been opened.

Known NF Consumers: SMF, MB-SMF, IMS-Node.

Inputs, Required: Subscriber identifier, session identifier, release reason.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

End of change